Figure 1-0. DENVER PEI_MIC Board (Sheet 2 of 7 )

Figure 1-0. DENVER PEI_MIC Board Isolation (Detail ‘A’) (Sheet 3 of 7 )

Figure 1-0. DENVER PEI_MIC Board Isolation (Detail ‘B’) (Sheet 4 of 7 )

Table 1-0. DENVER PEI

| FIG & INDEX NUMBER |  | PART NUMBER | DENVER C/L | DESCRIPTION | UNITS PER ASSY | SUPPLIER PART NO. | SUPL CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-0 |  |  |  |  |  |  |  |
|  | -1 |  |  | IC, NJM2072M (U24) | 1 | NJM2072M | NJR |
|  | -2 |  |  | IC, RC4558ID (U23) | 1 | RC4558IDR NJM14558E | TI NJR |
|  | -3 |  |  | Transistor, 2SJ347F (Q17) | 1 |  |  |
|  | -4 |  |  | Transistor, KRC106S (Q16) | 1 | KRC106S | KEC |
|  | -5 |  |  | EMI Filter, NFM21CC222R1H3 (F13) | 1 | NFM21CC222R1H3 | MRT |
|  | -6 |  |  | Connector, 35312-04 (J9) | 1 | 35312-0560 | MOLEX |
|  | -7 |  |  | MIC_NOP, (MI1) | 1 | MIC_NOP |  |
|  | -8 |  |  | CHIP TANTAL, 1uF/25V/A (CT77,CT80) | 2 | TCSCS1E105*AAR 293D105(1)025A(2) | SSE VISHA |
|  | -9 |  |  | CHIP TANTAL, 2.2uF/16V/A (CT83) | 1 | TCSCS1C225*AAR 293D225(1)016A(2) | SSE VISHA |
|  | -10 |  |  | CHIP TANTAL, 10uF/16V/B (CT78,CT79,CT84) | 3 | TCSCS1C106*BAR 293D106(1)016B(2) | SSE VISHA |
|  | -11 |  |  | CHIP TANTAL, 22uF/35V/D (CT75) | 1 | TCSCS1V226*DAR 293D226(1)035D(2) | SSE VISHA |
|  | -12 |  |  | CHIP TANTAL, 47uF/16V/B (CT76,CT82,CT99) | 3 |  |  |
|  | -13 |  |  | CHIP-C, 47㎊ (C89,C98) | 2 | CL10C470JB8NNN VJ0603A470JXAAT | SSE VISHA |
|  | -14 |  |  | CHIP-C, 220㎊ (C118,C119) | 2 | CL10C221JB8NNN VJ0603A221JXAAT | SSE VISHA |
|  | -15 |  |  | CHIP-C, 1㎋ (C117) | 1 | CL10C102JB8NNN VJ0603Y102JXAAT | SSE VISHA |
|  | -16 |  |  | CHIP-C, 4.7nF (C91) | 1 | CL10C472JB8NNN VJ0603Y472JXAAT | SSE VISHA |
|  | -17 |  |  | CHIP-C, 100㎋ (C116) | 1 | CL10B104KB8NNN VJ0603Y104JXAAT | SSE VISHA |
|  | -18 |  |  | CHIP-C, 1㎌ (C114,C115) | 2 | CL10F105ZP8NNN 0603B105J500CT | SSE WALS |
|  | -19 |  |  | CHIP-C, C_NOP (C90,C92) | 2 | C_NOP |  |
|  | -20 |  |  | CHIP-Re, 0Ω (R253) | 1 | RC1608J000CS WR06X000JTL | SSE WALS |
|  | -21 |  |  | CHIP-Re, 10Ω (R249) | 1 | RC1608J100CS WR06X100JTL | SSE WALS |
|  | -22 |  |  | CHIP-Re, 47Ω (R205) | 1 | RC1608J470CS WR06X470JTL | SSE WALS |
|  | -23 |  |  | CHIP-Re, 1KΩ (R207,R209,R212,R214) | 4 | RC1608J102CS WR06X102JTL | SSE WALS |
|  | -24 |  |  | CHIP-Re, 2.2KΩ (R211) | 1 | RC1608J222CS WR06X222JTL | SSE WALS |
|  | -25 |  |  | CHIP-Re, 4.7KΩ (R216) | 1 | RC1608J472CS WR06X472JTL | SSE WALS |

Table 1-0. DENVER PEI (CONT.)

| FIG & INDEX NUMBER |  | PART NUMBER | DENVERC/L | DESCRIPTION | UNITS PER ASSY | SUPPLIER PART NO. | SUPL CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-0 | -26 |  |  | CHIP Re, 10KΩ (R204,R206,R208,R210,R215,R218,R220,R247,R248) | 9 | RC1608J103CS WR06X103JTL | SSE WALS |
|  | -27 |  |  | CHIP Re, 47KΩ (R251,R252) | 2 | RC1608J473CS WR06X473JTL | SSE WALS |
|  | -28 |  |  | CHIP Re, 100KΩ (R254) | 1 | RC1608J104CS WR06X104JTL | SSE WALS |
|  | -29 |  |  | CHIP Re, 1MΩ (R250) | 1 | RC1608J105CS WR06X105JTL | SSE WALS |
|  | -30 |  |  | CHIP Re, NOT USED (R255) | 1 | R_NOP |  |

Acronyms and abbreviations

| Acronyms & abbreviations | Description | Acronyms & abbreviations | Description |
| --- | --- | --- | --- |
| AM | Allegro Microsystems, Inc. | NASE | National Semiconductor |
| AEL | ANDHRA Electronics Limited | NEC | NEC |
| ATMC | ATMEL Corporation | NJR | New Japan Radio |
| BBC | Burr-Brown Corporation | N.H | N.H |
| CAEL | California Eastern Labs | NFR | Ningbo Forward Relay |
| CEL | CEL | NXPS | NXP Semiconductors |
| CC | Comtec Crystals | OMR | Omron |
| CE | Cystech Electonics | OS | ON Semiconductor |
| DAST | DASTEK | ORE | ORE |
| DIGL | Diglight | PNSN | Panasonic |
| EE | EASTERN ELECTRONIC | PANJ | PANJIT |
| EI | ECX INC | PALI | Para Light |
| EICS | EIC discrete Semiconductors | PHSE | Philips Semiconductors |
| ERNI | ERNI | PILK | PILKO |
| E-S | E-Switch | RNS | RENESAS |
| FS | Fairchild Semiconductor | ROHM | ROHM |
| GE | Geyer Electronics | SWHC | SAMWHA CAPACITOR |
| GS | GULFSEMI | SWHE | SAMWHA ELECTRIC |
| HART | Harting | SAMS | SAMSUNG |
| HIRO | Hirose | SAMY | SAMYOUNG |
| HRS | HRS | SE | Sejung Electronics |
| HTC | HTC | SEL | SEMTECH ELECTRONICS LTD |
| INIC | INICS | SGST | SGS-THOMSON MICROELETRONICS |
| IR | International Rectifier | S-T | ShinHwa-Tech |
| JIC | JIC | SUCO | Sumida Corporation |
| JD | JOHANSON DIELECTRICS | SUTE | SUNGIL TECH |
| KEC | KEC(Korea Electronics) | STM | STMicroelectronics |
| KEM | KEMET | SYSE | SynSemi |
| KB | Kingbright | TASE | Taiwan Semiconductor |
| LRD | LESHAN RADIO DOMPANY, LTD. | TAKC | TAK CHEONG |
| LT | Linear Technology | TDK | TDK |
| MAX | MAXIM | TI | Texas Instruments |
| MDES | MDE Semiconductor | TSHB | TOSHIBA |
| MR | MEMORIVE | TEL | TRANSYS ELECTRONICS LIMITED |
| MCC | Micro Commercial Components | UTC | UNISONIC TECHNOLOGIE CO., |
| MOLE | MOLEX | VISH | VISHAY |
| MPS | Monolithic Power Systems | WALS | WALSIN |
| MTRL | Motorola | WOTE | Woori-Tech |
| MRT | MURATA | XILI | Xilinx |
| NAIS | Nais(Matsushita Electric Works) |  |  |