| THE DRAWING(S) AND THE INFORMATION CONTAINED HEREIN ARE THE PROPERTY OF WOOJIN INDUSTRIAL SYSTEMS CO., LTD. THEY SHALL NOT BE DISCLOSED, REPRODUCED OR USED, IN ANY MANNER WITHOUT THE PRIOR WRITTEN CONSENT OF WOOJIN INDUSTRIAL SYSTEMS CO., LTD. |  | DENVER RTD EAGLE P3 Communication System Software Test Report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DISTRIBUTION |  | R E V I S I O N |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | Oct 14, 2011 |  | Issued |  |  | J.Y.LEE | J.Y.KWON | B.J.JONE | E.K.LEE |
|  |  |  | No. | ISSUED DATE |  | REVISION NOTE |  |  | DESIGNED | CHECKED | APPROVED | REVIEWED |
|  |  |  |  |  |  |  |  |  | ENGINEERING |  |  | PM |
|  |  | PM |  |  | REVIEWED |  | E.K.LEE |  | CUSTOMER : |  |  |  |
|  |  | ENGINEERING |  |  | APPROVED |  | B.J.JONE |  | HYUNDAI-ROTEM COMPANY |  |  |  |
|  |  |  |  |  | CHECKED |  | J.Y.KWON |  |  |  |  |  |
|  |  |  |  |  | DESIGNED |  | J.Y.LEE |  | PROJECT : |  |  |  |
|  |  | ISSUED DATE |  |  | October 14, 2011 |  |  |  | DENVER RTD EAGLE P3 COMMUNICATION SYSTEM |  |  |  |
|  |  | ISSUED BY |  |  | R & D CENTER |  |  |  |  |  |  |  |
|  |  | PROJECT NO. |  |  | P2009-024 |  |  |  |  |  |  |  |
|  |  | ORDER NO. |  |  | P091201400 |  |  |  |  |  |  |  |
|  |  | DOCUMENT NO. |  |  |  |  |  | REVISION |  |  |  |  |
|  |  | WJ-DENVER-STR-V1.0 |  |  |  |  |  | 1 |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1.0	Purpose	4

2.0	Reference Documents	4

3.0	Summary	4

3.1	PAC	4

3.1.1	Fun-REQ-PAC-Initial-01 Test	5

3.1.2	Fun-REQ-PAC-Analysis-01 Test	5

3.1.3	Fun-REQ-PAC-Analysis-02 Test	5

3.1.4	Fun-REQ-PAC-Analysis-03 Test	5

3.1.5	Fun-REQ-PAC-Analysis-04 Test	5

3.1.6	Fun-REQ-PAC-Analysis-05 Test	5

3.1.7	Fun-REQ-PAC-Analysis-06 Test	6

3.1.8	Fun-REQ-PAC-Address-01 Test	6

3.1.9	Fun-REQ-PAC-Address-02 Test	6

3.1.10	Fun-REQ-PAC-Initial AVL-01 Test	6

3.2	CCP	6

3.2.1	Fun-REQ-CCPInitial_01	6

3.2.2	Fun-REQ-CCP Main-Control-01	7

3.2.3	Fun-REQ-CCP Main-Control-02	7

3.2.4	Fun-REQ-CCP Main-Control-03	7

3.2.5	Fun-REQ-CCP Main-Control-04	7

3.2.6	Fun-REQ-CCP Main-Control-05	7

3.2.7	Fun-REQ-CCP Main-Control-06	7

3.2.8	Fun-REQ-CCP Main-Control-07	8

3.2.9	Fun-REQ-CCP Main-Control-08	8

3.2.10	Fun-REQ-CCP Main-Control-09	8

3.2.11	Fun-REQ-CCP Main-Control-10	8

3.2.12	Fun-REQ-CCP Main-Control-11	8

3.2.13	Fun-REQ-CCP Main-Control-12	8

3.2.14	Fun-REQ-CCP Main-Display-01	9

3.2.15	Fun-REQ-CCP Main-Comm-02	9

3.2.16	Fun-REQ-CCP Main-Comm-03	9

3.3	CNCS	9

3.3.1	Fun-REQ-CNCS Launcher-01 test	9

3.3.2	Fun-REQ-CNCS Main-Initial-01 test	9

3.3.3	Fun-REQ-CNCS Main-Monitoring-01 test	10

3.3.4	Fun-REQ-CNCS Main-RS485-01 test	10

3.3.5	Fun-REQ-CNCS PP-Initial-01 test	10

3.3.6	Fun-REQ-CNCS PP-ContentPlay-01 test	10

3.3.7	Fun-REQ-CNCS SP-Initial-01 test	10

3.3.8	Fun-REQ-CNCS SP-ContentPlay-01 test	10

3.3.9	Fun-REQ-CNCS CDT Data-Initial-01 test	11

3.3.10	Fun-REQ-CNCS CDT Data-RS232-01 test	11

3.3.11	Fun-REQ-CNCS CDT Data-RS232-02 test	11

3.3.12	Fun-REQ-CNCS CDT Data-RS232-03 test	11

3.3.13	Fun-REQ-CNCS FTP-Initial-01 test	11

3.3.14	Fun-REQ-CNCS FTP-Wayside-01 test	11

3.3.15	Fun- REQ-CNCS FTP-Update-01 test	12

3.3.16	Fun-REQ-CNCS FTP-Update-02 test	12

3.3.17	Fun-REQ-CNCS ContentUpdate-Initial-01 test	12

3.3.18	Fun-REQ-CNCS ContentUpdate-RS485-01 test	12

3.3.19	Fun-REQ-CNCS ContentUpdate-Update-01 test	12

3.3.20	Fun-REQ-CNCS ContentUpdate-Update-02 test	13

3.3.21	Fun-REQ-CNCS ContentUpdate-Update-03 test	13

3.3.22	Fun-REQ-CNCS ContentUpdate-Update-04 test	13

3.4	LEDC	13

3.4.1	Fun-REQ-FDI-Initial-01 test	13

3.4.2	Fun-REQ-FDI-Comm-01 test	13

3.4.3	Fun-REQ-FDI-Display-01 test	14

3.4.4	Fun-REQ-FDI- Update -01 test	14

3.4.5	Fun-REQ-SDI-Initial-01 test	14

3.4.6	Fun-REQ-SDI-Comm-01 test	14

3.4.7	Fun-REQ-SDI-Display-01 test	14

3.4.8	Fun-REQ-SDI- Update -01 test	14

3.4.9	Fun-REQ-PII-Initial-01 test	15

3.4.10	Fun-REQ-PII-Comm-01 test	15

3.4.11	Fun-REQ-PII-Display-01 test	15

3.4.12	Fun-REQ-PII- Update -01 test	15

3.5	LIC	15

3.5.1	Fun-REQ-LIC Main-01	15

4.0	Variances	15

5.0	Summary of Result	15

6.0	Attachment. Test Execution Trace Report	16

6.1	PAC	16

6.2	CCP	24

6.3	CNCS	28

6.4	LEDC	32

6.5	LIC	34

Revision Records

| Revision | Description | Date |
| --- | --- | --- |
| 0 | First Issued | 2011.10.14 |

Purpose

The purpose of this document is to report the test result DENVER Communication System software.

Reference Documents

The following documents were used as sources of information for this test report.

| Publisher | Document Number | Title |
| --- | --- | --- |
| DENVER | 18FH012 | DENVER RTD TS (Confirmed Version, March 2010) Part C Rolling Stock Requirements |
| Rotem | REDU 106113 | Procement Technical Specification For DENVER RTD EMU Communication System |
| WOOJIN | WJ-SRS-001_r2 | DENVER RTD EAGLE P3 Communication System Software Requirements Specification |
|  | WJ-SRS-SDD-V1.0 | DENVER RTD EAGLE P3 Communication System Software Design Description |
|  | WJ-DENVER-STP-V1.0 | DENVER RTD EAGLE P3 Communication System Software Test Plan |

Summary

All features of communications systems SW were tested.

Test was executed in 2010.04.11 ~ 2010.04.15 by Y.J.KWON. Test was started with SW version 1.0.0.1, and this program passed all the tests.

PAC

Connect J-TAG debugger to PAC board and change the software variable for the test lists to check the result or connect the packet simulation program to RS485 and change an input value to check a RS485 output value.

External Interface Signals test :Operate an input signal for the test lists and check a RS485 output value by connecting the External Interface Signals test board operating Input and Output port signals which are input to PAC board .

Multi coupled unit process test: Connect at least four PAC boards then test to check the result value by using RS485.

Communication test : Check the communication result by connecting a speaker.

Polling test  : Test by connecting the other device packet simulation program in the communication system to RS485.

Fun-REQ-PAC-Initial-01 Test

Fun-REQ-PAC-Initial-01 features passed all the tests.

Fun-REQ-PAC-Initial-01 features was tested to validate ‘PAC Initialization’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.1.

Fun-REQ-PAC-Analysis-01 Test

Fun-REQ-PAC-Analysis-01 features passed all the tests.

Fun-REQ-PAC-Analysis-01 features was tested to validate ’Check the Status of all Devices within a specific Car’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.2.

Fun-REQ-PAC-Analysis-02 Test

Fun-REQ-PAC-Analysis-02 features passed all the tests.

Fun-REQ-PAC-Analysis-02 features was tested to validate ’Consist Inauguration’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.3.

Fun-REQ-PAC-Analysis-03 Test

Fun-REQ-PAC-Analysis-03 features passed all the tests.

Fun-REQ-PAC-Analysis-03 features was tested to validate ’ Retrieve the Device Status tables from all PRIMARY PAC’s’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.4.

Fun-REQ-PAC-Analysis-04 Test

Fun-REQ-PAC-Analysis-04 features passed all the tests.

Fun-REQ-PAC-Analysis-04 features was tested to validate ’ PA Audio Train Line level balancing’ satisfaction.

The following test documents are associated with this test.

S

EPTA PAC Software Test Plan section 12.3.5.

Fun-REQ-PAC-Analysis-05 Test

Fun-REQ-PAC-Analysis-05 features passed all the tests.

Fun-REQ-PAC-Analysis-05 features was tested to validate ’Monitor Consist Integrity’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.6.

Fun-REQ-PAC-Analysis-06 Test

Fun-REQ-PAC-Analysis-06 features passed all the tests.

Fun-REQ-PAC-Analysis-06 features was tested to validate ’ Monitor Intercom Status’  satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.7.

Fun-REQ-PAC-Address-01 Test

Fun-REQ-PAC-Address-01 features passed all the tests.

Fun-REQ-PAC-Address-01 features was tested to validate’ ‘Process Local PEI Address Status Interrupt’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.8.

Fun-REQ-PAC-Address-02 Test

Fun-REQ-PAC-Address-02 features passed all the tests.

Fun-REQ-PAC-Address-02 features was tested to validate’ Query Train Line PEI Address Status’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3.9.

Fun-REQ-PAC-Initial AVL-01 Test

Fun-REQ-PAC-Initial AVL-01 features passed all the tests.

A Fun-REQ-PAC-Initial AVL-01 feature was tested to validate ’Monitor & Process AVL Trigger Events’ satisfaction.

The following test documents are associated with this test.

DENVER PAC Software Test Plan section 12.3

CCP

CRA Rack  with CCP, PAC, CNCS, FDI, SDI, PII should be composed for CCP software test.

Following the power approval, CAB ON signals should be input and RS-485 communication line should be connected by operating CCP program.

Fun-REQ-CCPInitial_01

Fun-REQ-CCPInitail-01 features passed all the tests.

Fun-REQ-CCPInitial-01 is for the purpose of verifying if the function of Initial Process satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section  12.2.1

Fun-REQ-CCP Main-Control-01

Fun-REQ-CCPControl-01 features passed all the tests.

Fun-REQ-CCP Control -01 is for the purpose of verifying if the car number suitability judgment function satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section  12.2.2

Fun-REQ-CCP Main-Control-02

Fun-REQ-CCPControl-02features passed all the tests.

Fun-REQ-CCPControl-02 is for the purpose of verifying if the function of CREW ID Process satisfies the requirements. The following test documents are associated with this test.

DENVER Software Test Plan section 12.2.3

Fun-REQ-CCP Main-Control-03

Fun-REQ-CCPControl-03 features passed all the tests.

Fun-REQ-CCPControl-03 is for the purpose of verifying if the schedule management function satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.4

Fun-REQ-CCP Main-Control-04

Fun-REQ-CCP Control-04 features passed all the tests.

Fun-REQ-CCP Control-04 is for the purpose of verifying if the function to change settings satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.5

Fun-REQ-CCP Main-Control-05

Fun-REQ-CCP Control-05 features passed all the tests.

Fun-REQ-CCP Control-05 is for the purpose of verifying if the car monitoring function satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.6

Fun-REQ-CCP Main-Control-06

Fun-REQ-CCP Control-06 features passed all the tests.

Fun-REQ-CCP Control-06 is for the purpose of verifying if the Help function satisfies the requirements.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.7

Fun-REQ-CCP Main-Control-07

Fun-REQ-CCP Control-07 features passed all the tests.

Fun-REQ-CCP Control-07 is for the purpose of verifying if the PEI process function satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.8.

Fun-REQ-CCP Main-Control-08

Fun-REQ-CCP Control-08 features passed all the tests.

Fun-REQ-CCP Control-08 is for the purpose of verifying if the PEI process function satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.9.

Fun-REQ-CCP Main-Control-09

Fun-REQ-CCP Control-09 features passed all the tests.

Fun-REQ-CCPControl-09는CAB MESSAGE. Fun-REQ-CCPControl-09 is for the purpose of verifying if the function to output and delete satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.10.

Fun-REQ-CCP Main-Control-10

Fun-REQ-CCP Control-10 features passed all the tests.

Fun-REQ-CCP Control-10 is for the purpose of verifying if the function to update data satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.11.

Fun-REQ-CCP Main-Control-11

Fun-REQ-CCP Control-11 features passed all the tests.

Fun-REQ-CCP Control-11 is for the purpose of verifying if the function to update data satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.12.

Fun-REQ-CCP Main-Control-12

Fun-REQ-CCP Control-12 features passed all the tests.

Fun-REQ-CCPControl-12 is for the purpose of verifying if the function to update data satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.13.

Fun-REQ-CCP Main-Display-01

Fun-REQ-CCPDisplay-01 features passed all the tests.

Fun-REQ-CCPDisplay-01 is for the purpose of verifying if the function to display stations information satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.14.

Fun-REQ-CCP Main-Comm-02

Fun-REQ-CCPComm-02 features passed all the tests.

Fun-REQ-CCPComm-02 is for the purpose of verifying if the function to transmit communication data and to process packets satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.15.

Fun-REQ-CCP Main-Comm-03

Fun-REQ-CCP Comm-03 features passed all the tests.

Fun-REQ-CCP Comm-03 is for the purpose of verifying if the GPS search function satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.2.16.

CNCS

CRA Rack with CNCS and CCP and PID shall be required in order to perform CNCS Software test.

After supply the power, it shall be confirmed to operate CNCS program normally, and the state of CNCS and Wayside shall be confirmed their normal performances on the right bottom.

Fun-REQ-CNCS Launcher-01 test

Fun-REQ-CNCS Launcher-01 features passed all the tests.

Fun-REQ-CNCS Launcher-01 is intended to verify whether the function of Launcher Process satisfies requirements or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.

Fun-REQ-CNCS Main-Initial-01 test

Fun-REQ-CNCS Main-Initial-01 features passed all the tests.

Fun-REQ-CNCS Main-Initial-01 is intended to verify whether the initializing function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.2

Fun-REQ-CNCS Main-Monitoring-01 test

Fun-REQ-CNCS Main- Monitoring -01 features passed all the tests.

Fun-REQ-CNCS Main-Monitoring-01 is intended to verify whether the Sub-Component monitoring function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.3

Fun-REQ-CNCS Main-RS485-01 test

Fun-REQ-CNCS Main- RS485-01 features passed all the tests.

Fun-REQ-CNCS Main-RS485-01 is intended to verify whether the RS-485 communication function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.4

Fun-REQ-CNCS PP-Initial-01 test

Fun-REQ-CNCS PP-Initial-01 features passed all the tests.

Fun-REQ-CNCS PP-Initial-01 is for the purpose of verifying if the initialization function of PP satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.5

Fun-REQ-CNCS PP-ContentPlay-01 test

Fun-REQ-CNCS PP-ContentPlay-01 features passed all the tests.

Fun-REQ-CNCS PP-ContentPlay-01 is for the purpose of verifying if the contents play function of PP satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.6

Fun-REQ-CNCS SP-Initial-01 test

Fun-REQ-CNCS SP-Initial-01 features passed all the tests.

Fun-REQ-CNCS SP-Initial-01 is for the purpose of verifying if the initialization function of SP satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.7

Fun-REQ-CNCS SP-ContentPlay-01 test

Fun-REQ-CNCS SP-ContentPlay-01 features passed all the tests.

Fun-REQ-CNCS SP-ContentPlay-01 is for the purpose of verifying if the contents play function of SP satisfies the requirements. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.8

Fun-REQ-CNCS CDT Data-Initial-01 test

Fun-REQ-CNCS CDT Data-Initial -01 features passed all the tests.

Fun-REQ-CNCS Main-Initial-01 is intended to verify whether the initializing function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.9

Fun-REQ-CNCS CDT Data-RS232-01 test

Fun-REQ-CNCS CDT Data-RS232-01 features passed all the tests.

Fun-REQ-CNCS CDT Data-RS232-01 is intended to verify whether the data selecting function of CDT satisfies requirements via RS-232 communication or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.10

Fun-REQ-CNCS CDT Data-RS232-02 test

Fun-REQ-CNCS CDT Data-RS232-02 features passed all the tests.

Fun-REQ-CNCS CDT Data-RS232-02 is intended to verify whether the data selecting function of CDT Data Process to CNCS satisfies requirements via RS-232 communication or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.11

Fun-REQ-CNCS CDT Data-RS232-03 test

Fun-REQ-CNCS CNCS CDT Data-RS232-03 features passed all the tests.

Fun-REQ-CNCS CDT Data-RS232-03 is intended to verify whether the communication S/W version information transmitting function of CDT Data Process satisfies requirements via RS-232 communication or not. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.12

Fun-REQ-CNCS FTP-Initial-01 test

Fun-REQ- CNCS FTP-Initial-01 features passed all the tests.

Fun-REQ-CNCS FTP-Initial-0 is intended to verify whether the initializing function of FTP Process satisfies requirements or not. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.13

Fun-REQ-CNCS FTP-Wayside-01 test

Fun-REQ- CNCS FTP-Wayside-01 features passed all the tests.

Fun-REQ-CNCS FTP-Wayside-01 is intended to verify whether the FTP-Wayside monitoring function of FTP Process satisfies requirements or not. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.14

Fun- REQ-CNCS FTP-Update-01 test

Fun-REQ-CNCS CNCS FTP-Update-01 features passed all the tests.

Fun-REQ-CNCS FTP-Update-01 is intended to verify whether the CNCS data transmitting function to FTP Server satisfies requirements or not. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.15

Fun-REQ-CNCS FTP-Update-02 test

Fun-REQ-CNCS CNCS FTP-Update-02 features passed all the tests.

Fun-REQ-CNCS CNCS FTP-Update-02 is intended to verify whether the FTP Server data receiving function to CNCS satisfies requirements or not. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.16

Fun-REQ-CNCS ContentUpdate-Initial-01 test

Fun-REQ-CNCS ContentUpdate-Initial-01 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Initial-01 is intended to verify whether the initializing function of Content Update Process satisfies requirements or not. The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.17

Fun-REQ-CNCS ContentUpdate-RS485-01 test

Fun-REQ-CNCS ContentUpdate-RS485-01 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-RS485-01 is intended to verify whether the RS-485 communication function of Content Update Process satisfies requirements or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.18

Fun-REQ-CNCS ContentUpdate-Update-01 test

Fun-REQ-CNCS ContentUpdate-Update-01 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Update-01 is intended to verify whether the sub-component update function of CCP, PAC and LED satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.19

Fun-REQ-CNCS ContentUpdate-Update-02 test

Fun-REQ-CNCS ContentUpdate-Update-02 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Update-02 is intended to verify whether the content update function of CCP satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.20

Fun-REQ-CNCS ContentUpdate-Update-03 test

Fun-REQ-CNCS CNCS ContentUpdate-Update-03 features passed all the tests.

Fun-REQ-CNCS CNCS ContentUpdate-Update-03 is intended to verify whether the MP3 content update function of PAC satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.21

Fun-REQ-CNCS ContentUpdate-Update-04 test

Fun-REQ-CNCS ContentUpdate-Update-04 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Update-04 is intended to verify whether the LED content update function of PAC satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

DENVER Software Test Plan section 12.8.22

LEDC

FDI, SDI, PII Unit with LEDC, PSA of CRA Rack, CNCS and PAC should be composed for LEDC Software test.

Following the power approval, it should be checked that the device initial value and ID are set up as the front text of LED module(LDM). Besides, the normal operation status of FDI, SDI and PII Unit relevant should be identified by pressing the CHECK button of CCP.

Fun-REQ-FDI-Initial-01 test

Fun-REQ-FDI-Initial-01 features passed all the tests.

Fun-REQ-FDI-Initial-01 features was tested to validate FDI Initialization Process satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.1

Fun-REQ-FDI-Comm-01 test

Fun-REQ-FDI-Comm-01 features passed all the tests.

Fun-REQ-FDI-Comm-01 features was tested to validate FDI Status Request satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.2

Fun-REQ-FDI-Display-01 test

Fun-REQ-FDI-Display-01 features passed all the tests.

Fun-REQ-FDI-Display-01 features was tested to validate Display FDI Content satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.3

Fun-REQ-FDI- Update -01 test

Fun-REQ-FDI-Update-01 features passed all the tests.

Fun-REQ-FDI-Update-01 features was tested to validate FDI Content Update Management satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.4

Fun-REQ-SDI-Initial-01 test

Fun-REQ-SDI-Initial-01 features passed all the tests.

Fun-REQ-SDI-Initial-01 features was tested to validate SDI Initialization Process satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.5

Fun-REQ-SDI-Comm-01 test

Fun-REQ-SDI-Comm-01 features passed all the tests.

Fun-REQ-SDI-Comm-01 features was tested to validate Request for SDI status data satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.6

Fun-REQ-SDI-Display-01 test

Fun-REQ-SDI-Display-01 features passed all the tests.

Fun-REQ-SDI-Display-01 features was tested to validate Display SDI Content satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.7

Fun-REQ-SDI- Update -01 test

Fun-REQ-SDI-Update-01 features passed all the tests.

Fun-REQ-SDI-Update-01 features was tested to validate SDI Content Update Management satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.8

Fun-REQ-PII-Initial-01 test

Fun-REQ-PII-Initial-01 features passed all the tests.

Fun-REQ-PII-Initial-01 features was tested to validate PII Initialization Process satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.9

Fun-REQ-PII-Comm-01 test

Fun-REQ-PII-Comm-01 features passed all the tests.

Fun-REQ-PII-Comm-01 features was tested to validate PII Status Request satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.10

Fun-REQ-PII-Display-01 test

Fun-REQ-PII-Display-01 features passed all the tests.

Fun-REQ-PII-Display-01 features was tested to validate Display AVL Station Arrival and Departure Messages satisfaction.

The following test documents are associated with this test.

- DENVER Software Test Plan section 12.6.11

Fun-REQ-PII- Update -01 test

Fun-REQ-PII-Update-01 features passed all the tests.

Fun-REQ-PII-Update-01 features was tested to validate PII Content Update Management satisfaction.

The following test documents are associated with this test.

DENVER Software Test Plan section 12.6.12

LIC

Fun-REQ-LIC Main-01

Fun-REQ-LIC-01 features passed all the tests.

Fun-REQ-LIC-01is intended to verify whether Main Process function satisfies requirements or not. The following test documents are associated whit this test.

DENVER Software Test Plan section  12.9.1.

Variances

None

Summary of Result

All features of PA and PIS software passed their tests.

Attachment. Test Execution Trace Report

PAC

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| PAC Initialization | TC-PAC-INITIAL-01-01 | Send CRA(MA) port signal from external interface single test board. Send CRA(MB) port signal from external interface signal test board. | It is checked that CRA-AB bit becomes 1 in carline packet viewer. It is checked that CRA-AB bit becomes 0 in carline packet viewer. | 1) It is check that CRA-AB bit became '1' in carline packet viewer. 2) It is checked that CRA-AB bit became 0 in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-02 | Change LIC packet in packet simulation program and send #4001 car numbers thru RS485. Change LIC packet in packet simulation program and send #4002 car numbers thru RS485. | Check if CRA-MAB byte is 1 in carline packet viewer. Check if CRA-MAB byte is 2 in carline packet viewer. | 1) It is check CRA-MAB byte became '1' in carline packet viewer. 2) It is check CRA-MAB byte became '2' in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-03 | Send CAB ON port signal from external interface signal test board. Send CAB OFF port signal from external interface signal test board. | It is checked that CAB bit is 1 in carline packet viewer. It is checked that CAB bit is 0 in carline packet viewer. | 1) It is checked that CAB bit became' 1' in carline packet viewer. 2) It is checked that CAB bit became' 2' in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-04 | Send coupler ON port signal from external interface signal test board. Send coupler OFF port signal from external interface signal test board. | Check if coupler bit 1 in carline packet viewer. Check if coupler bit 0 in carline packet viewer. | It is checked that coupler bit became '1' in carline packet viewer. 2) It is checked that coupler bit became '0' in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-05 | Send CRA(MA) port signal of external interface signal test board on car #4001. Send CRA(MA) port signal of external interface signal test board on car #4003. | It works as a main PAC of car and polling packets as many as car device IDs(34) are observed. It works as a main PAC of car and polling packets as many as car device IDs(34) are observed. | 1) It works as a main PAC of car and polling packets as many as car device IDs(34) are observed. 2) It works as a mina PAC of married car and polling packets as many as car device IDs(34) are observed. | Pass |
|  | TC-PAC-INITIAL-01-06 | Set PAC internal program variable to #4001 and #4003 to double headed train. Send CRA(MA) port single from external interface single test board. Send CAB on port signal from external interface signal test board. | It is checked that temporary change of internal variable in carline packet viewer. Carline polling runs by main PAC. Carline polling of #4001, #4003 train runs by master PAC. | 1) Temporary change of internal variable is checked in carline packet viewer. 2) Carline polling is checked by Main PAC of the car composition. 3) Carline polling of #4001, #4003 train runs by master PAC. | Pass |
|  | TC-PAC-INITIAL-01-07 | Change LIC packet in packet simulation program and send #4005 car number thru RS485. Off the packet simulation. Reboot PAC board. | Save the car number #4005 from LIC packet on PAC flash memory and RAM. Runs as car number which save in RAM. LIC's car number existing in the absence of information, information work and are loaded from flash memory. | 1) Check if PAC stores #4005 car number information received from LIC packet in the internal flash memory and RAM. 2) Check if it is run by the car number stored in RAM. 3) Check if it is run by loading the existing car number from the flash memory even if no car number information of LIC. | Pass |
|  | TC-PAC-INITIAL-01-08 | PAC power ON. Change LIC packet in packet simulation program and send #4007 car number thru RS485. | It runs by loading car number information from internal flash memory. 2) Save the car number from LIC packet on PAC flash memory and RAM and runs #4007. | 1) Check if car number information is loaded from internal flash memory. 2) Check if the car number received from LIC packet is stored in PAC flash memory and RAM and is run as #703. | Pass |
|  | TC-PAC-INITIAL-01-09 | Power ON of CRA(MA) PAC and CRA(MB) PAC. Power OFF CRA(MA) PAC. | CRA(MA) PAC is polled to main PAC. Detect if CRA(MB) PAC and CRA(MA) PAC are dead and are running by changing the role from sub PAC to main PAC. | 1) Check if CAR PAC is polled to main PAC. 2) Check if CRA(MB) detects CRA(MA) PAC is dead and also CRA(MB) is run as Main PAC in Sub PAC by changing a role. | Pass |
| Check the Status of all Devices within a specific Car | TC-PAC-ANALYSIS-01-01 | Send CRA(MA) port signal from external interface signal test board. Power on PAC board. | Run as main PAC. Polling continuously as many as device IDs. | 1) Check if it is run as Main PAC. 2) Check if it is polling continuously as many as number of device IDs. | Pass |
|  | TC-PAC-ANALYSIS-01-02 | Make all other device packets with packet simulation program and send via RS485. Cancel the packet response of some devices. | If the device gives a feedback of response packet to polling packet of PAC, the device is displayed alive on the status information of communication rack. If there were no response of packet received, the device that didn’t receive the packet would be displayed dead. | Check if the device gives a feedback of response packet to polling packet of PAC, the device is displayed alive on the status information of communication rack. 2) Check if it is displayed that the device is dead when no response packet received. | Pass |
| Consist Inauguration | TC-PAC-ANALYSIS-02-01 | Turn on CRA(MA) and CAB port signal of external interface signal test board in car #4001. Turn on CRA(MB) and CAB port signal of external interface signal test board in car #4002. | It is checked with carline packet viewer that CAB is on at CRA(MA) on car #4001 and train is connected with #4001, #4002, #4003 and #4004. It is checked with carline packet viewer that CAB is on at CRA(MB) in car #4002 and train is connected with #4004, #4003, #4002 and #4001. | 1) Check If the train consists of #4001, #4002, #4003 and #4004 with cab on single car #4001 CRA(MA) in CarLine viewer 2) Check If the train consists of #4004, #4003, #4002 and #4001 with cab on car #4002 CRA(MB) in CarLine viewer. | Pass |
|  | TC-PAC-ANALYSIS-02-02 | Turn on CRA(MA) and CAB port signal of external interface signal test board in car #4003. Turn in CRA(MB) and CAB port signal of external signal test board in car #4004. | It is checked with carline packet viewer that CAB is on at CRA(MA) on car #4003 and train is connected with #4001, #4002, #4003 and #4004. It is checked with carline packet viewer that CAB is on at CRA(MB) on car #4004 and train in connected with #4004, #4003, #4002 and #4001. | 1) Check If the train consists of #4001, #4002, #4003 and #4004 with cab on car #4003 CRA(MB) in CarLine viewer. 2) Check If the train consists of #4004, #4003, #4002 and #4001 cab on car #4004 CRA(MB) in CarLine viewer. | Pass |
| Retrieve the Device Status tables from all PRIMARY PAC’s | TC-PAC-ANALYSIS-03-01 | Turn on CRA(MA) and CAB port signal of external interface signal test board in car #4001. Turn on CRA(MB) and CAB port signal of external interface signal test board in car #4002. | Device status table is created because CAB is on at CRA(MA) on car #4001 and double heading is complete. Device status table has same configuration as CAB on at CRA(MA) even if CAB is on at CRA(MB) in car #4002. | 1) Check if Device status table is renewed owing to CAB on at CRA(MA) on car #4001 and the completion of double heading. 2) Check if Device status table has the same configuration as CAB on at CRA(MA) even if CAB is on at CRA(MB) in car #4001. | Pass |
|  | TC-PAC-ANALYSIS-03-02 | Create CCP packet that crew ID and train number are changed on packet simulation program of car #4001. | Check if the information of master PAC with CAB on is reviewed with the same train number and crew ID on car #4003. | 1) Check if the information of master PAC with CAB on is renewed with the same train number and crew ID on married car #4003 | Pass |
|  | TC-PAC-ANALYSIS-03-03 | 1) #4003 CRA(MA), #4004 CRA(MB) PAC Power OFF. | Check if the dead device information of car #4003 is renewed on master PAC #4001. | 1) Check if the dead device information of car #4003 is renewed on master PAC #4001. | Pass |
| PA Audio Train Line level balancing | TC-PAC-ANALYSIS-04-01 | Turn on CRA(MA) and CAB port signal of external interface signal test board in car #4001. | 1) During double heading, 11Khz tone occurred is viewed on train line packet viewer. | 1) Check that 11Khz tone occurred is viewed on train line packet viewer during double heading. | Pass |
|  | TC-PAC-ANALYSIS-04-02 | Change the resistance of volume gain on external interface signal test board. Load the program which volume gain can’t be adjusted with respect to voltage values. Reboot PAC board. Load the program which volume gain can be adjusted with respect to voltage values. Change the resistance of volume gain on external interface signal test board. Reboot PAC board. | Load on audio line of train line. Test the program which doesn’t balance the level before the test. When double heading is complete, check that CNCS default volume and PAC Amp volume are the same. Load the final version of program to test. Load on audio line of train line. After double heading and level balance applied, check that the PAC Amp volume is increased on carline packet viewer. | 1) Check if it Loads on audio line of train line. 2) Check if the program not balanced level is tested in advance for the test. 3) Check if CNCS default volume and PAC Amp volume are the same after the completion of double heading. 4) Check if the program is load in order to test the program completed. 5) Check if it loads audio line of train line. 6) Check if PAC Amp volume is increased on Carline viewer by applying the level balance after the completion of double heading. | Pass |
| Monitor Consist Integrity | TC-PAC-ANALYSIS-05-01 | Turn on CRA(MA) and CAB port signal of external interface signal test board in car #4001. 2) Turn off CAB port signal of external interface signal test board in car #4001. | It’s shown in carline packet viewer that CAB is on at CAR in car #4001 and if train is connected with #4001, #4002, #4003 and #4004. The information of double headed train is set to initial value 10 sec after CAB signal is off which is viewed on carline packet viewer. | 1) Check If the train consists of #4001, #4002, #4003 and #4004 with cab on car #4001 CRA(MB) in CarLine viewer. 2) Check if The information of double headed train is initialized in Carline viewer 10sec after CAB signal is off. | Pass |
|  | TC-PAC-ANALYSIS-05-02 | Turn on CAR and CAB port signal of external interface signal test board in car #4001. Change coupler port signal of external interface signal test board on TOC carline #4002. | It’s shown in carline packet viewer that CAB is on at CRA(MA) on car #4001 and if train is connected with #4001, #4002, #4003 and #4004. On double heading, if coupler signal of HOC or TOC is changed, then it’s shown in train line packet viewer that re-double heading begins. | 1) Check if the train consists of #4001, #4002, #4003 and #4004 with cab on car #4001 CRA(MA) in Carline viewer. 2) Check if re-double heading begins in the train line packet viewer when coupler signal of HOC or TOC is changed. | Pass |
| Monitor Intercom Status | TC-PAC-ANALYSIS-06-01 | Turn in DPO live signal of external interface signal test board. Turn off DPO live signal of external signal test board. | It is checked on carline packet viewer that DPO live bit of device status tables is 0. It is checked on carline packet viewer that DPO live bit of device status tables is 1. | 1) Check if DPO Live bit of Device Status Tables becomes '0' on Carline Packet viewer. 2) Check if DPO Live bit of Device Status Tables becomes '1' on Carline Packet viewer. | Pass |
|  | TC-PAC-ANALYSIS-06-02 | Turn on DPH live signal of external interface signal test board. Turn off DPH live signal of external interface signal test board. | It is checked on carline packet viewer that DPH live bit of device status tables is 0. It is checked on carline packet viewer that DPH live bit of device status tables is 1. | 1) Check if DPH Live bit of Device Status Tables becomes '0' on Carline Packet viewer. 2) Check if DPH Live bit of Device Status Tables becomes '1' on Carline Packet viewer. | Pass |
|  | TC-PAC-ANALYSIS-06-03 | Turn on PEI live signal of external interface signal test board. Turn off PEI live signal of external interface signal test board. | It is checked on carline packet viewer that PEI live bit of device status tables is 0. It is checked on carline packet viewer that PEI live bit of device status tables is 1. | 1) Check if PEI Live bit of Device Status Tables becomes '0' on Carline Packet viewer. 2) Check if PEI Live bit of Device Status Tables becomes '1' on Carline Packet viewer. | Pass |
| Process Local PEI Address Status Interrupt | TC-PAC-ADDRESS-01-01 | Turn on PEI address signal of external interface signal test board. Turn off PEI address signal of external signal test board. | It is checked on carline packet viewer that PEI address bit of device status tables is 1. It is checked on carline packet viewer that PEI address bit of device status tables is 0. | 1) Check if PEI Address bit of Device Status Tables becomes '1' on Carline Packet viewer. 2) Check if PEI Address bit of Device Status Tables becomes '0' on Carline Packet viewer. | Pass |
| Query Train Line PEI Address Status | TC-PAC-ADDRESS-02-01 | Turn on CRA(MA) and CAB port signal of external interface signal test board. Turn on PEI address signal of external interface signal test board. | It’s shown in carline packet viewer that CAB is on at CAR on car #4001 and train is connected with #4001, #4002, #4003 and #4004. It’s shown in carline packet viewer that PEI address information called form car #4001 is renewed on car #4003. | 1) Check if the train consists of #4001, #4002, #4003 and #4004 with cab on car #4001 CRA(MA) in Carline packet viewer. 2) Check if it is shown in carline packet viewer that PEI address information called form car #4001 is renewed on car #4003. | Pass |
|  | TC-PAC-ADDRESS-02-02 | Turn on PEI address signal of external interface signals test board at car #4003 or #4004. | It is checked in carline packet viewer that PEI address information called from car #4003 or #4004 is renewed on master PAC #4001. | 1) Check if it is checked in carline packet viewer that PEI address information called from married car #4003 or #4004 is renewed on master PAC #4001. | Pass |
| Monitor & Process AVL Trigger Events | TC-PAC-INITAVL-01-01 | Turn on CRA(MA) and CAB port signal of external interface signal test board in car #4001. Create CCP packet which the trigger point is change on the packet simulation program of car #4001 and send it via RS485. | It’s shown in carline packet viewer that CAB is on at CRA(MA) on car #4001 and train is connected with #4001, #4002, #4003 and #4004. Trigger point information recognized at master PAC in car #4001 is also renewed in car #4003. | 1) Check if it is shown in carline packet viewer that CAB is on at CRA(MA) on car #4001 and train is connected with #4001, #4002, #4003 and #4004. 2) Check if trigger point information recognized at master PAC in car #4001 is also renewed in car #4003. | Pass |

CCP

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| CNCS Interface | TC- CCP-Initial-01 | Power 입력 상태 확인.(DC 37.5 V) | 1. Approve the power to the system. | 1. Booting by inputting a logo on the screen then CCP program is operated. | Pass |
|  |  | Signal Input 확인. | 2. Approve the signals. | 2.“Press Train to enter Train Number”. A string is displayed. | Pass |
|  | TC- CCP Control-01 | Cab Signal 입력 확인 및 운행 DATA 확인. | 1. Input the Train number valid | 1. Load the driving information for Train Number in the memory and display on the screen. | Pass |
|  |  | Cab Signal 입력 확인 및 운행 DATA 확인. | 2. Input the Train number invalid | 2. Display ”Invalid Train Number” | Pass |
|  | TC- CCP Control-02 | Cab Signal 입력 확인 및 운행 DATA 확인. | 1. Input CREW ID on the input-screen. | 1. CREW ID is loaded and “In Service” is displayed | Pass |
|  | TC- CCP Control-03 | 485 통신 Port 확인 및 입력 데이터 전송 상태 확인. | 1. Input the Train number valid. | 1. The Train Number which is input in PAC and the communication packet is transmitted by applying. |  |
|  |  | 485 통신 Port 확인 및 입력 데이터 전송 상태 확인. | 2. Press the SKIP button of a specific station on the screen. | 2. No automatic announcement for the skip button applied station when a simulation is operated. |  |
|  |  | 485 통신 Port 확인 및 입력 데이터 전송 상태 확인. | 3. Press the EXP button of a specific station on the screen. | 3. No automatic announcement for the EXP button applied the section of a station when a simulation is operated. |  |
|  |  | 485 통신 Port 확인 및 입력 데이터 전송 상태 확인. | 4. Press the STOP button of a specific station on the screen. | 4. Automatic announcement for the STOP button applied station when a simulation is operated. |  |
|  | TC- CCP Control-04 | 485 통신 Port 확인 및 입력 데이터 전송에 따른 PA System Volume 제어 확인. | 1. Press the NORMAL button on the CONFIG-SPEAKER VOLUME screen. | 1. The speaker volume is adjusted by the normal level when automatic announcement. | Pass |
|  |  | 485 통신 Port 확인 및 입력 데이터 전송에 따른 PA System Volume 제어 확인. | 2. Press the SOFT button on the CONFIG-SPEAKER VOLUME screen. | 2. The speaker volume is adjusted by the SOFT level when automatic announcement. |  |
|  |  | 485 통신 Port 확인 및 입력 데이터 전송에 따른 PA System Volume 제어 확인. | 3. Press the MUTE button on the CONFIG-SPEAKER VOLUME screen. | 3. No sounds are input when automatic announcement. |  |
|  |  | 485 통신 Port 확인 및 입력 데이터 전송에 따른 PA System Volume 제어 확인. | 4. Adjust a scroll bar on the CONFIG – LCD DIMMER screen. | 4. The brightness of the screen is adjusted by the direction of the scroll bar. |  |
|  |  | 485 통신 Port 확인 및 입력 데이터 전송에 따른 PA System Volume 제어 확인. | 5. After change of speaker volume and screen brightness, press the RESET button. | 5. After the reoperation of the system the screen brightness value is set as the initial value. |  |
|  | TC- CCP Control-05 | 485 통신 Port 확인 및 중연 접점 상태 확인. | 1. Check the CHECK screen after forming a single car. | 1. Only one car is shown as an icon. The rest of the car area is filled with white empty space. | Pass |
|  |  | 485 통신 Port 확인 및 중연 접점 상태 확인. | 2. Check the CHECK screen after the forming a multi car. | 2. The number of the composed cars is shown as icons. The rest of the car area is filled with white empty space. |  |
|  |  | 485 통신 Port 확인 및 중연 접점 상태 확인. | 3. Remove the power of the specific car formation device. | 4. The icon color of the car with relevant devices turns red. If click a car icon, the device removed the power becomes red. |  |
|  | TC- CCP Control-06 | CCP DB DATA 확인. | 1. Click the lists of the screen one by one. | 1. The relevant information of the lists chosen is displayed. | Pass |
|  | TC- CCP Control-07 | 485 통신 Port 확인 및 인터콤 연결 상태 확인. | 1. Press the PEI CALL button in a specific car after forming a multi car. | 1. The PEI button color is changed when the message including the car number with the button pushed on the CCP screen is displayed | Pass |
|  | TC- CCP Control-08 | 485 통신 상태 확인 및 내부 메모리 확인. | 1. Update the DB file by using CNCS Content Update program. | 1. The file updated from CNCS is stored in CCP storage device. | Pass |
|  | TC- CCP Control-09 | 485 통신 상태 확인 및 내부 메모리 확인. | 1. Update the DB file by using CNCS Content Update program. | 1. The file updated from CNCS is stored in CCP storage device. | Pass |
|  | TC- CCP Control-10 | 485 통신 상태 확인 및 내부 메모리 확인. | 1. Update the DB file by using CNCS Content Update program. | 1. The file updated from CNCS is stored in CCP storage device. | Pass |
|  | TC- CCP Display-01 | 485 통신 Port 상태 확인 및 DB DATA 확인. | 1. Input the normal Train Number. | 1. Departure, Actual and next station names for the Train number input by the main user on the main screen are displayed. | Pass |
|  | TC- CCP Display-02 | 485 통신 Port 상태 확인 및 DB DATA 확인. | 1. Input the Train Number and execute the Simulation. | 1. Automatic announcement is output by the schedule. 2. Check If Actual stations on CNCS screen are changed by a schedule. | Pass |
|  |  | 485 통신 Port 상태 확인 및 DB DATA 확인. | 1. Input the Train Number and transmit GIA packet to CCP by checking the TP information of the schedule input. | 1. Automatic announcement is output by the location of GIA packet transmitted. |  |
|  | TC- CCP Display-03 | 485 통신 Port 상태 확인 및 GIA 통신 DATA 수신 상태 확인. | 1. Input the Train Number and transmit GIA packet to CCP by checking the TP information of the schedule input. | 1. Automatic announcement is output by the location of GIA packet transmitted and the station information of the main screen is displayed with a change. | Pass |

CNCS

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| CNCS Interface | TC-CNCS Launcher-01 | Add a normal update file in the update folder. | In case of normal performance, the file is updated. | The file version has been modified. | Pass |
|  |  | Add an abnormal update file in the update folder. | In case of normal performance, the file is not updated. | file version has not been modified. | Pass |
|  | TC-CNCS Main-Initial-01 | Open the 485 communication port. | In case of normal performance, CNCS will ordinarily respond to the requirement of PAC. | CNCS is displayed as “OK” on the right side of CCP. | Pass |
|  |  | Open the UDP port. | In case of normal performance, Sub-Component should be monitored. | Process Monitor State of the Main Screen shows the state of Sub-Component. | Pass |
|  | TC-CNCS Main-Monitoring-01 | 1. Close Sub-Component program. | 1. After 5 minutes, processor should ordinarily be operated. | 1. The processor operates ordinarily after 5 minutes. | Pass |
|  | TC-CNCS Main-RS485-01 | 1. Check CNCS state on the right side of screen. | 1. In case of normal performance, CNCS will ordinarily respond to the requirement of PAC. | 1. CNCS is displayed as “OK” on the right side of CCP. | Pass |
|  | TC-CNCS PP-Initial-01 | 1. Switch on CNCS and wait until the content plays by operating PP. | 1. advertisement video or a still image normally play on the LCD Display screen and confirm if the PP state is 2 when stopped. | 1. The contents was playing on the LCD Display screen and the state of Main.exe was 2. | Pass |
|  | TC-CNCS PP-ContentPlay-01 | 1. Add the driving information (car and crew numbers) in CCP and test it by using simulation functions. | 1. Observe If the passenger guide information is displayed by the schedule with car numbers. | 1. The passenger guide message was displayed and a video and a still image were playing After the time appointed. | Pass |
|  | TC-CNCS SP-Initial-01 | 1. Switch on CNCS and wait until UDP communicates by operating SP. | 1. Check if SP state of Main.exe is 2. | 1. SP state of Main.exe was normally operated as 2. | Pass |
|  | TC-CNCS SP-ContentPlay-01 | 1. Add the driving information (car and crew numbers) in CCP and test it by using simulation functions. | 1. Observe If the message and contents are displayed well. | 1. The passenger guide message was displayed. | Pass |
|  | TC-CNCS CDT Data-Initial-01 | 1. Open a communication port for LIC. | 1. LIC response should be confirmed on the CDT Data screen. | 1. The response is conformed on the CDT Data screen. | Pass |
|  | TC-CNCS CDT Data-RS232-01 | 1. Request CDT Date file from LIC. | 1. The CDT data file from LIC should be created. | 1. The CDT data file from LIC was created. | Pass |
|  | TC-CNCS CDT Data-RS232-02 | 1. Transmit CNCS time information to LIC due to request from LIC. | 1. The time information of CNCS should be transmitted to LIC. | 1. The time information should be confirmed on the CDT Data Screen. | Pass |
|  | TC-CNCS CDT Data-RS232-03 | 1. Transmit Communication System version information be LIC request. | 1. The version information of CNCS should be transmitted to LIC. | 1. The version information should be confirmed on the CDT Data Screen. | Pass |
|  | TC-CNCS FTP-Initial-01 | 1. Open UDT Socket. | 1. The normal state of FTP should be displayed on the Main Process. | 1. The normal state of FTP was displayed on the Main Process. | Pass |
|  | TC-CNCS FTP-Wayside-01 | 1. Perform Ping test with FTP Server. | 1. If FTP Server is normal, the Wayside Icon of CCP should be displayed with green color. | 1. When FTP Server is normal, the Wayside Icon of CCP was displayed with green color. | Pass |
|  | TC- REQ-CNCS FTP-Update-01 | 1. Add CDT data to be transmitted to FTP Server into FTP Upload folder. | 1. If the target file is transmitted normally, two file have the same contents. | 1. The target file and uploaded file into the FTP Server are same. | Pass |
|  | TC-CNCS FTP-Update-02 | 1. Add a data file to be transmitted to CNCS into FTP Server. | 1. The target file and downloaded file into the CNCS should be same. | 1. The target file and downloaded file into the CNCS are same. | Pass |
|  | TC-CNCS ContentUpdate-RS485-01 | 1. Using RS-485 communication, obtain a communication authority from PAC and update each relevant sub-component. | 1. Data should ordinarily be transmitted and received via Car Line. | 1. RS-485 transmitting and receiving states were displayed normally. | Pass |
|  | TC-CNCS ContentUpdate-Update-01 | 1. Copy a new data of CCP into the specified folder. | 1. New data of CCP should be recognized normally in the Content Update Process. | 1. New data of CCP was recognized normally in the Content Update Process. | Pass |
|  |  | 2. Copy a new MP3 data of PAC into the specified folder. | 2. New data of PAC should be recognized normally in the Content Update Process. | 1. New data of PAC was recognized normally in the Content Update Process. | Pass |
|  |  | 3. Copy a new data of LED into the specified folder. | 3. New data of LED should be recognized normally in the Content Update Process. | 3. New data of LED was r ecognized normally in the Content Update Process. | Pass |
|  | TC-CNCS ContentUpdate-Update-02 | 1. Update CCP contents. | 1. If simulation is performed on the CCP, the updated data should be shown. | 1. The updated data is shown normally. | Pass |
|  | TC-CNCS ContentUpdate-Update-03 | 1. Update PAC contents. | 1. If simulation is performed on the CCP, the updated data should be announced.. | 1. The updated data is announced normally. | Pass |
|  | TC-CNCS ContentUpdate-Update-04 | 1. Update LED contents. | 1. If simulation is performed on the CCP, the updated data should be shown. | 1. The updated data is normally shown on the LED screen. | Pass |

LEDC

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| LEDC Signal Interface | TC-FDI-Initial-01 | 1. After provide the power, set the initial value and ID of device and open the RS-485 port. | 1. In case of normal operation, the LED of LEDC turns on. | 1. LEDC's LED for check was on. | Pass |
|  | TC-FDI-Comm-01 | 1. Push the CHECK button of CCP. | 1. In case of normal operation, the relevant FDI Unit indicates as green color, on the other hand, it indicates an red. | 1. The relevant FID Unit was displayed to green. | Pass |
|  | TC-FDI-Display-01 | 1. After input CREW ID and train number using CCP GUI, push the simulation button. | 1. In case of normal operation, the destination which is input, is displayed on the LED module(LDM) of the relevant FDI Unit | 1. The last station input in LED module(LDM) of the relevant FDI Unit was displayed. | Pass |
|  | TC-FDI-Update-01 | 1. Copy new images then add them in Update folder in CNCS , execute Update command , Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The new image updated in LED module(LDM) of the relevant FDI Unit is displayed. | 1. The new station image updated in LED module(LDM) of the relevant FDI Unit was displayed. | Pass |
|  | TC-SDI-Initial-01 | 1. Set the device initial value and ID and open RS-485 communication port after power comes. | 1. LEDC's LED for check is on when normally operated. | 1. LEDC's LED for check was on.. | Pass |
|  | TC-SDI-Comm-01 | 1. Push the CHECK button of CCP. | 1. The relevant SDI Unit becomes green if normally operated. If not, it is displayed to red. | 1. The relevant SDI Unit was displayed to green. | Pass |
|  | TC-SDI-Display-01 | 1. Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The last station input in LED module(LDM) of the relevant SDU Unit is displayed when normally operated. | 1. The last station input in LED module(LDM) of the relevant SDU Unit was displayed. | Pass |
|  | TC-SDI-Update-01 | 1. Copy new images then add them in Update folder in CNCS , execute Update command , Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The new image updated in LED module(LDM) of the relevant SDI Unit is displayed. | 1. The new station image updated in LED module(LDM) of the relevant SDI Unit was displayed. | Pass |
|  | TC-PII-Initial-01 | 1. Set the device initial value and ID and open RS-485 communication port after power comes. | 1. LEDC's LED for check is on when normally operated. | 1. LEDC's LED for check was on. | Pass |
|  | TC-PII-Comm-01 | 1. Push the CHECK button of CCP. | 1. The relevant PII Unit becomes green if normally operated. If not, it is displayed to red. | 1. The relevant PII Unit was displayed to green. | Pass |
|  | TC-PII-Display-01 | 1. Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The text message is displayed on LED module(LDM) of the relevant PII Unit when normally operated. | 1. The text message was displayed on LED module(LDM) of the relevant PII Unit. | Pass |
|  | TC-PII-Update-01 | 1. Copy new images then add them in Update folder in CNCS , execute Update command , Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The new text message updated is displayed on LED module(LDM) of the relevant PII Unit when normally operated. | 1. The new text message updated was displayed on LED module(LDM) of the relevant PII Unit | Pass |

LIC

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| LIC Interface | TC-LIC-Main-Comm-01 | 1. It can be checked boot process by using 232 monitoring program after connection RS-232 serial port. | 1. It indicates device name and program version during normal operations. | 1. The device operations status can confirmed by using console command. | Pass |
|  |  | 2. Abnormal update file are placed in designated update folder. | 2. The relevant update files are not updated during normal operations. | 2. It didn’t change relevant program’s version. | Pass |