| THE DRAWING(S) AND THE INFORMATION CONTAINED HEREIN ARE THE PROPERTY OF WOOJIN INDUSTRIAL SYSTEMS CO., LTD. THEY SHALL NOT BE DISCLOSED, REPRODUCED OR USED, IN ANY MANNER WITHOUT THE PRIOR WRITTEN CONSENT OF WOOJIN INDUSTRIAL SYSTEMS CO., LTD. |  | SEPTA Silverliner V Communication System Software Test Report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DISTRIBUTION |  | R E V I S I O N |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | Oct 14, 2011 |  | Issued |  |  | J.Y.LEE | J.Y.KWON | B.J.JONE | E.K.LEE |
|  |  |  | No. | ISSUED DATE |  | REVISION NOTE |  |  | DESIGNED | CHECKED | APPROVED | REVIEWED |
|  |  |  |  |  |  |  |  |  | ENGINEERING |  |  | PM |
|  |  | PM |  |  | REVIEWED |  | E.K.LEE |  | CUSTOMER : |  |  |  |
|  |  | ENGINEERING |  |  | APPROVED |  | B.J.JONE |  | HYUNDAI-ROTEM COMPANY |  |  |  |
|  |  |  |  |  | CHECKED |  | J.Y.KWON |  |  |  |  |  |
|  |  |  |  |  | DESIGNED |  | J.Y.LEE |  | PROJECT : |  |  |  |
|  |  | ISSUED DATE |  |  | October 14, 2011 |  |  |  | SEPTA SILVERLINER V COMMUNICATION SYSTEM |  |  |  |
|  |  | ISSUED BY |  |  | R & D CENTER |  |  |  |  |  |  |  |
|  |  | PROJECT NO. |  |  | P2009-024 |  |  |  |  |  |  |  |
|  |  | ORDER NO. |  |  | P091201400 |  |  |  |  |  |  |  |
|  |  | DOCUMENT NO. |  |  |  |  |  | REVISION |  |  |  |  |
|  |  | WJ-SEPTA-STR-V1.0 |  |  |  |  |  | 1 |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1.0	Purpose	5

2.0	Reference Documents	5

3.0	Summary	5

3.1	GIA	5

3.1.1	Fun-REQ-GIA-Initializer-01 Test	5

3.1.2	Fun-REQ-GIA-Comm-01 Test	6

3.2	CCP	6

3.2.1	Fun-REQ-CCPInitial_01	6

3.2.2	Fun-REQ-CCP Main-Control-01	6

3.2.3	Fun-REQ-CCP Main-Control-02	6

3.2.4	Fun-REQ-CCP Main-Control-03	6

3.2.5	Fun-REQ-CCP Main-Control-04	6

3.2.6	Fun-REQ-CCP Main-Control-05	6

3.2.7	Fun-REQ-CCP Main-Control-06	7

3.2.8	Fun-REQ-CCP Main-Control-07	7

3.2.9	Fun-REQ-CCP Main-Control-08	7

3.2.10	Fun-REQ-CCP Main-Control-09	7

3.2.11	Fun-REQ-CCP Main-Control-10	7

3.2.12	Fun-REQ-CCP Main-Control-11	7

3.2.13	Fun-REQ-CCP Main-Control-12	7

3.2.14	Fun-REQ-CCP Main-Display-01	8

3.2.15	Fun-REQ-CCP Main-Comm-02	8

3.2.16	Fun-REQ-CCP Main-Comm-03	8

3.3	PAC	8

3.3.1	Fun-REQ-PAC-Initial-01 Test	8

3.3.2	Fun-REQ-PAC-Analysis-01 Test	8

3.3.3	Fun-REQ-PAC-Analysis-02 Test	9

3.3.4	Fun-REQ-PAC-Analysis-03 Test	9

3.3.5	Fun-REQ-PAC-Analysis-04 Test	9

3.3.6	Fun-REQ-PAC-Analysis-05 Test	9

3.3.7	Fun-REQ-PAC-Analysis-06 Test	9

3.3.8	Fun-REQ-PAC-Address-01 Test	9

3.3.9	Fun-REQ-PAC-Address-02 Test	10

3.3.10	Fun-REQ-PAC-Initial AVL-01 Test	10

3.4	VRX	10

3.4.1	Fun-REQ-VRX-Initial-01 Test	10

3.4.2	Fun-REQ-VRX-Comm-01 Test	10

3.5	VTX	10

3.5.1	Fun-REQ-VTX-Initial-01 Test	10

3.5.2	Fun-REQ-VTX-Comm-01 Test	11

3.6	LEDC	11

3.6.1	Fun-REQ-FDI-Initial-01 test	11

3.6.2	Fun-REQ-FDI-Comm-01 test	11

3.6.3	Fun-REQ-FDI-Display-01 test	11

3.6.4	Fun-REQ-FDI- Update -01 test	11

3.6.5	Fun-REQ-SDI-Initial-01 test	11

3.6.6	Fun-REQ-SDI-Comm-01 test	12

3.6.7	Fun-REQ-SDI-Display-01 test	12

3.6.8	Fun-REQ-SDI- Update -01 test	12

3.6.9	Fun-REQ-PII-Initial-01 test	12

3.6.10	Fun-REQ-PII-Comm-01 test	12

3.6.11	Fun-REQ-PII-Display-01 test	12

3.6.12	Fun-REQ-PII- Update -01 test	12

3.7	TRIC	13

3.7.1	Fun-REQ-TRIC-Initial-01 test	13

3.7.2	Fun-REQ-TRIC-Comm-01 test	13

3.7.3	Fun-REQ-TRIC-FSK-01 test	13

3.7.4	Fun-REQ-TRIC-Control-01 test	13

3.7.5	Fun-REQ-TRIC-Message-01 test	13

3.8	CNCS	14

3.8.1	Fun-REQ-CNCS Launcher-01 test	14

3.8.2	Fun-REQ-CNCS Main-Initial-01 test	14

3.8.3	Fun-REQ-CNCS Main-Monitoring-01 test	14

3.8.4	Fun-REQ-CNCS Main-RS485-01 test	14

3.8.5	Fun-REQ-CNCS PP-Initial-01 test	14

3.8.6	Fun-REQ-CNCS PP-ContentPlay-01 test	14

3.8.7	Fun-REQ-CNCS SP-Initial-01 test	15

3.8.8	Fun-REQ-CNCS SP-ContentPlay-01 test	15

3.8.9	Fun-REQ-CNCS CDT Data-Initial-01 test	15

3.8.10	Fun-REQ-CNCS CDT Data-RS232-01 test	15

3.8.11	Fun-REQ-CNCS CDT Data-RS232-02 test	15

3.8.12	Fun-REQ-CNCS CDT Data-RS232-03 test	15

3.8.13	Fun-REQ-CNCS FTP-Initial-01 test	15

3.8.14	Fun-REQ-CNCS FTP-Wayside-01 test	16

3.8.15	Fun- REQ-CNCS FTP-Update-01 test	16

3.8.16	Fun-REQ-CNCS FTP-Update-02 test	16

3.8.17	Fun-REQ-CNCS ContentUpdate-Initial-01 test	16

3.8.18	Fun-REQ-CNCS ContentUpdate-RS485-01 test	16

3.8.19	Fun-REQ-CNCS ContentUpdate-Update-01 test	16

3.8.20	Fun-REQ-CNCS ContentUpdate-Update-02 test	16

3.8.21	Fun-REQ-CNCS ContentUpdate-Update-03 test	17

3.8.22	Fun-REQ-CNCS ContentUpdate-Update-04 test	17

3.9	LIC	17

3.9.1	Fun-REQ-LIC Main-01	17

4.0	Variances	17

5.0	Summary of Result	17

6.0	Attachment. Test Execution Trace Report	0

6.1	GIA	0

6.2	CCP	1

6.3	PAC	4

6.4	VRX	11

6.5	VTX	11

6.6	LEDC	12

6.7	TRIC	14

6.8	CNCS	16

6.9	LIC	19

Revision Records

| Revision | Description | Date |
| --- | --- | --- |
| 0 | First Issued | 2011.10.14 |

- Purpose

The purpose of this document is to report the test result SEPTA Communication System software.

- Reference Documents

The following documents were used as sources of information for this test report.

| Publisher | Document Number | Title |
| --- | --- | --- |
| SEPTA | Conformed Version April, 2009 | TECHNICAL SPECIFICATION FOR THE PURCHASE OF SILVERLINER V ELECTRIC MULTIPLE UNIT COMMUTER RAIL CARS, “Southeastern Pennsylvania Transportation Authority New Train Programs Conformed Version DATE April, 2009” |
| Rotem | REDP10013 (R1) | Procurement Technical Specification Communication System For SEPTA Conformed Version DATE 3 Aug. 2006 |
| WOOJIN | WJ-SRS-001_r2 | SEPTA Silverliner V Communication System Software Requirements Specification |
|  | WJ-SRS-SDD-V1.0 | SEPTA Silverliner V Communication System Software Design Description |
|  | WJ-SEPTA-STP-V1.0 | SEPTA Silverliner V Communication System Software Test Plan |

- Summary

All features of communications systems SW were tested.

Test was executed in 2010.04.11 ~ 2010.04.15 by Y.J.KWON. Test was started with SW version 1.0.0.1, and this program passed all the tests.

GIA

It is connected to the antenna in order to bring the coordinates from the satellite to DR+GPS module in GPA for the test. It consists of a laptop and USB to 485 to transmit 485 request packets to GIA board and performs the test to transmit the request packets to GIA board from the laptop.

Fun-REQ-GIA-Initializer-01 Test

Fun-REQ-GIA-Initializer-01 features passed all the tests.

Fun-REQ-GIA-Initializer-01 features was tested to validate GIA Initializing function satisfaction.

The following test documents are associated with this test.

- SEPTA Software Test Plan section 12.1.1

Fun-REQ-GIA-Comm-01 Test

Fun-REQ-GIA-Comm-01 features passed all the tests.

Fun-REQ-GIA-Comm-01 features was tested to validate GIA Comm function satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.1.2

CCP

CRA Rack  with CCP, PAC, CNCS, FDI, SDI, PII, TRIC should be composed for CCP software test.

Following the power approval, CAB ON signals should be input and RS-485 communication line should be connected by operating CCP program.

Fun-REQ-CCPInitial_01

Fun-REQ-CCPInitail-01 features passed all the tests.

Fun-REQ-CCPInitial-01 is for the purpose of verifying if the function of Initial Process satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section  12.2.1

Fun-REQ-CCP Main-Control-01

Fun-REQ-CCPControl-01 features passed all the tests.

Fun-REQ-CCP Control -01 is for the purpose of verifying if the car number suitability judgment function satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section  12.2.2

Fun-REQ-CCP Main-Control-02

Fun-REQ-CCPControl-02features passed all the tests.

Fun-REQ-CCPControl-02 is for the purpose of verifying if the function of CREW ID Process satisfies the requirements. The following test documents are associated with this test.

SEPTA Software Test Plan section 12.2.3

Fun-REQ-CCP Main-Control-03

Fun-REQ-CCPControl-03 features passed all the tests.

Fun-REQ-CCPControl-03 is for the purpose of verifying if the schedule management function satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.4

Fun-REQ-CCP Main-Control-04

Fun-REQ-CCP Control-04 features passed all the tests.

Fun-REQ-CCP Control-04 is for the purpose of verifying if the function to change settings satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.5

Fun-REQ-CCP Main-Control-05

Fun-REQ-CCP Control-05 features passed all the tests.

Fun-REQ-CCP Control-05 is for the purpose of verifying if the car monitoring function satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.6

Fun-REQ-CCP Main-Control-06

Fun-REQ-CCP Control-06 features passed all the tests.

Fun-REQ-CCP Control-06 is for the purpose of verifying if the Help function satisfies the requirements.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.7

Fun-REQ-CCP Main-Control-07

Fun-REQ-CCP Control-07 features passed all the tests.

Fun-REQ-CCP Control-07 is for the purpose of verifying if the PEI process function satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.8.

Fun-REQ-CCP Main-Control-08

Fun-REQ-CCP Control-08 features passed all the tests.

Fun-REQ-CCP Control-08 is for the purpose of verifying if the PEI process function satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.9.

Fun-REQ-CCP Main-Control-09

Fun-REQ-CCP Control-09 features passed all the tests.

Fun-REQ-CCPControl-09는CAB MESSAGE. Fun-REQ-CCPControl-09 is for the purpose of verifying if the function to output and delete satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.10.

Fun-REQ-CCP Main-Control-10

Fun-REQ-CCP Control-10 features passed all the tests.

Fun-REQ-CCP Control-10 is for the purpose of verifying if the function to update data satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.11.

Fun-REQ-CCP Main-Control-11

Fun-REQ-CCP Control-11 features passed all the tests.

Fun-REQ-CCP Control-11 is for the purpose of verifying if the function to update data satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.12.

Fun-REQ-CCP Main-Control-12

Fun-REQ-CCP Control-12 features passed all the tests.

Fun-REQ-CCPControl-12 is for the purpose of verifying if the function to update data satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.13.

Fun-REQ-CCP Main-Display-01

Fun-REQ-CCPDisplay-01 features passed all the tests.

Fun-REQ-CCPDisplay-01 is for the purpose of verifying if the function to display stations information satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.14.

Fun-REQ-CCP Main-Comm-02

Fun-REQ-CCPComm-02 features passed all the tests.

Fun-REQ-CCPComm-02 is for the purpose of verifying if the function to transmit communication data and to process packets satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.15.

Fun-REQ-CCP Main-Comm-03

Fun-REQ-CCP Comm-03 features passed all the tests.

Fun-REQ-CCP Comm-03 is for the purpose of verifying if the GPS search function satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.2.16.

PAC

Connect J-TAG debugger to PAC board and change the software variable for the test lists to check the result or connect the packet simulation program to RS485 and change an input value to check a RS485 output value.

- External Interface Signals test :Operate an input signal for the test lists and check a RS485 output value by connecting the External Interface Signals test board operating Input and Output port signals which are input to PAC board .

- Multi coupled unit process test: Connect at least four PAC boards then test to check the result value by using RS485.

- Communication test : Check the communication result by connecting a speaker.

- Polling test  : Test by connecting the other device packet simulation program in the communication system to RS485.

Fun-REQ-PAC-Initial-01 Test

Fun-REQ-PAC-Initial-01 features passed all the tests.

Fun-REQ-PAC-Initial-01 features was tested to validate ‘PAC Initialization’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.1.

Fun-REQ-PAC-Analysis-01 Test

Fun-REQ-PAC-Analysis-01 features passed all the tests.

Fun-REQ-PAC-Analysis-01 features was tested to validate ’Check the Status of all Devices within a specific Car’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.2.

Fun-REQ-PAC-Analysis-02 Test

Fun-REQ-PAC-Analysis-02 features passed all the tests.

Fun-REQ-PAC-Analysis-02 features was tested to validate ’Consist Inauguration’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.3.

Fun-REQ-PAC-Analysis-03 Test

Fun-REQ-PAC-Analysis-03 features passed all the tests.

Fun-REQ-PAC-Analysis-03 features was tested to validate ’ Retrieve the Device Status tables from all PRIMARY PAC’s’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.4.

Fun-REQ-PAC-Analysis-04 Test

Fun-REQ-PAC-Analysis-04 features passed all the tests.

Fun-REQ-PAC-Analysis-04 features was tested to validate ’ PA Audio Train Line level balancing’ satisfaction.

The following test documents are associated with this test.

S

EPTA PAC Software Test Plan section 12.3.5.

Fun-REQ-PAC-Analysis-05 Test

Fun-REQ-PAC-Analysis-05 features passed all the tests.

Fun-REQ-PAC-Analysis-05 features was tested to validate ’Monitor Consist Integrity’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.6.

Fun-REQ-PAC-Analysis-06 Test

Fun-REQ-PAC-Analysis-06 features passed all the tests.

Fun-REQ-PAC-Analysis-06 features was tested to validate ’ Monitor Intercom Status’  satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.7.

Fun-REQ-PAC-Address-01 Test

Fun-REQ-PAC-Address-01 features passed all the tests.

Fun-REQ-PAC-Address-01 features was tested to validate’ ‘Process Local PEI Address Status Interrupt’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.8.

Fun-REQ-PAC-Address-02 Test

Fun-REQ-PAC-Address-02 features passed all the tests.

Fun-REQ-PAC-Address-02 features was tested to validate’ Query Train Line PEI Address Status’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3.9.

Fun-REQ-PAC-Initial AVL-01 Test

Fun-REQ-PAC-Initial AVL-01 features passed all the tests.

A Fun-REQ-PAC-Initial AVL-01 feature was tested to validate ’Monitor & Process AVL Trigger Events’ satisfaction.

The following test documents are associated with this test.

SEPTA PAC Software Test Plan section 12.3

VRX

The  appropriate PID Unit with VTX, CCP, PAS's CRA Rack, CNCS, VTX and PAC should be composed for VRX Software test.

Following the power approval, it should be checked that the device initial value and ID are set up via the lighting of VRX''s LED for check. Besides, the normal operation status of PID Unit relevant should be identified by pressing the CHECK button of CCP.

Fun-REQ-VRX-Initial-01 Test

Fun-REQ-VRX-Initial-01 features passed all the tests.

Fun-REQ-VRX-Initial-01 features were tested to validate VRX Board Initialization Process satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.4.1.

Fun-REQ-VRX-Comm-01 Test

Fun-REQ-VRX-Comm-01 features passed all the tests.

Fun-REQ-VRX-Comm-01 features were tested to validate VRX status information satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.4.2.

VTX

PSA of CRA Rack, VTX and PAC should be composed and at least more than one PID Unit should be connected for VTX Software test.

Following the power approval, it should be checked that the device initial value is set up via VTX front LED. Besides, the normal operation status of the appropriate PID Unit connected should be identified by pressing the CHECK button of CCP.

Fun-REQ-VTX-Initial-01 Test

Fun-REQ-VTX-Initial-01 features passed all the tests.

A fun-REQ-VTX-Initial-01 feature was tested to validate VTX Board Initialization process satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.5.1.

Fun-REQ-VTX-Comm-01 Test

Fun-REQ-VTX-Comm-01 features passed all the tests.

A fun-REQ-VTX-Comm-01 feature was tested to validate VTX status information satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.5.2.

LEDC

FDI, SDI, PII Unit with LEDC, PSA of CRA Rack, CNCS and PAC should be composed for LEDC Software test.

Following the power approval, it should be checked that the device initial value and ID are set up as the front text of LED module(LDM). Besides, the normal operation status of FDI, SDI and PII Unit relevant should be identified by pressing the CHECK button of CCP.

Fun-REQ-FDI-Initial-01 test

Fun-REQ-FDI-Initial-01 features passed all the tests.

Fun-REQ-FDI-Initial-01 features was tested to validate FDI Initialization Process satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.1

Fun-REQ-FDI-Comm-01 test

Fun-REQ-FDI-Comm-01 features passed all the tests.

Fun-REQ-FDI-Comm-01 features was tested to validate FDI Status Request satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.2

Fun-REQ-FDI-Display-01 test

Fun-REQ-FDI-Display-01 features passed all the tests.

Fun-REQ-FDI-Display-01 features was tested to validate Display FDI Content satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.3

Fun-REQ-FDI- Update -01 test

Fun-REQ-FDI-Update-01 features passed all the tests.

Fun-REQ-FDI-Update-01 features was tested to validate FDI Content Update Management satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.4

Fun-REQ-SDI-Initial-01 test

Fun-REQ-SDI-Initial-01 features passed all the tests.

Fun-REQ-SDI-Initial-01 features was tested to validate SDI Initialization Process satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.5

Fun-REQ-SDI-Comm-01 test

Fun-REQ-SDI-Comm-01 features passed all the tests.

Fun-REQ-SDI-Comm-01 features was tested to validate Request for SDI status data satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.6

Fun-REQ-SDI-Display-01 test

Fun-REQ-SDI-Display-01 features passed all the tests.

Fun-REQ-SDI-Display-01 features was tested to validate Display SDI Content satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.7

Fun-REQ-SDI- Update -01 test

Fun-REQ-SDI-Update-01 features passed all the tests.

Fun-REQ-SDI-Update-01 features was tested to validate SDI Content Update Management satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.8

Fun-REQ-PII-Initial-01 test

Fun-REQ-PII-Initial-01 features passed all the tests.

Fun-REQ-PII-Initial-01 features was tested to validate PII Initialization Process satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.9

Fun-REQ-PII-Comm-01 test

Fun-REQ-PII-Comm-01 features passed all the tests.

Fun-REQ-PII-Comm-01 features was tested to validate PII Status Request satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.10

Fun-REQ-PII-Display-01 test

Fun-REQ-PII-Display-01 features passed all the tests.

Fun-REQ-PII-Display-01 features was tested to validate Display AVL Station Arrival and Departure Messages satisfaction.

The following test documents are associated with this test.

- SEPTA Software Test Plan section 12.6.11

Fun-REQ-PII- Update -01 test

Fun-REQ-PII-Update-01 features passed all the tests.

Fun-REQ-PII-Update-01 features was tested to validate PII Content Update Management satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.6.12

TRIC

RRDCC radio simulator, CCP, PSA of CRA and PAC should be basically composed for TRIC Software test.

Following the power approval, it should be checked that the device initial value and ID are set up via the lighting of TRIC's LEC for check. Besides, Besides, the normal operation status of TRIC Unit relevant should be identified by pressing the CHECK button of CCP.

Fun-REQ-TRIC-Initial-01 test

Fun-REQ-TRIC-Initial-01 features passed all the tests.

Fun-REQ-TRIC-Initial-01 features was tested to validate TRIC Initialization Process satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.7.1.

Fun-REQ-TRIC-Comm-01 test

Fun-REQ-TRIC-Comm-01 features passed all the tests.

Fun-REQ-TRIC-Comm-01 features was tested to validate Request for TRIC Status satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.7.2.

Fun-REQ-TRIC-FSK-01 test

Fun-REQ-TRIC-FSK-01 features passed all the tests.

Fun-REQ-TRIC-FSK-01 features was tested to validate Process the demodulated FSK data received from RRDCC satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.7.3.

Fun-REQ-TRIC-Control-01 test

Fun-REQ-TRIC-Control-01 features passed all the tests.

Fun-REQ-TRIC-Control-01 features was tested to validate Transmit an FSK encoded signal via the Train Radio Transceiver to the RRDCC satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.7.4.

Fun-REQ-TRIC-Message-01 test

Fun-REQ-TRIC-Message-01 features passed all the tests.

Fun-REQ-TRIC-Message-01 features was tested to validate Print Cab Text Message Using the CCP satisfaction.

The following test documents are associated with this test.

SEPTA Software Test Plan section 12.7.5.

CNCS

CRA Rack with CNCS and CCP and PID shall be required in order to perform CNCS Software test.

After supply the power, it shall be confirmed to operate CNCS program normally, and the state of CNCS and Wayside shall be confirmed their normal performances on the right bottom.

Fun-REQ-CNCS Launcher-01 test

Fun-REQ-CNCS Launcher-01 features passed all the tests.

Fun-REQ-CNCS Launcher-01 is intended to verify whether the function of Launcher Process satisfies requirements or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.

Fun-REQ-CNCS Main-Initial-01 test

Fun-REQ-CNCS Main-Initial-01 features passed all the tests.

Fun-REQ-CNCS Main-Initial-01 is intended to verify whether the initializing function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.2

Fun-REQ-CNCS Main-Monitoring-01 test

Fun-REQ-CNCS Main- Monitoring -01 features passed all the tests.

Fun-REQ-CNCS Main-Monitoring-01 is intended to verify whether the Sub-Component monitoring function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.3

Fun-REQ-CNCS Main-RS485-01 test

Fun-REQ-CNCS Main- RS485-01 features passed all the tests.

Fun-REQ-CNCS Main-RS485-01 is intended to verify whether the RS-485 communication function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.4

Fun-REQ-CNCS PP-Initial-01 test

Fun-REQ-CNCS PP-Initial-01 features passed all the tests.

Fun-REQ-CNCS PP-Initial-01 is for the purpose of verifying if the initialization function of PP satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.5

Fun-REQ-CNCS PP-ContentPlay-01 test

Fun-REQ-CNCS PP-ContentPlay-01 features passed all the tests.

Fun-REQ-CNCS PP-ContentPlay-01 is for the purpose of verifying if the contents play function of PP satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.6

Fun-REQ-CNCS SP-Initial-01 test

Fun-REQ-CNCS SP-Initial-01 features passed all the tests.

Fun-REQ-CNCS SP-Initial-01 is for the purpose of verifying if the initialization function of SP satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.7

Fun-REQ-CNCS SP-ContentPlay-01 test

Fun-REQ-CNCS SP-ContentPlay-01 features passed all the tests.

Fun-REQ-CNCS SP-ContentPlay-01 is for the purpose of verifying if the contents play function of SP satisfies the requirements. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.8

Fun-REQ-CNCS CDT Data-Initial-01 test

Fun-REQ-CNCS CDT Data-Initial -01 features passed all the tests.

Fun-REQ-CNCS Main-Initial-01 is intended to verify whether the initializing function of Main Process satisfies requirements or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.9

Fun-REQ-CNCS CDT Data-RS232-01 test

Fun-REQ-CNCS CDT Data-RS232-01 features passed all the tests.

Fun-REQ-CNCS CDT Data-RS232-01 is intended to verify whether the data selecting function of CDT satisfies requirements via RS-232 communication or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.10

Fun-REQ-CNCS CDT Data-RS232-02 test

Fun-REQ-CNCS CDT Data-RS232-02 features passed all the tests.

Fun-REQ-CNCS CDT Data-RS232-02 is intended to verify whether the data selecting function of CDT Data Process to CNCS satisfies requirements via RS-232 communication or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.11

Fun-REQ-CNCS CDT Data-RS232-03 test

Fun-REQ-CNCS CNCS CDT Data-RS232-03 features passed all the tests.

Fun-REQ-CNCS CDT Data-RS232-03 is intended to verify whether the communication S/W version information transmitting function of CDT Data Process satisfies requirements via RS-232 communication or not. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.12

Fun-REQ-CNCS FTP-Initial-01 test

Fun-REQ- CNCS FTP-Initial-01 features passed all the tests.

Fun-REQ-CNCS FTP-Initial-0 is intended to verify whether the initializing function of FTP Process satisfies requirements or not. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.13

Fun-REQ-CNCS FTP-Wayside-01 test

Fun-REQ- CNCS FTP-Wayside-01 features passed all the tests.

Fun-REQ-CNCS FTP-Wayside-01 is intended to verify whether the FTP-Wayside monitoring function of FTP Process satisfies requirements or not. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.14

Fun- REQ-CNCS FTP-Update-01 test

Fun-REQ-CNCS CNCS FTP-Update-01 features passed all the tests.

Fun-REQ-CNCS FTP-Update-01 is intended to verify whether the CNCS data transmitting function to FTP Server satisfies requirements or not. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.15

Fun-REQ-CNCS FTP-Update-02 test

Fun-REQ-CNCS CNCS FTP-Update-02 features passed all the tests.

Fun-REQ-CNCS CNCS FTP-Update-02 is intended to verify whether the FTP Server data receiving function to CNCS satisfies requirements or not. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.16

Fun-REQ-CNCS ContentUpdate-Initial-01 test

Fun-REQ-CNCS ContentUpdate-Initial-01 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Initial-01 is intended to verify whether the initializing function of Content Update Process satisfies requirements or not. The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.17

Fun-REQ-CNCS ContentUpdate-RS485-01 test

Fun-REQ-CNCS ContentUpdate-RS485-01 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-RS485-01 is intended to verify whether the RS-485 communication function of Content Update Process satisfies requirements or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.18

Fun-REQ-CNCS ContentUpdate-Update-01 test

Fun-REQ-CNCS ContentUpdate-Update-01 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Update-01 is intended to verify whether the sub-component update function of CCP, PAC and LED satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.19

Fun-REQ-CNCS ContentUpdate-Update-02 test

Fun-REQ-CNCS ContentUpdate-Update-02 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Update-02 is intended to verify whether the content update function of CCP satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.20

Fun-REQ-CNCS ContentUpdate-Update-03 test

Fun-REQ-CNCS CNCS ContentUpdate-Update-03 features passed all the tests.

Fun-REQ-CNCS CNCS ContentUpdate-Update-03 is intended to verify whether the MP3 content update function of PAC satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.21

Fun-REQ-CNCS ContentUpdate-Update-04 test

Fun-REQ-CNCS ContentUpdate-Update-04 features passed all the tests.

Fun-REQ-CNCS ContentUpdate-Update-04 is intended to verify whether the LED content update function of PAC satisfies requirements in Content Update Process or not.

The following test documents are associated whit this test.

SEPTA Software Test Plan section 12.8.22

LIC

Fun-REQ-LIC Main-01

Fun-REQ-LIC-01 features passed all the tests.

Fun-REQ-LIC-01is intended to verify whether Main Process function satisfies requirements or not. The following test documents are associated whit this test.

SEPTA Software Test Plan section  12.9.1.

- Variances

None

- Summary of Result

All features of PA and PIS software passed their tests.

- Attachment. Test Execution Trace Report

GIA

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| GIA Device Initialization Process | TC-GIA-Initializer-01 | Check the LED of GIA. | S1 LED becomes red when the power is on. | S1 LED became red when the power is on. | Pass |
|  | TC-GIA-Initializer-02 | Check 485 communication port of GIA. | Check if the communication port is open | It is check that communication port is open | Pass |
| GIA Device Status | TC- GIA-Comm-01 | Check the LED of GIA | - S1 LED becomes red when PAC receives the packet required. - S2 LED becomes yellow when the satellite receives coordinates in DR+GPS. | - S1 LED became red when the PAC receives the packet required. - S2 LED became yellow when the satellite receives coordinates in DR+GPS. | Pass |
|  | TC- GIA-Comm-02 | Check the packet coming out of GIA. | The normal packet is received in COM port by UART program. | The normal packet was received in COM port by UART program. | Pass |

CCP

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| CNCS Interface | TC-CNCS Launcher-01 | 1. Add a normal update file in the update folder appointed. | 1. The relevant update file is updated when a normal operation. | 1. The relevant program has been changed. | Pass |
|  |  | 2. Add an abnormal update file in the update folder appointed. | 2. The relevant update file is not updated when a normal operation. | 2. The relevant program has not been changed. | Pass |
|  | TC-CNCS Main-Initial-01 | 1. Open 485 communication port. | 1. CNCS normally responds to the request of PAC when a normal operation. | 1. CNCS is displayed as OK on the right screen of CCP. | Pass |
|  |  | 2. Open UDP Port. | 2. Sub Component should be monitored when a normal operation. | 2. CNCS is displayed as OK on the right screen of CCP. | Pass |
|  | TC-CNCS Main-Monitoring-01 | 1. Shut down Sub-Component program. | 1. The process should be normally operated in five minutes. | 1. The process which was shut was normally operated again in five minutes. | Pass |
|  | TC-CNCS Main-RS485-01 | 1. Check the status of CNCS on the right screen of CCP. | 1. CNCS is displayed as OK when a normal operation. | 1. OK is displayed on the right screen of CPP. | Pass |
|  | TC-CNCS PP-Initial-01 | 1. Switch on CNCS and wait until the contents play back by operating PP. | 1. Check if an advertisement video or a still image normally play on the LCD Display screen and confirm if the PP state is 2 when stopped. | 1. The contents was playing on the LCD Display screen and the state of Main.exe was | Pass |
|  | TC-CNCS PP-ContentPlay-01 | 1. Add the driving information (car and crew numbers) in CCP and test it by using simulation functions. | 1. Observe If the message and contents are displayed well. | 1. The message was displayed and a video and a still image were playing when the display time appointed was over | Pass |
|  | TC-CNCS SP-Initial-01 | 1. Switch on CNCS and wait until UDP communication is on by using SP. | 1. Check if SP State of Main.exe is 2. | 1. SP state of Main.exe was normally operated as 2. | Pass |
|  | TC-CNCS SP-ContentPlay-01 | 1. Switch on CNCS and until UDP communication is on by using SP. | 1. Observe If the passenger guide information is displayed by the schedule with car numbers. | 1. The passenger guide message was displayed. | Pass |
|  | TC-CNCS CDT Data-Initial-01 | 1. Open LIC and communication port. | 1. The response from the requirements of LIC should be confirmed on CDT Data screen. | 1. The response from the requirement of LIC was confirmed on CDT Data screen. | Pass |
|  | TC-CNCS CDT Data-RS232-01 | 1. Require CDT's data file from LIC. | 1. CDT's data file received from LIC should be created. | 1. CDT's data file received from LIC was created. | Pass |
|  | TC-CNCS CDT Data-RS232-02 | 1. Transmit CNCS' information to LIC by the requirement of LIC. | 1. CNCS' time information should be transmitted to LIC. | 1. Check if the time information is transmitted via CDT Data screen. | Pass |
|  | TC-CNCS CDT Data-RS232-03 | 1. Transmit the version information of PA system by the requirement of LIC. | 1. CNCS' version information should be transmitted to LIC. | 1. Check if the version information is transmitted via CDT Data screen. | Pass |
|  | TC-CNCS FTP-Initial-01 | 1. Open UDP Socket. | 1. FTP status should be normally marked in the Main Process. | 1. FTP status was normal in the Main Process. | Pass |
|  | TC-CNCS FTP-Wayside-01 | 1. Ping test with FTP Server. | 1. The Wayside icon of CCP should be marked to green when FTP Server is normal. | 1. The Wayside icon of CCP was marked to green when FTP Server is normal. | Pass |
|  | TC- REQ-CNCS FTP-Update-01 | 1. Add the CDT data in the FTP Upload folder to transmit to FTP Server | 1. The contents of the two files should be the same when the object file is normally transmitted to FTP Server. | 1. The contents of the two files should be the same when the object file is normally transmitted to FTP Server.. | Pass |
|  | TC-CNCS FTP-Update-02 | 1. Add the data file in the FTP Server to transmit to CNCS. | 1. The object file and the file downloaded to FTP Server should be the same. | 1. The object file and the file downloaded to FTP Server are the same. | Pass |
|  | TC-CNCS ContentUpdate-RS485-01 | 1. Acquire the communication authority and update each relevant sub device by using communication. | 1. The data should be normally received and transmitted via Car Line. | 1. The reception and transmission status of the data is normally displayed. | Pass |
|  | TC-CNCS ContentUpdate-Update-01 | 1. Copy new data to the folder appointed. | 1. New data of CCP should be normally read in the Content Update Process. | 1. New data of CCP was normally read in the Content Update Process. | Pass |
|  |  | 2. Copy new MP3 data of PAC to the folder appointed. | 2. New data of PAC should be normally read in the Content Update Process. | 1. New data of PAC was normally read in the Content Update Process. | Pass |
|  |  | 3. Copy new data of LED to the folder appointed. | 3. New data of LEC should be normally read in the Content Update Process. | 1. New data of LEC was normally read in the Content Update Process. | Pass |
|  | TC-CNCS ContentUpdate-Update-02 | 1. Perform the content update work of CCP. | 1. The updated data should be displayed when a simulation in CCP. | 1. The updated data was normally displayed on the CCP screen. | Pass |
|  | TC-CNCS ContentUpdate-Update-03 | 1. Perform the content update work of PCC. | 1. The updated data should be announced when a simulation in CCP. | 1. The updated data was normally announced on the CCP screen. | Pass |
|  | TC-CNCS ContentUpdate-Update-04 | 1. Perform the content update work of LED. | 1. The updated data should be displayed when a simulation in LED. | 1. The updated data was normally displayed on the LED screen. | Pass |

PAC

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| PAC Initialization | TC-PAC-INITIAL-01-01 | - Send CRA port signal from external interface single test board. - Send CRB port signal from external interface signal test board. | - It is checked that CRAB bit becomes 1 in carline packet viewer. - It is checked that CRAB bit becomes 0 in carline packet viewer. | 1) It is check that CRAB bit became '1' in carline packet viewer. 2) It is checked that CRAB bit became 0 in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-02 | - Change LIC packet in packet simulation program and send #700 car numbers thru RS485. - Change LIC packet in packet simulation program and send #800 car numbers thru RS485. | - Check if single-married byte is 1 in carline packet viewer. - Check if single-married byte is 2 in carline packet viewer. | 1) It is check Single-Married byte became '1' in carline packet viewer. 2) It is check Single-Married byte became '2' in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-03 | - Send CAB ON port signal from external interface signal test board. - Send CAB OFF port signal from external interface signal test board. | - It is checked that CAB bit is 1 in carline packet viewer. - It is checked that CAB bit is 0 in carline packet viewer. | 1) It is checked that CAB bit became' 1' in carline packet viewer. 2) It is checked that CAB bit became' 2' in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-04 | - Send coupler ON port signal from external interface signal test board. - Send coupler OFF port signal from external interface signal test board. | - Check if coupler bit 1 in carline packet viewer. - Check if coupler bit 0 in carline packet viewer. | - It is checked that coupler bit became '1' in carline packet viewer. 2) It is checked that coupler bit became '0' in carline packet viewer. | Pass |
|  | TC-PAC-INITIAL-01-05 | - Send CRA port signal of external interface signal test board on single car #700. - Send CRA port signal of external interface signal test board on married car #802. | - It works as a main PAC of single car and polling packets as many as single car device IDs(24) are observed. - It works as a mina PAC of married car and polling packets as many as married car device IDs(36) are observed. | 1) It works as a main PAC of single car and polling packets as many as single car device IDs(24) are observed. 2) It works as a mina PAC of married car and polling packets as many as married car device IDs(36) are observed. | Pass |
|  | TC-PAC-INITIAL-01-06 | - Set PAC internal program variable to #701 and #802 to double headed train. - Send CRA port single from external interface single test board. - Send CAB on port signal from external interface signal test board. | - It is checked that temporary change of internal variable in carline packet viewer. - Carline polling runs by main PAC. - Carline polling of #701, #802 train runs by master PAC. | 1) Temporary change of internal variable is checked in carline packet viewer. 2) Carline polling is checked by Main PAC of the car composition. 3) Carline polling of #701, #802 train runs by master PAC. | Pass |
|  | TC-PAC-INITIAL-01-07 | - Change LIC packet in packet simulation program and send #702 car number thru RS485. - Off the packet simulation. 3) Reboot PAC board. | 1) PAC stores #702 car number information received from LIC packet in the internal flash memory and RAM. 2) It is run by the car number stored in RAM. 3) It is run by loading the existing car number from the flash memory even if no car number information of LIC. | 1) Check if PAC stores #702 car number information received from LIC packet in the internal flash memory and RAM. 2) Check if it is run by the car number stored in RAM. 3) Check if it is run by loading the existing car number from the flash memory even if no car number information of LIC. | Pass |
|  | TC-PAC-INITIAL-01-08 | - PAC power ON. - Change LIC packet in packet simulation program and send #703car number thru RS485. | - It runs by loading car number information from internal flash memory. - Save the car number from LIC packet on PAC flash memory and RAM and runs #703. | 1) Check if car number information is loaded from internal flash memory. 2) Check if the car number received from LIC packet is stored in PAC flash memory and RAM and is run as #703. | Pass |
|  | TC-PAC-INITIAL-01-09 | - Power ON of CRA PAC and CRB PAC. - Power OFF CRA PAC. | - CAR PAC is polled to main PAC. - Detect if CRB PAC and CRA PAC are dead and are running by changing the role from sub PAC to main PAC. | 1) Check if CAR PAC is polled to main PAC. 2) Check if CRB detects CRA PAC is dead and also CRB is run as Main PAC in Sub PAC by changing a role. | Pass |
| Check the Status of all Devices within a specific Car | TC-PAC-ANALYSIS-01-01 | - Send CRA port signal from external interface signal test board. - Power on PAC board. | - Run as main PAC. - Polling continuously as many as device IDs. | 1) Check if it is run as Main PAC. 2) Check if it is polling continuously as many as number of device IDs. | Pass |
|  | TC-PAC-ANALYSIS-01-02 | - Make all other device packets with packet simulation program and send via RS485. - Cancel the packet response of some devices. | - If the device gives a feedback of response packet to polling packet of PAC, the device is displayed alive on the status information of communication rack. - If there were no response of packet received, the device that didn’t receive the packet would be displayed dead. | - Check if the device gives a feedback of response packet to polling packet of PAC, the device is displayed alive on the status information of communication rack. 2) Check if it is displayed that the device is dead when no response packet received. | Pass |
| Consist Inauguration | TC-PAC-ANALYSIS-02-01 | - Turn on CRA and CAB port signal of external unterface signal test board in single car #701. - Turn on CRB and CAB port signal of external interface signal test board in single car #701. | - It is checked with carline packet viewer that CAB is on at CRA on signal car #701 and train is connected with #701, #802 and #801. - It is checked with carline packet viewer that CAB is on at CRB in signal car #701 and train is connected with #802, #801 and #701. | 1) Check If the train consists of #701, #802, #801 with cab on single car #701 CRA in CarLine viewer 2) Check If the train consists of #802, #801, #701with cab on single car #701 CRB in CarLine viewer. | Pass |
|  | TC-PAC-ANALYSIS-02-02 | - Turn on CRA and CAB port signal of external interface signal test board in married car #802. - Turn in CRB and CAB port signal of external signal test board in married car #801. | - It is checked with carline packet viewer that CAB is on at CRA on married car #802 and train is connected with #802, #801 and #701. - It is checked with carline packet viewer that CAB is on at CRB on married car #801 and train in connected with #701, #801 and #802. | 1) Check If the train consists of #802, #801, #701with cab on married car #802 CRB in CarLine viewer. 2) Check If the train consists of #701, #801, #802 cab on married car #801 CRB in CarLine viewer. | Pass |
| Retrieve the Device Status tables from all PRIMARY PAC’s | TC-PAC-ANALYSIS-03-01 | - Turn on CRA and CAB port signal of external interface signal test board in signal car #701. - Turn on CRB and CAB port signal of external interface signal test board in signal car #701. | - Device status table is created because CAB is on at CRA on signal car #701 and double heading is complete. - Device status table has same configuration as CAB on at CRA even if CAB is on at CRB in signal car #701. | 1) Check if Device status table is renewed owing to CAB on at CRA on single car #701 and the completion of double heading. 2) Check if Device status table has the same configuration as CAB on at CRA even if CAB is on at CRB in singlel car #701. | Pass |
|  | TC-PAC-ANALYSIS-03-02 | - Create CCP packet that crew ID and train number are changed on packet simulation program of signal car #701. | - Check if the information of master PAC with CAB on is reviewed with the same train number and crew ID on married car #802. | 1) Check if the information of master PAC with CAB on is renewed with the same train number and crew ID on married car #802 | Pass |
|  | TC-PAC-ANALYSIS-03-03 | - #802 CRA, CRB PAC Power OFF. | - Check if the dead device information of married car #802 is renewed on master PAC #701. | 1) Check if the dead device information of married car #802 is renewed on master PAC #701. | Pass |
| PA Audio Train Line level balancing | TC-PAC-ANALYSIS-04-01 | - Turn on CRA and CAB port signal of external interface signal test board in signal car #701. | - During double heading, 11Khz tone occurred is viewed on train line packet viewer. | 1) Check that 11Khz tone occurred is viewed on train line packet viewer during double heading. | Pass |
|  | TC-PAC-ANALYSIS-04-02 | - Change the resistance of volume gain on external interface signal test board. - Load the program which volume gain can’t be adjusted with respect to voltage values. - Reboot PAC board. - Load the program which volume gain can be adjusted with respect to voltage values. - Change the resistance of volume gain on external interface signal test board. - Reboot PAC board. | - Load on audio line of train line. - Test the program which doesn’t balance the level before the test. - When double heading is complete, check that CNCS default volume and PAC Amp volume are the same. - Load the final version of program to test. - Load on audio line of train line. - After double heading and level balance applied, check that the PAC Amp volume is increased on carline packet viewer. | 1) Check if it Loads on audio line of train line. 2) Check if the program not balanced level is tested in advance for the test. 3) Check if CNCS default volume and PAC Amp volume are the same after the completion of double heading. 4) Check if the program is load in order to test the program completed. 5) Check if it loads audio line of train line. 6) Check if PAC Amp volume is increased on Carline viewer by applying the level balance after the completion of double heading. | Pass |
| Monitor Consist Integrity | TC-PAC-ANALYSIS-05-01 | - Turn on CRA and CAB port signal of external interface signal test board in signal car #701. - Turn off CAB port signal of external interface signal test board in signal car #701. | - It’s shown in carline packet viewer that CAB is on at CAR in signal car #701 and if train is connected with #701, #802 and #801. - The information of double headed train is set to initial value 10 sec after CAB signal is off which is viewed on carline packet viewer. | 1) Check If the train consists of #701, #802, #801with cab on single car #701 CRB in CarLine viewer. 2) Check if The information of double headed train is initialized in Carline viewer 10sec after CAB signal is off. | Pass |
|  | TC-PAC-ANALYSIS-05-02 | - Turn on CAR and CAB port signal of external interface signal test board in signal car #701. - Change coupler port signal of external interface signal test board on TOC carline #801. | - It’s shown in carline packet viewer that CAB is on at CRA on signal car #701 and if train is connected with #701, #802 and #801. - On double heading, if coupler signal of HOC or TOC is changed, then it’s shown in train line packet viewer that re-double heading begins. | 1) Check if the train consists of #701, #802, #801 with cab on single car #701 CRA in Carline viewer. 2) Check if re-double heading begins in the train line packet viewer when coupler signal of HOC or TOC is changed. | Pass |
| Monitor Intercom Status | TC-PAC-ANALYSIS-06-01 | - Turn in DPO live signal of external interface signal test board. - Turn off DPO live signal of external signal test board. | - It is checked on carline packet viewer that DPO live bit of device status tables is 0. - It is checked on carline packet viewer that DPO live bit of device status tables is 1. | 1) Check if DPO Live bit of Device Status Tables becomes '0' on Carline Packet viewer. 2) Check if DPO Live bit of Device Status Tables becomes '1' on Carline Packet viewer. | Pass |
|  | TC-PAC-ANALYSIS-06-02 | - Turn on CPO live signal of external interface signal test board. - Turn off CPO live signal of external interface signal test board. | - It is checked on carline packet viewer that CPO live bit of device status tables is 0. - It is checked on carline packet viewer that CPO live bit of device status tables is 1. | 1) Check if CPO Live bit of Device Status Tables becomes '0' on Carline Packet viewer. 2) Check if CPO Live bit of Device Status Tables becomes '1' on Carline Packet viewer. | Pass |
|  | TC-PAC-ANALYSIS-06-03 | - Turn on PEI live signal of external interface signal test board. - Turn off PEI live signal of external interface signal test board. | - It is checked on carline packet viewer that PEI live bit of device status tables is 0. - It is checked on carline packet viewer that PEI live bit of device status tables is 1. | 1) Check if PEI Live bit of Device Status Tables becomes '0' on Carline Packet viewer. 2) Check if PEI Live bit of Device Status Tables becomes '1' on Carline Packet viewer. | Pass |
| Process Local PEI Address Status Interrupt | TC-PAC-ADDRESS-01-01 | - Turn on PEI address signal of external interface signal test board. - Turn off PEI address signal of external signal test board. | - It is checked on carline packet viewer that PEI address bit of device status tables is 1. - It is checked on carline packet viewer that PEI address bit of device status tables is 0. | 1) Check if PEI Address bit of Device Status Tables becomes '1' on Carline Packet viewer. 2) Check if PEI Address bit of Device Status Tables becomes '0' on Carline Packet viewer. | Pass |
| Query Train Line PEI Address Status | TC-PAC-ADDRESS-02-01 | - Turn on CRA and CAB port signal of external interface signal test board. - Turn on PEI address signal of external interface signal test board. | - It’s shown in carline packet viewer that CAB is on at CAR on single car #701 and train is connected with #701, #802 and #801. - It’s shown in carline packet viewer that PEI address information called form signal car #701 is renewed on married car #802. | 1) Check if the train consists of #701, #802, #801 with cab on single car #701 CRA in Carline packet viewer. 2) Check if it is shown in carline packet viewer that PEI address information called form signal car #701 is renewed on married car #802. | Pass |
|  | TC-PAC-ADDRESS-02-02 | - Turn on PEI address signal of external interface signals test board at married car #802 or #801. | - It is checked in carline packet viewer that PEI address information called from married car #802 or #801 is renewed on master PAC #701. | 1) Check if it is checked in carline packet viewer that PEI address information called from married car #802 or #801 is renewed on master PAC #701. | Pass |
| Monitor & Process AVL Trigger Events | TC-PAC-INITAVL-01-01 | - Turn on CRA and CAB port signal of external interface signal test board in single car #701. - Create CCP packet which the trigger point is change on the packet simulation program of single car #701 and send it via RS485. | - It’s shown in carline packet viewer that CAB is on at CRA on single car #701 and train is connected with #701, #802 and #801. - Trigger point information recognized at master PAC in single car #701 is also renewed in married car #802. | 1) Check if it is shown in carline packet viewer that CAB is on at CRA on single car #701 and train is connected with #701, #802 and #801. 2) Check if trigger point information recognized at master PAC in single car #701 is also renewed in married car #802. | Pass |

VRX

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| VRX Signal Interface | TC-VRX-Initial-01 | 1. After provide the power, set the initial value and ID of device and open the RS-485 port. | 1. In case of normal operation, the LED of VRX turns on. | 1. VRX's LED for check was on. | Pass |
|  | TC-VRX-Comm-01 | - Push the CHECK button of CCP. | 1. In case of normal operation, the relevant FDI Unit indicates as green color, on the other hand, it indicates an red. | 1. The relevant PID Unit became green. | Pass |

VTX

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| VTX Signal Interface | TC-VTX-Initial-01 | 1. After provide the power, set the initial value and ID of device and open the RS-485 port. | 1. In case of normal operation, the front LED of VTX turns on. | 1. LEDC's LED for check was on. | Pass |
|  | TC-VTX-Comm-01 | - 1. Push the CHECK button of CCP. | 1. In case of normal operation, the relevant FDI Unit indicates as green color, on the other hand, it indicates an red. | 1. The relevant FID Unit was displayed to green. | Pass |

LEDC

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| LEDC Signal Interface | TC-FDI-Initial-01 | 1. After provide the power, set the initial value and ID of device and open the RS-485 port. | 1. In case of normal operation, the LED of LEDC turns on. | 1. LEDC's LED for check was on. | Pass |
|  | TC-FDI-Comm-01 | 1. Push the CHECK button of CCP. | 1. In case of normal operation, the relevant FDI Unit indicates as green color, on the other hand, it indicates an red. | 1. The relevant FID Unit was displayed to green. | Pass |
|  | TC-FDI-Display-01 | 1. After input CREW ID and train number using CCP GUI, push the simulation button. | 1. In case of normal operation, the destination which is input, is displayed on the LED module(LDM) of the relevant FDI Unit | 1. The last station input in LED module(LDM) of the relevant FDI Unit was displayed. | Pass |
|  | TC-FDI-Update-01 | 1. Copy new images then add them in Update folder in CNCS , execute Update command , Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The new image updated in LED module(LDM) of the relevant FDI Unit is displayed. | 1. The new station image updated in LED module(LDM) of the relevant FDI Unit was displayed. | Pass |
|  | TC-SDI-Initial-01 | 1. Set the device initial value and ID and open RS-485 communication port after power comes. | 1. LEDC's LED for check is on when normally operated. | 1. LEDC's LED for check was on.. | Pass |
|  | TC-SDI-Comm-01 | 1. Push the CHECK button of CCP. | 1. The relevant SDI Unit becomes green if normally operated. If not, it is displayed to red. | 1. The relevant SDI Unit was displayed to green. | Pass |
|  | TC-SDI-Display-01 | 1. Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The last station input in LED module(LDM) of the relevant SDU Unit is displayed when normally operated. | 1. The last station input in LED module(LDM) of the relevant SDU Unit was displayed. | Pass |
|  | TC-SDI-Update-01 | 1. Copy new images then add them in Update folder in CNCS , execute Update command , Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The new image updated in LED module(LDM) of the relevant SDI Unit is displayed. | 1. The new station image updated in LED module(LDM) of the relevant SDI Unit was displayed. | Pass |
|  | TC-PII-Initial-01 | 1. Set the device initial value and ID and open RS-485 communication port after power comes. | 1. LEDC's LED for check is on when normally operated. | 1. LEDC's LED for check was on. | Pass |
|  | TC-PII-Comm-01 | 1. Push the CHECK button of CCP. | 1. The relevant PII Unit becomes green if normally operated. If not, it is displayed to red. | 1. The relevant PII Unit was displayed to green. | Pass |
|  | TC-PII-Display-01 | 1. Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The text message is displayed on LED module(LDM) of the relevant PII Unit when normally operated. | 1. The text message was displayed on LED module(LDM) of the relevant PII Unit. | Pass |
|  | TC-PII-Update-01 | 1. Copy new images then add them in Update folder in CNCS , execute Update command , Input CREW ID and train number via CCP GUI and push the simulation button. | 1. The new text message updated is displayed on LED module(LDM) of the relevant PII Unit when normally operated. | 1. The new text message updated was displayed on LED module(LDM) of the relevant PII Unit | Pass |

TRIC

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| TRIC Signal Interface | TC-TRIC-Initial-01 | 1. After provide the power, set the initial value and ID of device and open the RS-485 port. | 1. In case of the normal operation, if the number 1 of the LED for TRIC turns out, it indicates ID 1, and if the number 2 of the LED for TRIC turns out, it indicates ID 2. | 1. Displayed ID1 by flashing TRIC's LEC for check once and ID2 by flashing twice. | Pass |
|  | TC-TRIC-Comm-01 | - 1. Push the CHECK button of CCP. | 1. In case of normal operation, the relevant TRIC Unit indicates as green color, on the other hand, it indicates an red. | 1. The relevant TRIC Unit was displayed to green. | Pass |
|  | TC-TRIC-FSK-01 | - Transfer a FSK header data (1) from the RRDCC simulator. - Transfer a FSK header data (2) from the RRDCC simulator. - Transfer a FSK header data (3) from the RRDCC simulator. | 1. f the FSK header data is (1), output the RTP signal as a HIGH level with the PAC, and output the audio which is received from a radio with the PAC. 2. If the FSK header data is (2), output the RTP signal as a LOW level with the PAC. 3. If the FSK header data is (3), output with the CAB text message printer and save up to 100 messages. | 1. RTP signal was output as PAC with HIGH Level and the audio received from the radio was output as PAC. 2. RTP signal was output as PAC with LOW level. 3. Output by CAB text message printer and stored up to 100 messages. | Pass |
|  | TC-TRIC-Control-01 | 1. Push the PPT button on the dashboard. | 1. Transfer a train number, car number and crew ID to the header data which transfers to the RRDCC. | 1. Transmitted a train number, car number and crew ID to the header data which transfers to the RRDCC | Pass |
|  | TC-TRIC-Message-01 | 1. Transfer a FSK header data (3) from the RRDCC simulator. 2. Touch the ‘Clear’ button in a relevant message of the GUI interface of the CCP. 3. Touch the ‘Repair’ button in a relevant message of the GUI interface of the CCP. | 1. After output with the CAB text message printer, ‘Printed’ will be displayed on the GUI interface of CCP. 2. After delete a relevant message in the TRIC memory and check whether the message is deleted or not, delete the message title in the GUI screen. 3. Output with the CAB text message printer by reading a relevant message in TRIC memory. | 1. ‘Printed’ was displayed on GUI interface of CCP after output with CAB text message printer. 2. Deleted the message on GUI screen after checking to delete or not to delete. 3. . It was output by CAB text message printer by reading the relevant message in TRIC memory. | Pass |

CNCS

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| CNCS Interface | TC-CNCS Launcher-01 | - Add a normal update file in the update folder. | - In case of normal performance, the file is updated. | - The file version has been modified. | Pass |
|  |  | - Add an abnormal update file in the update folder. | - In case of normal performance, the file is not updated. | - file version has not been modified. | Pass |
|  | TC-CNCS Main-Initial-01 | - Open the 485 communication port. | - In case of normal performance, CNCS will ordinarily respond to the requirement of PAC. | - CNCS is displayed as “OK” on the right side of CCP. | Pass |
|  |  | - Open the UDP port. | - In case of normal performance, Sub-Component should be monitored. | - Process Monitor State of the Main Screen shows the state of Sub-Component. | Pass |
|  | TC-CNCS Main-Monitoring-01 | 1. Close Sub-Component program. | 1. After 5 minutes, processor should ordinarily be operated. | 1. The processor operates ordinarily after 5 minutes. | Pass |
|  | TC-CNCS Main-RS485-01 | 1. Check CNCS state on the right side of screen. | 1. In case of normal performance, CNCS will ordinarily respond to the requirement of PAC. | 1. CNCS is displayed as “OK” on the right side of CCP. | Pass |
|  | TC-CNCS PP-Initial-01 | 1. Switch on CNCS and wait until the content plays by operating PP. | 1. advertisement video or a still image normally play on the LCD Display screen and confirm if the PP state is 2 when stopped. | 1. The contents was playing on the LCD Display screen and the state of Main.exe was 2. | Pass |
|  | TC-CNCS PP-ContentPlay-01 | 1. Add the driving information (car and crew numbers) in CCP and test it by using simulation functions. | 1. Observe If the passenger guide information is displayed by the schedule with car numbers. | 1. The passenger guide message was displayed and a video and a still image were playing After the time appointed. | Pass |
|  | TC-CNCS SP-Initial-01 | 1. Switch on CNCS and wait until UDP communicates by operating SP. | 1. Check if SP state of Main.exe is 2. | 1. SP state of Main.exe was normally operated as 2. | Pass |
|  | TC-CNCS SP-ContentPlay-01 | 1. Add the driving information (car and crew numbers) in CCP and test it by using simulation functions. | 1. Observe If the message and contents are displayed well. | 1. The passenger guide message was displayed. | Pass |
|  | TC-CNCS CDT Data-Initial-01 | 1. Open a communication port for LIC. | 1. LIC response should be confirmed on the CDT Data screen. | 1. The response is conformed on the CDT Data screen. | Pass |
|  | TC-CNCS CDT Data-RS232-01 | 1. Request CDT Date file from LIC. | 1. The CDT data file from LIC should be created. | 1. The CDT data file from LIC was created. | Pass |
|  | TC-CNCS CDT Data-RS232-02 | 1. Transmit CNCS time information to LIC due to request from LIC. | 1. The time information of CNCS should be transmitted to LIC. | 1. The time information should be confirmed on the CDT Data Screen. | Pass |
|  | TC-CNCS CDT Data-RS232-03 | 1. Transmit Communication System version information be LIC request. | 1. The version information of CNCS should be transmitted to LIC. | 1. The version information should be confirmed on the CDT Data Screen. | Pass |
|  | TC-CNCS FTP-Initial-01 | 1. Open UDT Socket. | 1. The normal state of FTP should be displayed on the Main Process. | 1. The normal state of FTP was displayed on the Main Process. | Pass |
|  | TC-CNCS FTP-Wayside-01 | 1. Perform Ping test with FTP Server. | 1. If FTP Server is normal, the Wayside Icon of CCP should be displayed with green color. | 1. When FTP Server is normal, the Wayside Icon of CCP was displayed with green color. | Pass |
|  | TC- REQ-CNCS FTP-Update-01 | 1. Add CDT data to be transmitted to FTP Server into FTP Upload folder. | 1. If the target file is transmitted normally, two file have the same contents. | 1. The target file and uploaded file into the FTP Server are same. | Pass |
|  | TC-CNCS FTP-Update-02 | 1. Add a data file to be transmitted to CNCS into FTP Server. | 1. The target file and downloaded file into the CNCS should be same. | 1. The target file and downloaded file into the CNCS are same. | Pass |
|  | TC-CNCS ContentUpdate-RS485-01 | 1. Using RS-485 communication, obtain a communication authority from PAC and update each relevant sub-component. | 1. Data should ordinarily be transmitted and received via Car Line. | 1. RS-485 transmitting and receiving states were displayed normally. | Pass |
|  | TC-CNCS ContentUpdate-Update-01 | 1. Copy a new data of CCP into the specified folder. | 1. New data of CCP should be recognized normally in the Content Update Process. | 1. New data of CCP was recognized normally in the Content Update Process. | Pass |
|  |  | 2. Copy a new MP3 data of PAC into the specified folder. | 2. New data of PAC should be recognized normally in the Content Update Process. | 1. New data of PAC was recognized normally in the Content Update Process. | Pass |
|  |  | 3. Copy a new data of LED into the specified folder. | 3. New data of LED should be recognized normally in the Content Update Process. | 3. New data of LED was r ecognized normally in the Content Update Process. | Pass |
|  | TC-CNCS ContentUpdate-Update-02 | 1. Update CCP contents. | 1. If simulation is performed on the CCP, the updated data should be shown. | 1. The updated data is shown normally. | Pass |
|  | TC-CNCS ContentUpdate-Update-03 | 1. Update PAC contents. | 1. If simulation is performed on the CCP, the updated data should be announced.. | 1. The updated data is announced normally. | Pass |
|  | TC-CNCS ContentUpdate-Update-04 | 1. Update LED contents. | 1. If simulation is performed on the CCP, the updated data should be shown. | 1. The updated data is normally shown on the LED screen. | Pass |

LIC

| Features to be Tested | Test Case ID | Test Step | Expected Output Specification | Real Output Specification | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- |
| LIC Interface | TC-LIC-Main-Comm-01 | 1. It can be checked boot process by using 232 monitoring program after connection RS-232 serial port. | 1. It indicates device name and program version during normal operations. | 1. The device operations status can confirmed by using console command. | Pass |
|  |  | 2. Abnormal update file are placed in designated update folder. | 2. The relevant update files are not updated during normal operations. | 2. It didn’t change relevant program’s version. | Pass |