| THE DRAWING(S) AND THE INFORMATION CONTAINED HEREIN ARE THE PROPERTY OF WOOJIN INDUSTRIAL SYSTEMS CO., LTD. THEY SHALL NOT BE DISCLOSED, REPRODUCED OR USED, IN ANY MANNER WITHOUT THE PRIOR WRITTEN CONSENT OF WOOJIN INDUSTRIAL SYSTEMS CO., LTD. |  | DENVER RTD EAGLE P3 Communication System Software Requirements Specification |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DISTRIBUTION |  | R E V I S I O N |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | Jan 20, 2010 |  | Issued |  |  | J.Y.LEE | D.S.JANG | B.J.JONE | E.K.LEE |
|  |  |  | No. | ISSUED DATE |  | REVISION NOTE |  |  | DESIGNED | CHECKED | APPROVED | REVIEWED |
|  |  |  |  |  |  |  |  |  | ENGINEERING |  |  | PM |
|  |  | PM |  |  | REVIEWED |  | E.K.LEE |  | CUSTOMER : |  |  |  |
|  |  | ENGINEERING |  |  | APPROVED |  | B.J.JONE |  | HYUNDAI-ROTEM COMPANY |  |  |  |
|  |  |  |  |  | CHECKED |  | D.S.JANG |  |  |  |  |  |
|  |  |  |  |  | DESIGNED |  | J.Y.LEE |  | PROJECT : |  |  |  |
|  |  | ISSUED DATE |  |  | October 20, 2011 |  |  |  | DENVER RTD COMMUNICATION SYSTEM |  |  |  |
|  |  | ISSUED BY |  |  | R & D CENTER |  |  |  |  |  |  |  |
|  |  | PROJECT NO. |  |  | P2011-020 |  |  |  |  |  |  |  |
|  |  | ORDER NO. |  |  | P110701149 |  |  |  |  |  |  |  |
|  |  | DOCUMENT NO. |  |  |  |  |  | REVISION |  |  |  |  |
|  |  | WJ-RTD-COMM-SRS-V1.0 |  |  |  |  |  | 1 |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |

REVISION RECORD

| Rev. No. | PAGE | Details |  | Date |
| --- | --- | --- | --- | --- |
|  |  | FROM | TO |  |
| 0 |  | First Issued |  | October 20, 2011 |

Table of Contents

1.0	Introduction	3

1.1	Purpose	3

1.2	Scope	3

1.3	Acronyms and Abbreviations	4

1.4	Reference Documents	4

1.5	Structure of Document	5

2.0	General Description	6

2.1	Product Perspective	6

2.2	Devices with S/W Functions	11

2.3	User Characteristics	15

2.4	Constraints	15

2.5	Assumptions and Dependencies	15

2.6	Apportioning of Requirements	16

3.0	Specific Requirements	16

3.1	External Interface	16

3.2	Functional Requirements	26

3.3	Performance Requirements	70

3.4	Logical Database Requirements	71

3.5	Design Constraints	71

3.6	Software System Attributes	72

- Introduction

Purpose

This Software Requirement Specification (SRS) describes both the functional and non-functional requirements necessary to fulfill the technical specifications of the DENVER RTD Eagle P3 Communication System (CS) as detailed in “DENVER RTD TS (Conformed Version, March 2010)”. This document shall be used by the project team members who will develop and verify the Communication System. If there’s no special mention, every requirement defined in this document has the priority.

Scope

The Communication System to be installed by WOOJIN within the DENVER RTD Eagle P3 Cars is a fully integrated Public Announcement (PA) and Transit Passenger Information System (TPIS). The system provides for the storage and management of operational train information, emergency information, and public announcements through a fully integrated audio visual communications system. WOOJIN shall supply, install, test and commission all communications equipment necessary to meet the requirements of DENVER RTD TS (Conformed Version, March 2010) Part C Rolling Stock Requirements.

Acronyms and Abbreviations – 알파벳 순으로 정렬

| Acronyms & Abbreviations | Description |
| --- | --- |
| AAB | Audio Amplifier Board |
| AMF | Automated Message File |
| AMF | Audio Message Format |
| ANSM | ANS Microphone |
| AVL | Automated Vehicle Location System |
| CCP | Communication Control Panel |
| CMR | Cab Make-Up Relay |
| CNCS | Content & Control Server |
| CRA | Communication Rack A |
| CS | Communication System |
| DMA | Driver Microphone Assembly |
| DPH | Driver PA Handset |
| DPO | Driver PA Operation Box |
| ESP | Exterior Speaker |
| FDI | Front Destination Indicator |
| GPS | Global Positioning System |
| ISP | Interior Speaker |
| LIC | LonWorks Interface Card |
| MDS | Monitoring and Diagnostic System |
| PA | Public Announcement |
| PAC | PA Control Board |
| PAS | Power Supply Assembly |
| PEI | Passenger Emergency Intercom |
| PID | Passenger Information Display |
| PII | LED Passenger Information Indicator |
| PIS | Passenger Information System |
| PTU | Portable Test Unit |
| SDI | Side Destination Indicator |
| TRF | Transformer |
| UDP | User Datagram Protocol |
| VRX | Video RX Board |
| VTX | Video TX Board |
| WLAN | Wireless LAN |
| WLANS | Wireless LAN System |
| WLR | Wireless LAN Router |

Reference Documents

| Publisher | Document Number | Title |
| --- | --- | --- |
| DENVER | 18FH012 | DENVER RTD TS (Conformed Version, March 2010) Part C Rolling Stock Requirements |
| Rotem | REDU106113 | Procurement Technical Specification For Denver RTD EMU Communication System |
| IEEE | 830-1998 | ANSI/IEEE Standard 730, 828, 830, 1012A, 1016, 1228 |

Structure of Document

The remainder of this SRS is divided into two main sections:

Section 2:    General Description

This section describes the hardware and software context of this particular communication, as well as its intended operations, functions and constraints. This section does not specify the requirements for the communication system, but provides the background information necessary to interpret the requirements.

Section 3:   Specific Requirements

This section provides details of all of the specific requirements for the CS.  The document is organized in a manner consistent with and in a corresponding order to the Master Software component list as described in Section 7 of the WOOJIN SFD document 423-309.  Each software component is addressed separately regarding its external interfaces and functional requirements.

The following categories:

- External Interfaces

- Functional Requirements

- Performance Requirements

- Database Requirements

- Design Constraints

- Software System Attributes

- General Description

Product Perspective

• The WOOJIN CS provides for all communication functionality within the newly fabricated DENVER RTD Eagle P3 cars. The system manages all aspects of audio, video, and data communications pertaining to passenger information and operational information relative to the vehicles and their interaction with DENVER RTD Wayside & Control Network.  The context diagram shown in section 2.1.1 below, (System Interfaces), illustrates the various interface points between the CS and external entities.

• All CS application S/W components running on the onboard CNCS are operating under the Windows XP embedded OS platform.  Device specific software components are written as low level code utilizing the inherent instruction set of the specific Micro processor controlling the device.  There are also a number of COTS software components utilized within the Wayside and On-Board systems.

• The WOOJIN CS software components utilized do not require any ongoing updating, unless there are changes made to the H/W configuration.  In the event that additional functional requirements are specified in the future, they may result in the need for S/W modifications to specific components.

System Interfaces

Figure 1. Communication System Context Diagram

From a software overview perspective, the CS as a functional entity interacts with various internal and external entities as illustrated in Figure 1 above.  To accomplish this interaction, the CS relies on a range of user, hardware, software, and communications interfaces to provide for the required DENVER RTD specified functionality. The following subsections describe the various interface categories, the specific entities to be communicated with, the nature of the communication, and the physical devices involved.

User Interfaces

There are four user types that require human interfacing to the communication system.  Each of these entities can function as either the source or destination of information processed and managed by the system.

| External Entity | Description |
| --- | --- |
| Driver | CAB Speakers, Driver Mic., CCP Touch Screen, DPO Box |
| Conductor | PA Speakers, |
| Passenger | LED, LCD, Internal & External Speakers, PEI Box, |
| Maintenance | CCP, PTU |

Hardware Interfaces

| External Entity | Description |
| --- | --- |
| MDS | The MDS interfaces with LIC using LonWorks Interface protocol. |
| MASCON Key Switch | The CCP and PAC interface with the MASCON Key via a hard wired signal line utilizing a DCV input. |
| Access door Equipment | The PAC interfaces with access door equipment via a hard wired signal line utilizing a DCV input. |
| PTU | The CNCS interfaces with the PTU via the build in USB / RS232 port located on the front of the CS Rack |
| Other Cars | The PAC’s of each individual Car interfaces with the other cars of the consist via the use of RS-485 over coupled train lines. |

Software Interfaces

| External Entity | Description |
| --- | --- |
| MDS Data Collection Node | The onboard CNCS communicates with the several MDS Data Collection Nodes located at the Yards and Union Station. This software interface accommodates the automated process of downloading MDS data from the Onboard communication system to the Wayside MDS data collection nodes. |
| Data Upload | This interface represents an automated process by which data is uploaded to the Onboard systems from the Wayside. This interface consists of a predefined set of processes which are governed by a set of operational parameters. This interface is used to provide for maintaining current versions of software, schedules, and content. |

Memory Constraints

Reflect the design not to use the usable memory more than 80% when designing CNCS S/W.

| Sub System | Memory Type | Size | Description | Use | Usable Memory |
| --- | --- | --- | --- | --- | --- |
| CCP | Flash | 2Gbyte | NAND Flash RAM incorporated into the microcontroller | Execute Code | 970Mbyte |
|  | Ram Usable memory | 256Mbyte | RAM incorporated into the microcontroller | Variable, Stack, Working | 59Mbyte |
| PAC | On-Chip Flash | 256Kbyte | Flash ROM incorporated into the microcontroller | Execute Code | 156Kbyte |
|  | On-Chip RAM | 64Kbyte | RAM incorporated into the microcontroller | Variable, Stack, Working | 32Kbyte |
|  | Off-Chip RAM | 1Mbyte | Flash RAM accessed through the Microcontroller Bus | Configuration data | 90Kbyte |
| CNCS | External RAM | 2Gbyte | RAM accessed through the Microprocessor Bus (DDR2 SDRAM) | OS, Execute Code, Working | 1Gbyte |
| LIC | On-Chip Flash | 256Kbyte | Flash ROM incorporated into the microcontroller | Execute Code | 200Kbyte |
|  | On-Chip RAM | 64Kbyte | RAM incorporated into the microcontroller | Variable, Stack, Working | 32Kbyte |
| FDI, SDI PII | On-Chip Flash | 64Kbyte | Flash ROM incorporated into the microcontroller | Execute Code | 32Kbyte |
|  | On-Chip RAM | 16Kbyte | RAM incorporated into the microcontroller | Variable, Stack, Working | 3Kbyte |

Operational Assumptions

- The Onboard Communication System must be designed to remain fully functioning at all times during normal operations of the vehicle.

- Regardless of each software component’s specific function, all components must operate in a synergistic manner consistent with reasonable expectations for a completely integrated communications system.

- All software components (as applicable) must be designed in a manner which supports the collection of system fault data and its subsequent logging into the MDS data terminal.

- PA functionality must include the ability to automatically mute the exterior speaker(s) audio level for specified stations at predetermined times.

- The Train Radio sub-system will be replaced with the VOIP function. Manual PA announcement functionality must remain active in the event of power loss for a period of 90 minutes.

- The priority order for functional use of the Communication System should follow the constraints of section 2.5.

Site Adaptation Requirements

None.

Devices with S/W Functions

The on-board Communication System utilizes a combination of application software, device firmware, and logic circuitry to provide for the required functionality.  The following paragraphs describe each CS device that utilizes application or firmware software, as well as a general description of the type of functionality each sub-component provides. A complete detailed functional description of each software component is included in section 3.1.

PAC

The CS includes two distinct PA Controllers which are housed in both CRA. These devices provide for the physical aspects of managing and distributing both Audio and Control Data within a specific vehicle, as well as amongst the various vehicles making up a Train Consist. The PAC’s process and distribute all audio messaging in accordance with the parameters specified within the AVL messaging system, the Intercom system, and the PA system. PAC Control software provides the following functions;

The PAC manages all audio messages such as AVL, IC (Intercom), PEI, PA Announcement  and also distributes them to the appropriate devices.

The Master PAC stores the pre-recorded MP3 audio contents for Automatic PA announcements.

The individual PAC’s monitor the status of all major Communication System devices and aggregates the status data at the Master PAC.

The Master PAC has a direct connection to the Master CCP and provides much of the information that is used by the CCP to display information to the crew.

The PAC’s manage all RS-485 communication both within the Cars and across the Consist.

This includes performing the Consist Inauguration process.

DPO

DPO is an intercom communication device which driver is able to use. It receives the input signal with tone signal and transmits output signal with tone. DPO is able to respond to the call by OCC, PEI communication request. 3-way communication with the OCC is available during the communication if required. If the DPO cannot respond to the PEI call, OCC will respond instead of the DPO.

DPO consists of 7 buttons.

- Make an announcement by selecting the range of the manual PA announcement

- Communication between OCC and DPO, Multi-communication between DPO and other DPOs.

- Communication between DPO and PEI

- Initialize the call between intercoms and the call between PEIs.

- Communication between DPO and OCC, Make a call when 3-way communication with the OCC is required during communication between DPO or PEI.

DPH

DPH is a device which driver is able to make manual announcement. It has employed the manual announcement function separately from the DPO.

DPH consists of 3 buttons.

- Make an announcement by selecting the range of the manual PA announcement

PEI

PEI is a device which passenger is able to communicate with the driver or crew in an emergency situation. The location of the pressed PEI will be announced through the driver’s speaker when passenger presses the CALL button, and communication will be available by composing communication path when PEI button has been pressed on the DPO.

CCP

The Communication Control Panel is the primary interface between the onboard CS and the Driver.  This device is mounted in the Cab and is easily accessible to the Driver.  It is equipped with a touch screen and GUI user interface which provides the crew with the ability to initialize the Consist, set & edit operating parameters, test functionality, review real time schedule adherence data, and monitor the CS operational status.  The CCP performs the process of calculating the Train’s current position as compared to the defined route schedule which support both the Schedule Adherence and AVL automated message triggering functions.

CNCS

The Communication and Control Server represents the heart of the onboard communication system. It is housed in CRA and is equipped with an Intel dual core processor, 2GB Ram, a 32GB solid state drive and operates under the Microsoft Windows XP Embedded OS. The CNCS provides for the storage and execution of many of the onboard CS software components, as well as for the storage and distribution of most of the visual content. The following lists the various software elements which are sub-components of the CNCS and the functionality provided by each;

- Launcher S/W Component (Launcher.exe): This application is automatically executed at the                conclusion of the boot process.  The primary functions of this software are;

Performs version control comparisons of newly downloaded program updates and sets update flags as appropriate.

Performs the application updates as set by the update flags.

This process is only executed once at power up, and then loads the “Main.exe”  application upon completion.

- Main Control S/W Component (Main.exe):  This is the Content & Control Server’s (CNCS) primary software component and it resides on the server’s internal hard drive.  After the CNCS has been initialized and the “Launcher.exe” application has completed all component update functions, then the CNCS automatically loads “Main.exe”. Main.exe provides for management, storage and communications between and amongst the subsystems, as well as provides for the delivery of content parameters as defined in the scheduler. This application also spawns various other memory resident service applications and provides for communication between the various services utilizing UDP. The majority of all devices within the communication system are in some way related to, driven by, managed by, or dependent upon functionality provided for by this application.

- Primary Media Player S/W (PP.exe):  The Primary Media Player controls the main (upper) video playback zone of the LCD display devices. It receives content scheduling information provided  by the CNCS’s Main Control software application and facilitates the playback of specified  content within the defined zone of the specified output devices.

- Secondary Media Player S/W (SP.exe):  The Secondary Media Player controls the lower (scroll) video playback zone of the LCD display devices.  It receives content scheduling information provided by the CNCS’s Main Control software application and facilitates the playback of specified content within the defined zone of the specified output devices.

- FTP Data Update S/W (FTPData.exe):  This software component is used to provide for both the uploading and downloading of content and data files between the on-board system and  the Wayside network. This software utilizes FTP protocol to manage the transfer of the files over the Wayside’s Wireless LAN. The software will employ the use of a process script to establish the priority order and manner by which it determines the specific files to be transferred, the naming conventions to be utilized, when they are to be transferred, and where they are to be placed. This software will continually monitor the availability of the Wayside LAN, will automatically connect to the LAN and determine the location of access, and will then perform any currently required file transfers.

- MDS Data Update S/W (MDSData.exe):  This software component provides functionality related to a) transferring MDS data from the MDS to the CNCS utilizing the LonWorks interface, and

b) transferring CS software version information and GPS time.

- Content Update S/W (ContentUpdate.exe):  This software component provides functionality related to the automatic version control, updating and management of content and schedule files within the on-board communication system (AMF, MP3, Schedule, Play List, etc.).  It manages the distribution of these content components from the CNCS to the internal memory modules of the various intended devices.  This software utilizes the RS-485 Car Lines to distribute LED content to the displays, and to distribute the schedule file components to the CCP.  It utilizes USB to distribute the MP3 audio files to the PAC.

VTX

VTX Board receives video signal from the CNCS and transmits it to the VRX Board. CNCS transmits video signal to the VTX through LVDS interface. VTX receives LVDS signal and outputs to the VRX by transferring to the SDI(Serial Digital Interface) signal which remote transmission is available.

VRX

VRX Board displays video of PID1 and PID2 devices by receiving video signal from the VTX. VTX Board transmits SDI(Serial Digital Interface) video signal which remote transmission is available to the VRX, and the VRX drives LCD of PID1 and PID2 by transferring SDI signal to the signal which is able to drive the LCD panel. Then it plays (3R , Regeneration/Retiming/Reamplication) SDI signal with PID1 and PID2.

FDI

The Front Destination Indicator (FDI) Control S/W component provides for the inherent functionality of the LED display device, as well as the functionality enabling the device to display Route# & Destination information. The S/W also facilitates RS-485 communication with the PAC controller to provide for content updating, content management, and system diagnostics.

It provides the control functionality for the actual LED Display.

It manages the receipt and execution of messaging parameters, content, and control codes.

It manages all RS-485 communication between the FDI’s and the PAC’s

It provides operational status information to the PAC via the RS-485.

SDI

The Side Destination Indicator (SDI) Control S/W component provides for the inherent functionality of the LED display device, as well as the functionality enabling the device to display Route# & Destination information. The S/W also facilitates RS-485 communication with the PAC controller to provide for content updating, content management, and system diagnostics.

It provides the control functionality for the actual LED Display.

It manages the receipt and execution of messaging parameters, content, and control codes.

It manages all RS-485 communication between the SDI’s and the PAC’s

It provides operational status information to the PAC via the RS-485.

PII

The Passenger Information Indicator (PII) Control S/W component provides functionality consistent with the FDI and SDI software but also monitors the AVL messaging system commands to display the automated information notifications.

It provides the control functionality for the actual LED Display.

It manages the receipt and execution of messaging parameters, content, and control codes.

It manages all RS-485 communication between the PII’s and the PAC’s

It provides operational status information to the PAC via the RS-485.

It monitors the AVL triggers and control signals to facilitate automated messaging on the PII’s.

LIC

The LonWorks Interface Card (LIC) provides for the interface between the MDS and the CS. The LIC is inserted into the backplane of CRA, utilizes RS-232 to communicate with the CNCS, LonWorks to communicate with the MDS, and RS-485 to communicate with the PAC. The card has two distinct processor chips, which each having firmware elements as described below.

User Characteristics

| User | Description |
| --- | --- |
| Crew | - Communicates with OCC through Driver Microphone and CAB Speaker. - Broadcasts public announcements of guidance to passengers using Microphone. - Operates and manages Communication System using Touch Screen. |
| Passenger | - Receives route guidance information and public announcements through LED and LCD. - Receives manual announcements of guidance from crew and automatic announcements of guidance from Communication System through Internal & External Speakers - Receives direct manual announcements of guidance from OCC as occasion demands |

Constraints

- All application S/W operating on CNCS and CCP shall be created with Visual C++ 2008.

- Neuron C Language and Node Builder v3.1 compiler shall be used as the programming language for Lon Works Communication.

- IAR EWARM 4.42a, IARAVR 5.2b, Keil uVision2 and Keil uVision3 shall be used as the programming tool for microprocessors.

Assumptions and Dependencies

Public address Priority Order:  The Public Address system utilizes the following priority order:

- Manual PA announcement OCC

- Manual PA announcement driver.

- Intercom communication Pre recorded Emergency PA announcement

- Pre recorded Automated PA announcement

All communication devices shall be available when the CAB-ON state is indicated. The Communication System should be fully functional during normal train operations.

Any manual announcement shall be train lined to all DENVER RTD cars within the Consist, and be routed to the external and/or internal speakers as appropriate.  The Crew will utilize the CCP located in the Master Cab to configure and monitor the CS.

Apportioning of Requirements

None.

- Specific Requirements

External Interface

- CAB ON

| Signal Name | CAB ON |
| --- | --- |
| Type | Digital Input |
| Description | Each PAC utilizes the adjacent Cab’s CAB ON signal to determine if it should be set as the Master PAC for the entire Consist. The CAB ON signal will be derived from the Cab Make-Up Relay (CMR) by monitoring the output voltage of the CAB ON signal line. If the CAB ON signal is present, then the specified PAC will be designated as the Master PAC. As such it will manage and control the RS485 Train Line, perform the Train Consist configuration process, and manage all audio functionality related to the audio Train Lines. |

- LEFT DOOR OPEN (LDO)

| Signal Name | LEFT DOOR OPEN (LDO) |
| --- | --- |
| Type | Digital Input |
| Description | The PAC utilizes this signal to determine if the left side door is in the Open or Closed position. The Door Control Unit (DCU) is equipped with a DCV output signal to indicate if the door is open or closed. The signal is activated just prior to the door beginning to open, and is terminated just after the door completes closing. |

- RIGHT DOOR OPEN (RDO)

| Signal Name | RIGHT DOOR OPEN (RDO) |
| --- | --- |
| Type | Digital Input |
| Description | The PAC utilizes this signal to determine if the right side door is in the Open or Closed position. The Door Control Unit (DCU) is equipped with a DCV output signal to indicate if the door is open or closed. The signal is activated just prior to the door beginning to open, and is terminated just after the door completes closing. |

- FORWARD

| Signal Name | FORWARD |
| --- | --- |
| Type | Digital Input |
| Description | The Master PAC receives the FORWARD signal from the Traction Control System. The FORWARD signal is provided by the Traction Control System whenever the MASCON Key is turned to the “Forward” position. |

- REVERSE

| Signal Name | REVERSE |
| --- | --- |
| Type | Digital Input |
| Description | The Master PAC receives the REVERSE signal from the Traction Control System. The REVERSE signal is provided by the Traction Control System whenever the MASCON Key is turned to the “Reverse” position. |

- RS-485 CAR LINE

| Signal Name | RS-485 CAR LINE |
| --- | --- |
| Type | Digital Input / Output |
| Description | The RS-485 Car Line data Input & Output signal is used to provide for digital communication between the PAC’s and the various devices within a Car. The RS-485 Car Line provides for data communication between the Primary PAC, CCP, Display Signs, CNCS, LIC, and the secondary PAC in the same car. Within any given car the Primary PAC maintains protocol control of the RS-485 Car Line network. All other RS-485 devices within the Car will typically be polled within a continuous loop by the Primary PAC. In most cases the primary PAC will issue requests for specific responses from each device, and the addressed device will provide the requested response. In certain cases it will be necessary for the Primary PAC to temporarily pass RS-485 control to another device so that it may affect the desired data communication. |

- RS-485 TRAIN LINE

| Signal Name | RS-485 TRAIN LINE |
| --- | --- |
| Type | Digital Input / Output |
| Description | The RS-485 Train Line data I/O signal is used to provide for digital communication between the Master PAC and all slave PAC’s throughout a Consist. The Master PAC maintains protocol control of the RS-485 Train Line network, and polls all other PAC’s within the Consist utilizing are polled via a continuous loop by the Master PAC. The Master PAC either a) issues information telegrams addressed to specific PAC’s, or b) issues requests for specific responses from slave PAC’s. The slave PAC’s will either receive and store the data, or respond to a request by providing the requested information. In certain cases the Master PAC may temporarily pass RS-485 control to a slave PAC to perform a specific function, which will then return control after the function is complete. |

3.1.1.1-A: RS-485 Polling Table

| RS- 485 Polling Sequences & Times |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Train Line Poll Table |  |  |  | Device Status of Train Line |  |  |  |  |  |
| Equipment | Remarks | Access Tim e |  | Equip. |  | Status Inf o. |  | Interface | Remarks |
| PAC(Primary) | Cab On Car | 0 |  | PAC |  | DPO/ PEI/CCP/ETC. . |  | Train Line RS- 485 | PAC Status of All Train |
| PAC(Primary)# 1 | Other Car | 200 ms |  |  |  |  |  |  |  |
| PAC(Primary)# 2 | Other Car | 200 ms |  |  |  |  |  |  |  |
| PAC(Primary)# 3 | Other Car | 200 ms |  | Device Status of Car Line |  |  |  |  |  |
| PAC(Primary)# 4 | Other Car | 200 ms |  | Equip. |  |  | Status Inf o. | Interface | Remarks |
| PAC(Primary)# 5 | Other Car | 200 ms |  | CCP |  |  | Comm State/ Device state | Car Line RS- 485 | Primary & Secondary |
| Car Line Poll Table |  |  |  |  |  |  |  |  |  |
| Equipment | Remarks | Access Tim e |  | CR | LIC |  |  |  | Primary & Secondary |
| CCP | CCP1~ 2 | 500m sec(Slave is about 14sec) |  |  | PAC |  |  |  | Primary & Secondary |
| LIC | LIC1~2 | 2sec |  |  | VTX |  |  |  | Primary & Secondary |
| CNCS | CNCS1~2 | 2sec |  |  | CNCS |  |  |  | Primary & Secondary |
| VOIP | VOIP1~2 |  |  |  | VOIP |  |  |  | Primary & Secondary |
| REC | REC1~2 |  |  |  | REC |  |  |  | Primary & Secondary |
| PAC | PAC(Secondary) | Slave is about 14sec |  | PID |  |  |  |  |  |
| VTX | VTX | about 14sec |  | FDI |  |  |  |  |  |
| PID | PID1~ PID12 | about 14sec |  | SDI |  |  |  |  |  |
| FDI | FDI1~ FDI2 | about 14sec |  | PII |  |  |  |  |  |
| PII | PII1~ PII4 | about 14sec |  | DPO |  |  | Fault | Digital Output | * When Transmit to Mast PAC * Train Line 485 |
| SDI | SDI1~ SDI8 | about 14sec |  | PEI |  |  |  |  |  |

3.1.1.1-B: RS-485 Address Table

| Denver Comm Internal 485 ID Table Item |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CRA |  |  |  |  |  |  |  |
|  | LIC1 | VOIP1 |  | VTX1 | CNCS1 | PAC1 | REC1 |  |
|  | 0x11 | 0x12 |  | 0x14 | 0x15 | 0x16 | 0x17 |  |
| 2 | CCP1 | CCP2 |  |  |  |  |  |  |
|  | 0x31 | 0x32 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 | PII1 | PII2 | PII3 | PII4 |  |  |  |  |
|  | 0x51 | 0x52 | 0x53 | 0x54 |  |  |  |  |
| 5 | FDI1 | FDI2 |  |  |  |  |  |  |
|  | 0x61 | 0x62 |  |  |  |  |  |  |
| 6 | SDI1 | SDI2 | SDI3 | SDI4 | SDI5 | SDI6 | SDI7 | SDI8 |
|  | 0x71 | 0x72 | 0x73 | 0x74 | 0x75 | 0x76 | 0x77 | 0x78 |
| 7 | PID1-1 | PID1-2 | PID1-3 | PID1-4 | PID1-5 | PID1-6 | PID1-7 | PID1-8 |
|  | 0x81 | 0x82 | 0x83 | 0x84 | 0x85 | 0x86 | 0x87 | 0x88 |
| 8 | PID2-1 | PID2-2 | PID2-3 | PID2-4 |  |  |  |  |
|  | 0x91 | 0x92 | 0x93 | 0x94 |  |  |  |  |

| Denver Comm Train line 485 ID Table |  |  |  |
| --- | --- | --- | --- |
| Number | Train Number | Item | Train line 485 ID |
| 1 | Car1 | PAC1(MA) | 0x1A |
|  |  | PAC2(MB) | 0x1B |
| 2 | Car2 | PAC1(MA) | 0x2A |
|  |  | PAC2(MB) | 0x2B |
| 3 | Car3 | PAC1(MA) | 0x3A |
|  |  | PAC2(MB) | 0x3B |
| 4 | Car4 | PAC1(MA) | 0x4A |
|  |  | PAC2(MB) | 0x4B |
| 5 | Car5 | PAC1(MA) | 0x5A |
|  |  | PAC2(MB) | 0x5B |
| 6 | Car6 | PAC1(MA) | 0x6A |
|  |  | PAC2(MB) | 0x6B |
| 7 | Car7 | PAC1(MA) | 0x7A |
|  |  | PAC2(MB) | 0x7B |
| 8 | Car8 | PAC1(MA) | 0x8A |
|  |  | PAC2(MB) | 0x8B |
| 9 | Car9 | PAC1(MA) | 0x9A |
|  |  | PAC2(MB) | 0x9B |
| 10 | Car10 | PAC1(MA) | 0xAA |
|  |  | PAC2(MB) | 0xAB |
| 11 | Car11 | PAC1(MA) | 0xBA |
|  |  | PAC2(MB) | 0xBB |
| 12 | Car12 | PAC1(MA) | 0xCA |
|  |  | PAC2(MB) | 0xCB |

- COUPLER

| Signal Name | RS-485 TRAIN LINE |
| --- | --- |
| Type | Digital Input |
| Description | Each PAC utilizes this signal to determine if the specific coupler which is directly adjacent to itself is in either the closed (coupled) or open (uncoupled) state. The COUPLER signal is a DCV hardwired signal which is generated by the Train Control system when a coupler device is in an open (uncoupled) state. During the Consist inauguration process, each PAC will evaluate the state of this signal to determine if another Car (2 PAC’s) is attached to the coupler, or if the Car in which the PAC is located is the last Car of the Consist. Each PAC must evaluate the state of the attached coupler at a minimum rate of once every 500ms. The stored parameter value will be updated to reflect the current state when the physical Coupler is switched from the Open to Closed position, and vice versa. |

- IC LIVE Lines

| Signal Name | IC LIVE Lines |
| --- | --- |
| Type | Digital Input |
| Description | Each PAC has 12 hardwired IC LIVE signal lines which are directly connected to two DPO’s, two DPH’s, four PEI control boxes. Each of these IC Devices has a distinct IC LIVE line to indicate if the device is powered on or not. If a DCV signal is present, then the device is powered on and is ready for operation. This signal does not indicate that a PEI Call is in progress, but merely indicates that the device is powered on and is operational. |

- DPO ADDRESS Lines

| Signal Name | DPO ADDRESS Lines |
| --- | --- |
| Type | Digital Input |
| Description | Each PAC receives input from a discrete DPO ADDRESS signal line which is hardwired to the PAC from the DPO box located in the adjacent Cab. This ADDRESS line is used to determine if the DPO device has been activated by the driver. |

- DPH ADDRESS Lines

| Signal Name | DPH ADDRESS Lines |
| --- | --- |
| Type | Digital Input |
| Description | Each PAC receives input from a discrete DPH ADDRESS signal line which is hardwired to the PAC from the DPH box located in the adjacent Cab. This ADDRESS line is used to determine if the DPH device has been activated by the driver. |

- PEI ADDRESS Lines

| Signal Name | DPH ADDRESS Lines |
| --- | --- |
| Type | Digital Input |
| Description | Each PAC receives input from a discrete PEI ADDRESS signal line which is hardwired to the PAC from each of the three PEI boxes located within each Car. These ADDRESS lines are used to determine if the PEI devices have been activated by a Passenger pressing the PEI CALL button. |

- ANSM

| Signal Name | ANSM |
| --- | --- |
| Type | Analog Input |
| Description | This interface is included to provide for the ability to measure the noise level within a car and automatically adjust the volume level as appropriate to maintain a consistent level of audio intelligibility and quality within a Car. To measure the car’s ambient noise the PA sub-system is equipped with two automatic noise sensing (ANS) microphones, one connected to each PAC. The ANS Microphone monitors the ambient noise level within the car and provides an analog DCV output to the input of the PAC. The PAC will then sample the analog input at approximately 2 second intervals and calculate an ongoing weighted average of the most recent samplings. The sampling will be suspended during any PA announcements to alleviate errors in sampling. This calculated DC voltage value will then be compared to pre-set “MINIMUM” and “MAXIMUM” ANS Gain parameters, (The ANS Gain parameters are settable via the PTU). Based upon the results of this comparison, the appropriate ANS Gain level will be selected. This value will be processed through an analog to digital converter and the resulting value will be stored as a parameter in the PAC parameter table. *** Reference: At the time when an AVL or PA audio announcement is initiated, a separate software function of the PAC will utilize the current ANSM parameter value to adjust the audio output level of the volume control circuitry utilizing the I2C interface. To guard against Noise created by the switching of mechanical relays, the PAC will mute the amplifiers during the switching of the volume control relays.. |

- VOLUME CONTROL Signal

| Signal Name | VOLUME CONTROL Signal |
| --- | --- |
| Type | Serial Output I2C |
| Description | The PAC volume control mechanism consists of both hardware and software components to manage different aspects of the functionality. With reference to the software functionality, the PAC generates a VOLUME CONTROL output signal which is used to deliver parameters and control codes to the Interior, Left Exterior, and Right Exterior volume control units. This function monitors all parameters set by the various Communication System devices, assembles a data packet reflecting the appropriate control codes necessary to invoke the desired functionality, and sends the data over the I2C serial interface to the Volume Control Units. |

- PAC_ID ADDRESS

| Signal Name | PAC_ID ADDRESS |
| --- | --- |
| Type | Digital Input |
| Description | This identifies each PAC as being located in either CRA. The pin is hardwired in the chassis to be “0” at PAC-MA and “1” at PAC-MB. |

- USB_MP3

| Signal Name | USB_MP3 |
| --- | --- |
| Type | Serial Data Input and Output |
| Description | This is a USB port that connects the PAC’s on-board MP3 Player memory directly to the CNCS. It is used to transfer Audio Content files from the CNCS HDD to the local PAC’s MP3 Audio Player memory module. |

DPO

- Internal

| Signal Name | Internal |
| --- | --- |
| Type | Digital Input |
| Description | Generates alarm tone which informs manual announcement is in progress via interior speaker when signal which informs internal button has been pressed enters. The interior manual announcement is only available when the button has been pressed. |

- External

| Signal Name | External |
| --- | --- |
| Type | Digital Input |
| Description | Generates alarm tone which informs manual announcement is in progress via exterior speaker when signal which informs external button has been pressed enters. The exterior manual announcement is only available when the button has been pressed. |

- BOTH

| Signal Name | Both |
| --- | --- |
| Type | Digital Input |
| Description | Generates alarm tone which informs manual announcement is in progress via interior and exterior speaker when signal which informs both buttons has been pressed enters. The interior and exterior manual announcement is only available when the button has been pressed. |

- IC

| Signal Name | IC |
| --- | --- |
| Type | Digital Input |
| Description | LED of the IC button is usually turned off. If intercom communication is required and the button has been pressed, the LED of the IC button starts to blink, and recognizes that driver or crew requires intercom communication. Intercom communication will be made when pressing the blinking IC button. All requests and communication status will be cancel when cancel button has been pressed. |

- PEI

| Signal Name | PEI |
| --- | --- |
| Type | Digital Input |
| Description | LED of the PEI button is usually turned off. If intercom communication is required and the button has been pressed, the PEI LED of the DPO starts to blink, and recognizes that driver or crew requires intercom communication. Intercom communication will be made when pressing the blinking PEI button. All requests and communication status will be cancel when cancel button has been pressed. |

- CANCEL

| Signal Name | Cancel |
| --- | --- |
| Type | Digital Input |
| Description | Communication request with intercoms, communication status between intercoms, communication request of PEI, communication status with PEI, communication request with the OCC, and communication status with the OCC will be cancel when cancel button has been pressed. |

- OCC

| Signal Name | OCC |
| --- | --- |
| Type | Digital Input |
| Description | LED of the OCC button is usually turned off. Requesting communication with the OCC is available by pressing the OCC button when DPO wants to communicate with the OCC or communication with passenger through PEI. LED of the OCC button blinks at this time. LED of the OCC button will be turned on if the communication with the OCC has been made. LED of the OCC button will blink when the OCC requests communication to the DPO. Every call including communication request with the OCC, communication status will be canceled when the cancel button has been pressed, and the LED of the OCC button will be turned off. |

DPH

- Internal

| Signal Name | Internal |
| --- | --- |
| Type | Digital Input |
| Description | Generates alarm tone which informs manual announcement is in progress via interior speaker when signal which informs internal button has been pressed enters. The interior manual announcement is only available when the button has been pressed. |

- External

| Signal Name | External |
| --- | --- |
| Type | Digital Input |
| Description | Generates alarm tone which informs manual announcement is in progress via exterior speaker when signal which informs external button has been pressed enters. The exterior manual announcement is only available when the button has been pressed. |

- BOTH

| Signal Name | Both |
| --- | --- |
| Type | Digital Input |
| Description | Generates alarm tone which informs manual announcement is in progress via interior and exterior speaker when signal which informs both buttons has been pressed enters. The interior and exterior manual announcement is only available when the button has been pressed. |

PEI

| Signal Name | PEI Call |
| --- | --- |
| Type | Digital Input |
| Description | Generates alarm tone which informs manual announcement is in progress via interior speaker when signal which informs internal button has been pressed enters. The interior manual announcement is only available when the button has been pressed. |

CCP

- CAB ON

| Signal Name | CAB ON |
| --- | --- |
| Type | Digital Input |
| Description | Each CCP utilizes the adjacent Cab’s CAB ON signal to determine if it should be set as the Active CCP for the entire Consist. The CAB ON signal will be derived from the Cab Make-Up Relay (CMR) by monitoring the output voltage of the CAB ON signal line. If the CAB ON signal is present, then the specified CCP will be designated as the Active CCP. As such it will provide the operational interface to the driver. |

- RS-485 Car Line

| Signal Name | RS-485 CAR LINE |
| --- | --- |
| Type | Digital Input / Output |
| Description | The RS-485 Car Line data Input & Output signal is used to provide for digital communication between the PAC’s and the various devices within a Car. The RS-485 Car Line provides for data communication between the Primary PAC, CCP, Display Signs, CNCS, LIC, and the secondary PAC in the same car. Within any given car the Primary PAC maintains protocol control of the RS-485 Car Line network. All other RS-485 devices within the Car will typically be polled within a continuous loop by the Primary PAC. In most cases the primary PAC will issue requests for specific responses from each device, and the addressed device will provide the requested response. In certain cases it will be necessary for the Primary PAC to temporarily pass RS-485 control to another device so that it may affect the desired data communication. Additionally, the CCP will continually monitor the RS-485 port for relevant information which will be broadcast by other CS devices. |

- CCP GUI Keyboard Input

| Signal Name | CCP GUI Interface Keyboard Input |
| --- | --- |
| Type | Set of logical values |
| Description | This is the primary operational interface by which Crew members select the desired functions, enter operational parameters, or review status data that is provided by the on-board communication system. By touching the screen directly over the displayed icons the Touch Screen records the specific screen location and equates the touch to represent a specific response. As such, the CCP touch screen is used by the Crew to select the desired function from the Menu Bar, or to enter specific data into fields that are presented by the various screens. |

- CCP ID Address Signal

| Signal Name | CCP ID Address Signal |
| --- | --- |
| Type | Digital Input |
| Description | This identifies the CCP as being located adjacent to either CRA. The Address Line pin is hardwired in the harness to be “0” at PAC-MA and “1” at PAC-MB. |

CNCS

- RS-485 Car Line

| Signal Name | RS-485 CAR LINE |
| --- | --- |
| Type | Digital Input / Output |
| Description | The CNCS is located in CRA of each Car. After the Inauguration Process is completed the CNCS will communicate with the other CS devices utilizing the RS-485 Car Line. |

- UDP communication is utilized to receive status data and to send and receive operating commands related to the sub-software components.

| Signal Name | UDP Communication |
| --- | --- |
| Type | UDP data Input |
| Description | Utilize UDP protocol receive the status as well as to send and receive operating commands from each sub software component within the CNCS. |

- HDD

| Interface Name | HDD |
| --- | --- |
| Type | File Input / Output |
| Description | The HDD located within the CNCS is the storage media used to store S/W components, schedule files, and various content files needed by the CNCS and other devices for ongoing operations. The CNCS software components utilize the HDD as the primary data source and destination relative to the storage and retrieval of data files. Files which are downloaded from the WLAN to the CNCS are stored to the HDD for later distribution. Data that is collected or compiled by the CNCS is stored into files on the HDD for later review or transfer to the WLAN. |

- LIC – RS232

| Signal Name | LIC RS232 |
| --- | --- |
| Type | Digital Input / Output |
| Description | This interface is used to provide for a direct data channel between the CNCS and the LonWorks Interface Card (LIC) for access to the MDS. This channel is used to deliver the MDS data from the MDS to the CNCS prior to sending the data to the Wayside via FTP. It is also used to transmit the CS software version and the GPS time to MDS. |

- Wireless LAN to Wayside

| Signal Name | WLAN |
| --- | --- |
| Type | 802.11g Wireless LAN connection |
| Description | Provides for wireless LAN connectivity between the onboard communications equipment and the Wayside network infrastructure. |

- USB link to PAC MP3 Memory

| Signal Name | USB-MP3 |
| --- | --- |
| Type | Standard USB 2.0 protocol |
| Description | Provides for a direct USB connection between the CNCS and the PAC’s onboard USB Memory module. This allows for the direct copying of audio files for use by the MP3 audio player. |

- CNCS-LVDS Video Link

| Signal Name | LVDS Video Signal |
| --- | --- |
| Type | ANSI/TIA/EIA-644 Low Voltage Differential Signaling (LVDS) Interface |
| Description | Video output signal from the CNCS to the VTX board |

VTX

VTX H/W sets up its own ID through VTX_ID Address. Creates Protocol by the Protocol Generator Circuit and responds its own status with the PAC equipment via RS-485.

- RS-485 Communication Port

| Signal Name | RS-485 Serial Data Protocol |
| --- | --- |
| Type | Digital Input/Output |
| Description | Data Input/Output |

- VTX_ID Address

| Signal Name | VTX_ID Signal |
| --- | --- |
| Type | Digital Input |
| Description | Scan the device connector pins to determine the exact location of the device within the vehicle. The unique VTX_ID is assigned based upon this location from a lookup table. |

VRX

VRX H/W sets up its own ID through VRX_ID Address. Creates Protocol by the Protocol Generator Circuit and responds its own status with the PAC equipment via RS-485.

- RS-485 Communication Port

| Signal Name | RS-485 Serial Data Protocol |
| --- | --- |
| Type | Digital Input/Output |
| Description | Data Input/Output |

- VRX_ID Address

| Signal Name | VRX_ID Signal |
| --- | --- |
| Type | Digital Input |
| Description | Scan the device connector pins to determine the exact location of the device within the vehicle. The unique VRX_ID is assigned based upon this location from a lookup table. |

FDI

The FDI software component controls the actual LED display, and provides for the functionality allowing the device to display Route#, Destination information.  The S/W also facilitates RS-485 communication with the PAC controller to provide for content updating, content management, and system diagnostics.

- FDI RS-485 Communication Port

| Signal Name | FDI RS-485 CAR LINE |
| --- | --- |
| Type | Digital Input / Output |
| Description | The RS-485 Car Line data Input & Output signal is used to provide for digital communication between the FDI devices and other RS-485 devices within a Car. Within any given car the Primary PAC maintains protocol control of the RS-485 Car Line network. All other RS-485 devices within the Car will typically be polled by the Primary PAC within a continuous loop. In most cases the primary PAC will issue requests for specific responses from each device, and the addressed device will provide the requested response. In certain cases it will be necessary for the Primary PAC to temporarily pass RS-485 control to another device so that it may affect the desired data communication. |

- FDI_ID Address

| Signal Name | FDI_ID Signal |
| --- | --- |
| Type | Digital Input |
| Description | This signal is used to establish a unique FDI_ID parameter and as such to assign each FDI with a predetermined and specific RS-485 ID addresses. By scanning the connector pins, the FDI can determine the exact location within the Car where the specific FDI is installed. It will then establish an RS-485 Device _ID based upon parameter data retrieved from the local non-volatile memory. |

SDI

The SDI software component controls the actual LED display, and provides for the functionality allowing the device to display Route#, Destination information.  The S/W also facilitates RS-485 communication with the PAC controller to provide for content updating, content management, and system diagnostics.

- SDI RS-485 Communication Port

| Signal Name | SDI RS-485 CAR LINE |
| --- | --- |
| Type | Digital Input / Output |
| Description | The RS-485 Car Line data Input & Output signal is used to provide for digital communication between the SDI devices and other RS-485 devices within a Car. Within any given car the Primary PAC maintains protocol control of the RS-485 Car Line network. All other RS-485 devices within the Car will typically be polled by the Primary PAC within a continuous loop. In most cases the primary PAC will issue requests for specific responses from each device, and the addressed device will provide the requested response. In certain cases it will be necessary for the Primary PAC to temporarily pass RS-485 control to another device so that it may affect the desired data communication. |

- SDI_ID Address

| Signal Name | SDI_ID Signal |
| --- | --- |
| Type | Digital Input |
| Description | This signal is used to establish a unique SDI_ID parameter and as such to assign each SDI with a predetermined and specific RS-485 ID addresses. By scanning the connector pins, the SDI can determine the exact location within the Car where the specific SDI is installed. It will then establish an RS-485 Device_ID based upon parameter data retrieved from the local non-volatile memory. |

PII

This software is consistent with the FDI and SDI software but also monitors the AVL messaging system commands to display automated route, way finding, and emergency passenger information notifications.

- PII RS-485 Communication Port

| Signal Name | RS-485 CAR LINE |
| --- | --- |
| Type | Digital Input / Output |
| Description | The RS-485 Car Line data Input & Output signal is used to provide for digital communication between the PII and other RS-485 devices within a Car. Within any given car the Primary PAC maintains protocol control of the RS-485 Car Line network. All other RS-485 devices within the Car will typically be polled by the Primary PAC within a continuous loop. In most cases the primary PAC will issue requests for specific responses from each device, and the addressed device will provide the requested response. In certain cases it will be necessary for the Primary PAC to temporarily pass RS-485 control to another device so that it may affect the desired data communication. |

- PII_ID Address

| Signal Name | PII_ID Signal |
| --- | --- |
| Type | Digital Input |
| Description | This signal is used to establish a unique PII_ID parameter and as such to assign each PII with a predetermined and specific RS-485 ID addresses. By scanning the connector pins, the PII can determine the exact location within the Car where the specific PII is installed. It will then establish an RS-485 Device _ID based upon parameter data retrieved from the local non-volatile memory. |

LIC

Functional Requirements

PAC

When power is applied to the CS, each PAC will perform a series of initialization processes. The following diagram is intended to provide an overview of the PAC software components and the sequence of flow throughout the PAC initialization process.

Diagram 3.1.1.2-A: Overview & Flow of PAC Functional Components

- PAC Initialization

Diagram 3.1.1.2-B: PAC Initialization Flow Chart

| ID | Fun-REQ-PAC-Initial-01 |
| --- | --- |
| Function | Initializes all components & parameters inherent to the PAC device |
| Description | This functional requirement definition includes elements necessary to initialize a number of operating parameters, perform a set of default system tests, and place each PAC into a default operating mode. - Scan the PAC_ID Address line to determine if it is located in CRA. Set the PAC_ID Parameter. - All PACs identified to be PAC-MA must set an internal parameter indicating that they are to initially operate as the PRIMARY PAC. All PACs identified as PAC-MB must set an internal parameter indicating that they will initially operate in the SECONDARY PAC mode. The PRIMARY & SECONDARY designations are made with reference to the two PACs located within a Single and/or Married Pair Car. The PRIMARY designation indicates which PAC has priority within each Car and should not be confused with the designations of MASTER and SLAVE PACs. - Each PAC sets a separate parameter indicating that it will initially operate in the default SLAVE mode. MASTER and SLAVE designations are used to identify PAC priority within an entire Train Consist. All PACs will operate in the SLAVE mode with the exception of the one PAC within the Consist that is directly connected to the Master Cab, (The Cab in which the Driver has entered his key and placed it into the CAB ON state). - Evaluate and record the state of the COUPLER Signal as the COUPLER Parameter to indicate if the adjacent Coupler is in an open or closed position. If the Coupler Signal state changes, then the PAC must immediately update the COUPLER Parameter to indicate the correct state. - If the COUPLER parameter indicates an “Open” state, then set a TOC (Tail of Consist) Parameter indicating that the specific PAC has a TOC token (Flag). - Retrieve the RS-485 Car Line Address ID table from nonvolatile local memory. - Attempt to establish RS-485 Car Line communication between the two sister PACs, (PAC-MA and PAC-MB). Perform a series of retries if initially unsuccessful in case the different PAC devices require varying lengths of time to complete their own POST processes. - Create a Pass or Fail status parameter depending upon the results. - Repeat this process for each of the remaining RS-485 Car Line devices. - If any of the devices have successfully responded to the request for status command, then retry to establish communication with the Sister PAC. - Report the Status data to the MDS - Retrieve the RS-485 Train Line Address ID table from nonvolatile local memory. - Issue a command to OPEN the RS-485 Train Line relay on the Coupler side of the PAC leaving the line between the two PACs intact, (The Sister PAC will similarly be performing the same function). - Attempt to establish RS-485 Train Line communication between the two Sister PACs, (PAC-MA and PAC-MB). - Create a Pass or Fail status parameter depending upon the results. - Report the Status data to the MDS - Check for the existence of a valid CAR NUMBER Parameter stored within its internal memory. If no valid CAR NUMBER parameter exists, then Step #9 must be performed, if a valid CAR NUMBER does exist then utilize the existing Parameter and skip to Step #9 below. - If the Car Number parameter doesn’t exist, PAC-MA and PAC-MB will each receive the car number from the MDS in a different manner. - PAC-MA will request and retrieve the Car Number from the MDS via the RS-485 Car Line connection to the LIC and thus the MDS. After the number has been received and stored as the CAR NUMBER parameter in local memory, the PAC-MA will send an RS-485 telegram to PAC-MB containing the appropriate number. - PAC-MB relies on PAC-MA to provide the CAR NUMBER information. - Both PAC-MA and PAC-MB will perform an analysis of status data and test results to evaluate the viability of PAC-MA as the PRIMARY PAC. If PAC-MA is determined to be in a failed state, then PAC-MB will replace PAC-MA as the PRIMARY PAC and set its operating parameter accordingly. PAC-MA will either a) set itself as SECONDARY PAC if possible and appropriate, or b) be considered as disabled if its watchdog circuitry has shut down the RS-485 connectivity. - Evaluate the CAB ON Parameter. - Execute Fun-REQ-PAC-Analysis-01. This is the second phase of the PAC initialization process and performs an analysis of all devices within the specific vehicle. |

- Check the Status of all Devices within a specific Car

Diagram 3.1.1.2-C: RS-485 Car Line Device Analysis

| ID | Fun-REQ-PAC-Analysis-01 |
| --- | --- |
| Function | Poll and record the status of all RS-485 Car Line devices |
| Description | All Primary PACs perform the function of checking the Status of all devices within a specific Car. The PAC accomplishes this by utilizing the RS-485 Car Line infrastructure to poll each predefined device within the Car and attempts to retrieve its status data. - Load the RS-485 Car Line Address Table data & Polling Sequence data from nonvolatile memory. These tables will include specified polling loops, sequential device lists by polling loop, and each device’s corresponding RS-485 Address ID. - Select the first specified loop & device. (Process sequentially 1 through N) 1. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; VTX; CNCS_M; CCP_M; REC_M; SDI3; PIS_ALL. 2. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; CCP_S; CNCS_M; CCP_M; REC_M; SDI4; PIS_ALL. 3. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; REC_S; CNCS_M; CCP_M; REC_M; PID11; PIS_ALL. 4. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; PII1; CNCS_M; CCP_M; REC_M; PID12; PIS_ALL. 5. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; PII2; CNCS_M; CCP_M; REC_M; VOIP_S; PIS_ALL. 6. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; FDI1; CNCS_M; CCP_M; REC_M; PID13; PIS_ALL. 7. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; FDI2; CNCS_M; CCP_M; REC_M; PID14; PIS_ALL. 8. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; SDI1; CNCS_M; CCP_M; REC_M; PID21; PIS_ALL. 9. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; SDI2; CNCS_M; CCP_M; REC_M; PID22; PIS_ALL. 10. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; PII3; CNCS_M; CCP_M; REC_M; PID15; PIS_ALL. 11. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; PII4; CNCS_M; CCP_M; REC_M; PID16; PIS_ALL. 12. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; SDI5; CNCS_M; CCP_M; REC_M; PID17; PIS_ALL. 13. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; SDI6; CNCS_M; CCP_M; REC_M; PID18; PIS_ALL. 14. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; SDI7; CNCS_M; CCP_M; REC_M; PID23; PIS_ALL. 15. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; SDI8; CNCS_M; CCP_M; REC_M; PID24; PIS_ALL. 16. CNCS_M; CCP_M; VOIP_M; PAC_S; LIC_M; CNCS_S; CNCS_M; CCP_M; REC_M; LIC_S; PIS_ALL. - Select specified device address. - Request device status via RS-485 Car Line - Does device respond to request (Y/N) - If YES: Record the retrieved Device Status in the local PAC status table. - If NO: Retry request three times, if there is still no response then initiate the error handler and record the fault in the PAC status table - Select next Device and return to step #3 until all devices have been polled. - Select the next specified loop & device, and return to step #3 above. (Process sequentially 1 through N). When all loops are complete, STEP forward. - All device status for the specified Car will now be included within the Device Status Table stored in the local PAC’s internal memory. Number of devices per Car; Scan time per each status request; Number of Retries; Approximate cycle time per Car. Max 36; 100ms; 3; About 15 sec. 9) Evaluate CAB ON Parameter (Skip this step if a Valid Consist Exists) - a) If CAB ON = “1” (True): Launch Fun-REQ-PAC-Analysis-02 - b) If CAB ON = ”0” (False): Launch Fun-REQ-PAC-Analysis-04 - 10) Repeat the scan cycle at step #2 based upon ”Cycle timer” and “Interrupts” |

- Consist Inauguration

Diagram 3.1.1.2-D: Consist Inauguration Process Flow Chart

| ID | Fun-REQ-PAC-Analysis-02 |
| --- | --- |
| Function | Develop the Train Consist Configuration Table by utilizing the RS-485 Train Line infrastructure to discover and sequence all coupled Cars and attached PAC devices. Store the completed table on all PAC devices. |
| Description | This functional element performs the actual Train Consist Inauguration process and compiles a consolidated Consist Configuration Table. This process is performed by the Master PAC which writes the data table to its internal memory and replicates the table to all other PAC’s in the Consist. Diagram 3.1.1.2-D above, (Consist Inauguration Process Flow Chart), illustrates the sequence of steps which are performed by the Master PAC throughout the Inauguration process. The following items represent a summary of the entire process. - Given that the CAB ON signal is present, set this PAC as the Master PAC. - Does the Master PAC hold a TOC Token, (Is the Coupler Open)? - If YES, Create an HOC token on the Master PAC (It is Head of the Consist) - If NO, Open the RS-485 Train Line relay toward the sister PAC, and issue a request for response from the TOC Token holder. The PAC that responds is the true HOC. Instruct the responding PAC to set itself as the HOC and record its PAC_ID and Car# in the Consist Configuration table. - Close all RS-485 relays and issue a request for response from the remaining TOC Token holder. Only one other Car in the Consist can possibly hold a TOC Token. Therefore, the PAC that responds is the true TOC. Record its PAC_ID and Car# into the Consist configuration table. - Place a PASS Token on the PAC holding the HOC token. - Pass the PASS token from the HOC to the TOC in a sequence of one PAC at a time. Record the PAC_ID, Car#, and Consist Position of each PAC before the Token is passed from one PAC to the next PAC. The RS-485 relay device state configurations and software processes necessary to accomplish this portion of the Inauguration are contained in the following table, (Table 3.1.1.2-A). - When the PASS Token and TOC Token reside on the same Car # and have the same PAC_ID, then the Token passing process is complete. - Finalize the Consist Configuration Table to the Local Master PAC memory. - Initiate function Fun-REQ-PAC-Analysis-03 to consolidate all of the Device Status tables located on each of the PRIMARY PACs within the Consist back to the Master PAC for distribution to the CCP. |

- Table 3.1.1.2-A: Consist Inauguration, RS-485 relay State table

- Retrieve the Device Status tables from all PRIMARY PAC’s.

Diagram 3.1.1.2-E: Master PAC – Consist PAC Device Status Table retrieval

| ID | Fun-REQ-PAC-Analysis-03 |
| --- | --- |
| Function | Retrieve the Device Status table from the PRIMARY PAC of each Car in the Consist and record the data in the Master PAC’s Device Status table. |
| Description | Upon the successful completion of the Inauguration Process, the Master PAC will have a valid Consist with confirmed RS-485 Car Line continuity. As such, it can perform the function of retrieving the Device Status tables from each of the PRIMARY PACs throughout the Consist. The Master PAC must poll each of the PRIMARY PACs listed in the Consist Configuration table, and then aggregates the data within the Master PAC. Once aggregated in the Master PAC all Consist Device Status data is routed to the CCP where it can be viewed by the Driver. - Retrieve the appropriate data from the Consist Configuration Table to develop a sequential list of all PRIMARY PAC devices within the Consist. The list will identify Cars 1 through N of the Consist, including each Car’s position in the Consist, its true Car #, and the device ID of each PRIMARY PAC. - Select the first PAC_ID in the sequential list, and retrieve its RS-485 ID. - Is the selected PRIMARY PAC also the MASTER PAC? - YES: Copy the Local Car’s device status table data to the Consist device status table. - NO: The Master PAC sends a “Request to Send the Device Status Table” to the specified PAC. The specified PAC responds by sending the requested data. The Master PAC appends the retrieved data to the Consist device status table. - Process any errors via an error handler - Select the next PAC_ID in the sequential list, retrieve its RS-485 ID, repeat step #3 until all remaining PRIMARY PAC’s in the Consist have been polled. - Select the RS-485 Device ID of the CCP - Send the consolidate Consist Device Status Table data to the CCP for display and to the CNCS to be written in a Log file. |

- PA Audio Train Line level balancing

| ID | Fun-REQ-PAC-Analysis-04 |
| --- | --- |
| Function | The Master PAC initiates a process that injects a known “Reference Tone” onto the PA audio train line which is then used by each PAC in the Consist to establish a “PA Train Line Gain” parameter. This will ensure a consistent train line audio level. |
| Description | This component is provided to meet the technical specification requiring that the PA audio level of the Train Line not sustain a loss in excess of 3db across the entire Consist. To accommodate this requirement, the PAC design includes a signal generator which is capable of producing a 11 KHz “Reference Tone”. The Master PAC will inject this “Reference Tone” onto the PA Audio train line for use by all SLAVE PACs as a known reference level. Each SLAVE PACs will then discover the actual output level of the PA audio line and compare it to the parameter defined “PA Train Line Output Level”, and calculate an appropriate “PA Train Line Gain” parameter. This element must function in two different ways depending upon whether a specific PAC is operating in the MASTER or SLAVE mode. - • MASTER PAC: - Utilize the onboard tone generator to create a 11 KHz tone. - Feed the 11 KHz signal onto the PA Audio train line at an output level which is consistent with a pre-defined parameter for the PA TEST signal. - Sustain the Test Tone for a sufficient length of time to allow each of the other PACs in the Consist to detect the 11 KHz tone, evaluate the ensuing output level, and establish an appropriate “PA Train Line Gain” parameter. -  - • SLAVE PAC: - Load the “PA Train Line Output Level” parameter from the PAC memory. - Monitor the PA Audio train line for a 11 KHz tone. Once the 11 KHz tone is detected for a specified period of time, then measure and record the audio line level as a reference to evaluate the sustained db loss at that point. - Compare the current output level at each PAC to the level specified in the “PA Train Line Output Level” parameter. - The MASTER PAC will establish the overall PA Audio line loss across the Consist by requesting the TOC PAC to report its level variance in comparison to the predefined reference level parameter value. - Is the overall loss across the Consist greater than 5db? a) NO: No level adjustment is required. Set the PA Gain parameter to = “0” b) YES: Calculate an appropriate audio level gain adjustment that may be applied by each PAC to maintain the line level within 3db across the Consist. - Establish a “PA Train Line Gain” parameter to reflect the determined gain adjustment, and store it in the local memory of each PAC. 7) The PACs will apply this adjustment level for all PA announcements. **Note: The specific algorithm utilized to calculate an appropriate gain level will be determined by the hardware engineer during the development process. |

- Monitor Consist Integrity

| ID | Fun-REQ-PAC-Analysis-05 |
| --- | --- |
| Function | Monitor Consist Integrity, Coupler Status and CAB ON Signal |
| Description | This function continually evaluates the integrity of the Consist by monitoring the current states of the CAB ON and COUPLER signals. Any state change relative to either signal will require some form of analysis and/or resulting action. In the event that a CAB ON signal state change from “1” to “0” is detected, then the MASTER PAC must set a timer to evaluate the length of time the “0” state persists. If the time interval is determined to be within a specified parameter value range, then the existing Consist Configuration shall be considered to remain valid. If the time interval exceeds a parameter defined limit, then the PAC will re-inaugurate. In the event that a COUPLER state change is detected by any PAC in the Consist, then a similar analysis must be performed. Relatively short COUPLER state changes, (parameter defined duration), may be considered to be inconsequential, while longer ones must result in further analysis, notifications, error logging, and re-inauguration. |

- Monitor Intercom Status

| ID | Fun-REQ-PAC-Analysis-06 |
| --- | --- |
| Function | Evaluate the IC LIVE Lines signal states to provide device Status Data |
| Description | This function monitors the 12 hardwired IC LIVE signal lines which are direct inputs to the PAC from the two DPO’s, two DPH’s, and eight PEI control boxes. Each of these IC LIVE signal lines must be scanned and evaluated on an ongoing basis to indicate if the device is functional on or not. If a DCV signal is present, then the device is powered on and is ready for operation. This signal does not indicate that an IC Call is in progress, but merely indicates that the device is powered on and is operational. The value of each IC LIVE Line signal state corresponding to the 12 IC devices must be stored as separate parameters within the in-memory PAC Parameter Table. The parameter table values must be regularly updated to reflect the state of the corresponding physical device’s IC LIVE Line. *Note: “0” and “1”: (“1”= DPO/ DPH /PEI box is Operational, “0” = Non-Operational) |

- Process Local PEI Address Status Interrupt

| ID | Fun-REQ-PAC-Address-01 |
| --- | --- |
| Function | Process Local PEI ADDRESS Status Interrupt |
| Description | When a PEI CALL button is pressed, the specific device will generate a 15VDC signal on the PEI Address line. This function monitors the several PEI ADDRESS lines and updates parameters indicating the state of each Address line. It writes a value representing the device state to the PAC Parameter table located within the local memory of the PAC. Values: -  “0” = Inactive (No CALL is in progress on this specific PEI Box). -  “1” = Active (The CALL button has been pushed and a call is in progress) |

- Query Train Line PEI Address Status

| ID | Fun-REQ-PAC-Address-02 |
| --- | --- |
| Function | Query Train Line PEI Status |
| Description | - The MASTER PAC retrieves the Local PEI ADDRESS status data elements from each primary PAC in the Consist and writes the data to the Master PAC Status Table. - The stored status data identifies all PEI devices in the Consist, indicating the physical car#, specific location within, and the PEI ADDRESS status of each device. - Compares the retrieved status values to the previously stored values to determine if any state changes have occurred. - In the event that any state change has occurred, sends a telegram to the CCP via the RS-485 Car Line indicating the state change event. - Queues the corresponding MP3 Audio File ID(s) to be played via the Automatic Broadcast player. - Forwards the appropriate status data to the CCP enabling the CCP to display the current device states and proper indications to the crew. |

- Query Train Line IC LIVE device status, (DPO/ DPH/PEI).

| ID | Fun-REQ-PAC-Device Status-01 |
| --- | --- |
| Function | Query IC LIVE Device States |
| Description | - The MASTER PAC retrieves the Local IC LIVE status data elements from each primary PAC in the Consist and writes the data to the Master PAC Status Table. - The stored status data identifies all IC devices in the Consist, indicating the physical car#, specific location within, and the IC LIVE status of each device. - Compares the retrieved status values to the previously stored values to determine if any state changes have occurred. - In the event that any state change has occurred, sends a telegram to the CCP via the RS-485 Car Line indicating the state change event. - Forwards the appropriate status data to the CCP enabling the CCP to display the current device states and proper indications to the crew. |

- Monitor & Process AVL Trigger Events

| ID | Fun-REQ-PAC-Initial AVL-01 |
| --- | --- |
| Function | Monitor & Process AVL Trigger Events |
| Description | This function processes RS-485 Car Line data packets related to the handling of AVL messaging triggers. AVL triggers may reference any or all of the following types of massages and content: - Audio Announcements – Route Service Announcements - Audio Announcements – PEI Emergency Assistance Announcements - LED Sign Messages – Route Service Notifications - LCD Display – Route Service Notifications - LCD Display – Advertisement & Public Service Notifications As part of the AVL route processing performed by the Master CCP, the CCP generates an AVL trigger whenever the vehicles GPS coordinates are within a margin of error proximity of a defined trigger point coordinate pair. All relevant trigger information is formatted into a data packet and distributed via the RS-485 Car Line. The Master PAC must perform the following functions. - Evaluate all RS-485 data packets to determine if they represent a triggered event. - If so, determine the nature of the triggered event and initiate the appropriate AVL messaging action(s). - IF an audio message is indicated by the trigger, then load the PA system operating parameters associated with the trigger definition and the current system configuration. This may include speaker selection, volume level, muting control, and Gain control parameters. - IF this is the MASTER PAC: - Serve as a gateway to route all trigger data packets from the RS-485 Car Line to the RS-485 Train Line. - Pass initiation parameters to the MP3 Broadcast Player for any audio message(s) specified by the trigger. - Route the output, (If any), of the MP3 player to the specified local amplifiers and to the PA Audio Train Line. - IF this is a SLAVE PAC: a) Serve as a gateway to route all trigger data packets from the RS-485 Train Line to the local RS-485 Car Line. b) If an audio signal has been placed onto the Audio Train Line by the MASTER PAC, then turn on the amplifiers as specified by the parameters and route the Train Line audio signal to the local amplifiers. - Monitor the door control state as required in order to confirm AVL trigger point authenticity decisions. - Adjust the volume of the interior / exterior speakers. |

CCP

- CCP Initialization

| ID | Fun-REQ-CCP-Initial-01 |
| --- | --- |
| Function | CCP Device initialization |
| Description | When the MASCON Key is placed in any of the “ON” positions, all CCP devices in all cars of the Consist will be supplied with power and begin their initialization process. The individual CCPs will respond differently depending on the following conditions. - Is the CCP directly connected to the MASTER PAC. If so, then it is designated as the MASTER CCP and will be used as the primary interface between the driver and the CS. - Is the CCP located in the same Car as the Master PAC but not directly connected. If so, it is designated as the SECONDARY CCP and could possibly be used to interface with the CS in a backup role. - The CCP is located in a different Car from the MASTER PAC. If so the CCP has no direct RS-485 Car Line connection to the MASTER PAC and as such can only support limited functionality. These will be referred to as PASSIVE PACs. Upon power up each CCP will perform a POST process followed immediately by loading the Windows CE operating system from its local memory module. Once the OS has been loaded the CCP will automatically execute the following sequence: - Load the RS-485 Drivers - Load the “CCP GUI” application software component. - Evaluate the connector Signal to determine if the specific CCP is installed adjacent to CRA or CRB and set a CCP_ID parameter accordingly. - Set a parameter indicating the appropriate RS-485 Car Line ID - Set a parameter indicating that the current Train Number = “ “ - Set a parameter indicating that the current Crew ID Number = “ “ - Restore all available operating parameters from the local Nonvolatile RAM. This will include the parameter indicating the physical Car number. If no Car number exists, then request it from the PAC via the RS-485. - Evaluate the CAB ON signal to determine the appropriate operating mode: - IF the CAB ON signal is present, then the CCP sets a parameter indicating that it will operate as the MASTER PAC. - If the CAB ON Signal is not present then the CCP must further evaluate whether it is located in the same Car as the MASTER PAC and CCP. If “YES” then it will operate in a SECONDARY mode and will restore and evaluate any current operating parameters from memory and display the current operating parameters settings. If “NO” then it will operate in a PASSIVE mode and will only be able to display parameters relating to the specific Car in which it is located. The screen will remain dark unless touched. Once activated it will return to dark after 75 seconds of no activity. - Is this CCP the MASTER CCP: - NO: Go To VIII - YES: Execute the Fun-REQ-CCP-Control-01 which will prompt for and set the train #. - Idle Wait |

- Set Train Number

| ID | Fun-REQ-CCP-Control-01 |
| --- | --- |
| Function | Set Train Number |
| Description | This function provides for a mechanism by which the operator can enter the desired train number into the CCP GUI. Once entered, the number is validated against the master DENVER RTD train schedule and causes the CCP to retrieve the corresponding schedule elements and AVL trigger data from the CNCS and load it into the CCP memory. Upon initiation by touching the “TRAIN” button on the selection bar of the CCP GUI, it will perform the following functions: - The CCP must display the “Set Train Number” screen. - Prompt the operator to “Please enter Train Number” and simultaneously display a graphical key pad entry screen. - The driver then enters the train number via the presented Key pad and touches the “Enter” button to confirm the entry. The train number must be visibly displayed at the time of input and the operator must be provided with a “Clear” key to correct any erroneous entries. - Once the entry has been confirmed, the CCP must validate the entered “Train Number” against the current DENVER RTD operating train schedule. If the entry is not valid, then the operator must be notified of the error and prompted to re-enter a different Train Number. - If the entry is a valid “Train Number”, then the CCP must retrieve the corresponding schedule elements and AVL trigger data from the CNCS and load it into the CCP memory. |

- Set Crew ID

| ID | Fun-REQ-CCP-Control-02 |
| --- | --- |
| Function | Set Crew ID |
| Description | This function provides for a mechanism by which the operator can enter his assigned crew ID into the CCP GUI. Once entered, the crew ID data is written as a parameter to the CCP memory and is also forwarded via the RS-485 Car Line to the CNCS for recording into the activity log file. This function is initiated by touching the “CREW” button on the selection bar of the CCP GUI. At the time of train inauguration, this function “Set Crew ID” is automatically invoked by the CCP initialization procedure right after a valid Train Number is entered by the operator. - The CCP must display the “Enter Crew ID” screen. - The operator will be prompted to “Please enter Train Number” and will simultaneously be presented with a graphical key pad entry application. - The driver enters the crew ID via the presented Key pad and touches the “Enter” button to confirm the entry. The crew ID must visibly display at the time of input and the operator must be provided with a “Clear” key to correct any erroneous entries. - Once the operator touches the “Enter” keypad to confirm the crew ID, the CCP will validate the entered “Crew ID”. The CCP will display an error message to the operator to re-enter a valid crew ID if the entered crew ID is not valid. - Once a valid crew ID is entered, the Crew ID# is stored in the CCP’s parameter table within internal memory, and forwarded to the CNCS for inclusion in the activity log file. |

- Set Route Information

| ID | Fun-REQ-CCP-Control-03 |
| --- | --- |
| Function | Set Route Information |
| Description | This function provides a mechanism by which the operator can initially view and then edit the route information on the CCP screen as it pertains to the current Train Number. The CCP will display the departure station stop, scheduled time of departure and any delays related to the schedule adherence for each station stop of the train route. This function is initiated by touching the “ROUTE” button on the selection bar of the CCP GUI. To change or skip station stops of the scheduled train route, the operator needs to touch the “ROUTE” button again. The CCP will then provide a different route screen whereby the operator can select: - “STOP”: to stop at a specified train station of the train route. - “SKIP” : to skip one or more station(s) of the train route; - “EXP” : to specify an Express schedule without stopping at most stations of the scheduled train route. - “LOCAL”: to change an Express train schedule to stop any every station. After each route change, the CCP will relay any relevant route change data via the RS-485 back to the PAC and CNCS. All changes are logged to the CNCS activity log file. |

- Set Configuration Parameters

| ID | Fun-REQ-CCP-Control-04 |
| --- | --- |
| Function | Set Configuration Parameters |
| Description | This function provides a mechanism by which the operator can change the PA audio volume level, the LCD panel’s brightness, or reset the CS parameters to their initial default values. The function is initiated by touching the “CONFIG” button on the selection bar of the CCP GUI. This is the main “CONFIG” screen by which the user will be presented with various options for adjusting the system parameter settings. Three spare buttons are available for future features. The default level values pertaining to the NORMAL and SOFT parameter settings for both the interior and exterior speakers are set by a separate function via the PTU. This function merely allows the operator to select the NORMAL, SOFT, or MUTE operating modes. - To change the Speaker volume, the operator will select the “SPEAKER VOLUME” button on the “CONFIG” screen. The GUI will then display a separate screen allowing the operator to independently set the volume level of both the Interior and Exterior speakers across the Consist. The parameter settings must include the following choices: a) Interior Speakers:  NORMAL  SOFT b) Exterior Speakers:  NORMAL  SOFT  MUTE - To change the LCD panel brightness, the operator needs to first touch the “CONFIG” button on the selection bar of the CCP GUI. Next, touch the “LCD DIMMER” button and a level adjustment screen will be displayed allowing the operator to chose the desired level via a sliding value bar within a range from 20% to 100%. - By selecting the “CONFIG” button followed by the “RESET” button, the CCP will restore the CS parameters to their initial default values. Upon completion, press the “HOME” button to return to the normal CCP operational review screen. The changed parameters will be stored to the CCP memory and sent via the RS-485 to the PAC for distribution throughout the Consist. |

- Display the status of RS-485 monitored devices throughout the Consist

| ID | Fun-REQ-CCP-Control-05 |
| --- | --- |
| Function | Display the status of RS-485 monitored CS devices throughout the Consist |
| Description | This function enables the driver or a crew member to quickly assess the status of the CS devices within the entire Train Consist on a Car by Car basis. By touching the “CHECK” button on the selection bar of the CCP GUI, a graphical representation of the current Consist and the status of the various sub-systems are displayed. Each car within the Consist is graphically depicted on a single screen allowing the operator to select a specific Car by touching the numbered Icon. The specific Car is then graphically depicted by illustrating the CS components within that Car as well as the current operating status of each device. Different colors and text descriptions are used to indicate different system fault conditions. Properly functioning components are labeled and depicted in green. Non-functional components are labeled and depicted in red. The exact color scheme to depict the different system faults will be defined in the SDDS. Exact message verbiage of the different system faults messages will be defined in the SDDS. The actual device states displayed are derived from device status data elements provided to the CCP by the MASTER PAC via the RS-485 Car line. The MASTER PAC maintains a parameter table which includes the current status of all monitored CS devices throughout the Consist. For further information regarding the device status collection process, please refer to functions Fun-REQ-PAC-Analysis-01, 03, 06; Fun-REQ-PAC-Address-01, 02; and Fun-REQ-PAC-Device Status-01 which are all located within the PAC Functional Requirements section of this document. |

- Display the CCP Help Screens

| ID | Fun-REQ-CCP-Control-06 |
| --- | --- |
| Function | Display the CCP Help Screens |
| Description | This function is a mechanism by which the operator can briefly view the on-board quick reference instruction guide of the Denver RTD Eagle P3 vehicle. To start this function, the driver touches the “HELP” button on the selection bar of the CCP GUI. The HELP menu system is consist of seven sections. The sections are named “TRAIN”, CREW”, “ROUTE”, “CONFIG”, “CHECK”, “HELP” and “PEI”. Touch and select any of the seven HELP topics presented within the HELP menu screen. If the operator touches the “TRAIN” button on the HELP menu screen, a new HELP subtopic screen should appear that describes and guides the operator through entering the Train Number into the CCP GUI. Likewise if the operator touches and selects the “CHECK” button of the HELP menu screen, a new HELP subtopic screen should appear that describe and guide the operator in understanding the different color coding of all the components on the train. |

- Emergency PEI Call Notification

| ID | Fun-REQ-CCP-Control-07 |
| --- | --- |
| Function | Emergency PEI Call Notification |
| Description | This function provides visual notification to the driver or the operator in the event that a passenger presses a PEI Call button from any car of the Consist. When a passenger presses a PEI “CALL” button, the Master PAC will immediately be forwarded status data indicating both the car # and the specific PEI device # which has initiated the call. The PAC forwards this status data to the CCP which is converted into a visual message displayed by the CCP informing the operator that a passenger needs assistance in a specific Car number. |

- Monitors, Reads and Transmits RS-485 Car Line data

| ID | Fun-REQ-CCP-Control-08 |
| --- | --- |
| Function | Receives & Transmits RS-485-data |
| Description | This function provides for the ability to monitor and respond to RS-485 requests. The CCP must be capable of responding to normal RS-485 requests, and must be capable of initiating a request by setting a status flag indicating that it has information to send. The Primary PAC will respond by requesting that the CCP provide the required data according to the appropriate protocol. The following requirements and process elements must be handled by this function: - Load / adhere to RS-485 protocol definitions - Set Status and Function flags as necessary - Support RS-485 communication in either a “Normal” mode or in a “Bulk Data Transfer” mode. - Respond to requests for status initiated by either the Primary PAC or the CNCS. - Evaluate RS-485 data received and compare it to the protocol definitions to determine the nature of the data received and the action required. - Evaluate the specific command sequences specified - Assimilate the appropriate controls, protocols, and processes necessary to affect the desired actions. - Send RS-485 data responses as required and in the specified manner. |

- Set Status Update Flag

| ID | Fun-REQ-CCP-Control-09 |
| --- | --- |
| Function | Set Status Update Flag |
| Description | This function enables the CCP to set or clear a flag indicating that it either needs data from another source or has data to deliver to another source. Additionally this function must also incorporate the flexibility to set the operating mode of the CCP’s RS-485 communication to either the “NORMAL” or “BULK DATA” mode. |

- Receive bulk data Mode

| ID | Fun-REQ-CCP-Control-10 |
| --- | --- |
| Function | Receive bulk data Mode |
| Description | The purpose of this function is to provide for a mechanism by which the CCP can be switched between operating in a “NORMAL” handshake mode or operating in a “BULK DATA” broadcast mode. |

- Display Current Route and Schedule Adherence data

| ID | Fun-REQ-CCP-Display-01 |
| --- | --- |
| Function | Displays Current Train Operational Information |
| Description | This function displays the normal schedule route of a train schedule. It allows the driver to review and edit the Train route in the event of unknown weather created or road closure or station stops conditions that may require schedule changes and other - The current Train number and corresponding Route number is displayed on the CCP’s LCD based on information retrieved from the train service schedule corresponding to the train number parameter input by the crew. - Station names corresponding to the designations of “Departure”, “Actual” and “Next1” as indicated in the train service schedule are displayed. Adjacent to each display entry, the function will also display the scheduled departure times, actual departure times, and delayed times standardized on the basis of present time. |

- AVL GPS Route Processor

| ID | Fun-REQ-CCP-Comm-02 |
| --- | --- |
| Function | Plot real time GPS coordinates against the prerecorded route coordinates. |
| Description | This function is used to track a train’s progress through the specific scheduled route that has been selected by entering the Train Number into the CP. The CCP will compare real time Train location coordinates provided by the GPS against prerecorded route coordinate data as a means to plot the vehicles current position within the route. Each time a coordinate has been acquired, the CCP will compare that coordinate to the scheduled trigger point coordinates to determine if the acquired point is also a defined trigger point. - Retrieve the Train # from the CCP parameter table. - Extract the route definition coordinate data list, and the trigger point coordinate list from the master schedule data base file which corresponds to the selected route. - Compare the current location data to the route definition list data to identify the data element that represents the closest proximity to the current location and calculate the actual distance between the two points. - In the event that the measured distance to any given point is deemed to be less than a predefined margin of error distance from that given point, then the CCP will deem the specific point to have been acquired. - Each acquired pointed will be recorded and stored as the “Current Location” parameter to the CCP memory. This parameter will be updated to a new location value each time a subsequent coordinate location within the defined route has been acquired. - Compare the “Current Location” parameter to the “Trigger Point Coordinate List” to determine if the current location is a defined trigger point. - If the current location is a defined trigger point, then set a status flag indicating that a new trigger point has been acquired and indicate the specific trigger point number in a “Current Trigger Point” parameter. |

- AVL Event Trigger Processor

| ID | Fun-REQ-CCP-Comm-03 |
| --- | --- |
| Function | Continually evaluates the “Current Trigger Point” parameter and creates appropriate trigger point RS-485 telegrams accordingly. |
| Description | Once a trigger point has been identified by function Fun-REQ-CCP-Comm-02 above, then this function actually identifies the actions that are to occur as a result of acquiring the specified event trigger. - Retrieve Train # from the CCP Parameter table. - Extract the specified route station list, trigger point definitions, and content play list definitions from the master schedule data base file, which correspond to the selected route. - Continually monitor the “Current Trigger Point" parameter from the CCP parameter table. In the event that the value has changed since the previous read of the parameter, then a new trigger point has been acquired. - Search the selected schedule data for entries associated with the current trigger point value. Create an RS-485 trigger event telegram by incorporating the selected data into a valid RS-485 protocol data packet. - Set a status flag indicating that a “Trigger Telegram” is pending. - Send the telegram when requested by the MASTER PAC. |

CNCS

- After the CNCS completes its boot process it will automatically execute an application called LAUNCHER.EXE which will perform a series of software version checks.

| ID | Fun-REQ-CNCS Launcher-01 |
| --- | --- |
| Function | Validate & Update Software Components |
| Description | LAUNCHER.EXE must perform the following functions: 1) Access a specified directory within the HDD file structure. 2) Search for the existence of newly uploaded Application (.EXE) files. 3) Perform a CRC check for the newly uploaded Application (.EXE) files. If any new applications fails the CRC check, that file will not be updated) 4) Analyze the associated metadata and retrieve version information. 5) Compare the uploaded File Version information with the File Version information associated with the currently and correspondingly installed components. If the uploaded version(s) are newer, then set flag(s) indicating the necessity to install the specific elements. 6) Determine if there is a new version of LAUNCHER.EXE Application. If so, exit the application while initiating a script to MOVE the updated version to the appropriate location on the HDD and when complete reload the LAUNCHER.EXE application. 7) Clear the LAUNCHER.EXE update flag. 8) MOVE (Update) any remaining flagged Application (.EXE) files from the upload directory of the HDD to its respective installation directory on the CNCS HDD. 9) Exit LAUNCHER.EXE with a parameter to load: CONTENTUPDATE.EXE (Fun-REQ-CNCS Content Update-Initial-01) |

- Initialize the CNCS-Main program component

| ID | Fun-REQ-CNCS Main-Initial-01 |  |
| --- | --- | --- |
| Function | Initialize CNCS-MAIN |  |
| Description | This function is utilized to initialize necessary CNCS operating parameters, load and install drivers and communication protocol, and launch all of the various ongoing CNCS component services. - Load the default CNCS operating parameters from the onboard nonvolatile memory. - Establish an RS-485 Car Line gateway to the User Datagram Protocol (UDP). a) Load and install the required RS-485 Car Line drivers and communication protocol. b) Load the device specific RS-485 Car Line ID. c) Open an RS-485 Car Line port. d) Load the RS-485 device ID table and all relevant packet protocol definitions. 3) Open a UDP Server Port 4) Load and start all CNCS services a) Execute Primary Media Player S/W b) Execute Secondary Media Player S/W c) Execute FTP Data S/W d) Execute MDS Data S/W e) Set Program monitoring timer ON |  |

- Monitor CNCS S/W Sub-component Services

| ID | Fun-REQ-CNCS Main-Monitoring-01 |
| --- | --- |
| Function | Sub S/W Monitoring |
| Description | This function monitors the various CNCS S/W component services and evaluates the current operational status of each. 1) Initiate a “Request for Status” update from each S/W component at intervals less than 2 seconds. 2) If a specific S/W component does not respond within 5 minute period, then force a conclusion of corresponding S/W process and restart the component. |

- Monitors, Reads and Transmits RS-485 Car Line data

| ID | Fun-REQ-CNCS Main-RS485-01 |
| --- | --- |
| Function | Receives & Transmits RS-485-data |
| Description | This function is used by the CNCS to monitor the RS-485 Car Line activity and enable the required responses. This function performs the following functions: 1) Load RS-485 protocol definitions 2) Monitor the RS-485 port data and compare the received data to the protocol definitions to determine the nature of the data received. 3) Evaluate the specific command sequences specified 4) Assimilate the appropriate controls, protocols, and processes necessary to affect the desired actions. 5) Serve as a gateway to the UDP protocol for delivery of commands to the various sub-component services. |

- Primary Player Initialization

| ID | Fun-REQ-CNCS PP-Initial-01 |
| --- | --- |
| Function | Initialization |
| Description | Upon the execution of the Primary Player S/W by the main control S/W, this function performs a number of required initialization steps. 1) Open a UDP port. 2) Turn on a timer to indicate that the program is working correctly. 3) Load default parameters 4) Initialize Content Loop functionality. |

- Play CNCS Primary Player Loop Content

| ID | Fun-REQ-CNCS PP-ContentPlay-01 |
| --- | --- |
| Function | Play Video Loop Content |
| Description | The PP S/W plays Advertisements, Public service announcements, or Way finding content in the order specified within the specified loop play list. 1) After loading the content schedule, play the first content file indicated on the schedule. 2) Advertisement content plays at a full size (1280x768) of the LCD as default. 3) When a content is played by the Secondary Player application, the Primary Player will play its content in 1280x668 in the upper zone of the LCD. 4) After completing Play of the first content element, it searches the schedule and plays the next element in the play list if available. 5) It repeats the play list from the beginning once all files registered in the schedule have been played. 6) The content Play Parameters, (including the content file name) are provided by the “Playlist.ini”. 7) Content played via this function will be interrupted in the event that a scheduled AVL trigger is encountered for train service announcements. The loop will resume with the subsequent content element upon completion of the triggered event. |

- Play CNCS Primary Player AVL triggered Content

| ID | Fun-REQ-CNCS PP-RouteInformationPlay-01 |
| --- | --- |
| Function | Play AVL route guidance & triggered content |
| Description | Play route guidance content followed by train operating information which has been indicated in parameters provided by the Main S/W via UDP communication. 1) Analyze parameters (START, APPROACH, ARRIVAL, DOOR, STATION #, and NEXT STATION) received via UDP communication. 2) Stop playing current advertisement content and play the content file specified by the control parameters for the specified station number. 3) Play route guidance content in full screen mode of the LCD. 4) Continue playing the advertisement content file in the content schedule after finishing the route guidance content play. 5) The content Play Parameters are provided by the Main S/W via UDP communication after the Main S/W retrieves the content play parameters from the PAC. |

- CNCS Secondary Player Initialization

| ID | Fun-REQ-CNCS SP-Initial-01 |
| --- | --- |
| Function | Initialization |
| Description | Upon execution of the Secondary Player S/W by the main control S/W, this function performs a number of required initialization steps. 1) Open a UDP port. 2) Load default parameters. 3) Set scrolling area definitions. |

- Plays CNCS-SP AVL triggered Content

| ID | Fun-REQ-CNCS SP-ContentPlay-01 |
| --- | --- |
| Function | Plays AVL Triggered Content |
| Description | Plays the specified AVL triggered content file. 1) Analyze parameters (START, APPROACH, ARRIVAL, DOOR, STATION #, and NEXT STATION) received via UDP communication. 3) Play text-based AVL content in the lower portion (1280x100 from the bottom of the LCD). 3) If the content length fits in the LCD screen, the content will be displayed statically. 4) If the content length is longer than the LCD screen size, the content will be scrolled from the right to the left. |

- Initialize the MDS Data transfer process.

| ID | Fun-REQ-CNCS MDS Data-Initial-01 |
| --- | --- |
| Function | Initializing |
| Description | Executes the following actions to initialize the MDS Data transfer component: 1) Opens the RS-232 port 2) Opens the specifically reserved UDP server port. |

- MDS Data Transfer S/W

| ID | Fun-REQ-CNCS MDS Data-RS232-01 |
| --- | --- |
| Function | Retrieve MDS data packet |
| Description | This function retrieves MDS Data from the MDS via the LIC using the RS-232 interface and stores the data to the CNCS HDD. When a specified station code or GPS coordinate is detected, the MDS Data requests to MDS to transmit its data file, and then retrieves it. If approaching a YARD: 1) Retrieve “historical” data from the MDS 2) Store the MDS data into the CNCS HDD (file name = [Car Number]_yyyymmddhhmmss[MDS ID].bin) If approaching STATION: 1) Retrieve “recent” data from the MDS 2) Store the MDS data into the CNCS HDD (file name = [Car Number]_yyyymmddhhmmss[MDS ID].bin) |

- Send Time Information

| ID | Fun-REQ-CNCS MDS Data-RS232-02 |
| --- | --- |
| Function | Transmit the time information to the MDS via LIC |
| Description | 1) MDS Data monitors the time information of CNCS. 2) Receives a request for time information from LIC 3) Transmits the current time information to LIC |

- Send CS Software Versions

| ID | Fun-REQ-CNCS MDS Data-RS232-03 |
| --- | --- |
| Function | Transmit the CS subcomponent S/W version information to the MDS via LIC |
| Description | 1) MDS Data collects and monitors the CS subcomponents SW version information from each CS subcomponents. 2) Receives a request for SW versions from the LIC. 3) Transmits the SW versions to the LIC. |

- CNCS-FTP Initialization Process

| ID | Fun-REQ-CNCS FTP-Initial-01 |
| --- | --- |
| Function | Initializes the FTP protocol and sets operating parameters for communication with the Wayside servers. |
| Description | The FTP-Initial-01 function is executed immediately upon launching the FTP S/W component. It must perform the following tasks: 1) Opens a predefined UDP server port. 2) Turns on a Ping timer to specify how often the on-board system Pings the Wayside network for a response. a) If current location is “Union Station” or a “Yard”, then set the timer parameter value to indicate 5 second intervals. 4) Launch the Wayside Timer Fun-REQ-CNCS FTP-Wayside-01 |

- FTP-Wayside Program Timer

| ID | Fun-REQ-CNCS FTP-Wayside-01 |
| --- | --- |
| Function | The FTP-Wayside Timer carries out a ping test against the wayside periodically to determine the connection status and/or availability. |
| Description | The Wayside timer attempts to establish communication with the Wayside network by perform a Ping test at parameter defined intervals. 1) Opens a defined UDP port to the CNCS Main application. 2) Set the timer interval based upon a parameter setting 3) Turn on the Wayside timer a) It performs a ping test to the Wayside Network each connection interval to attempt to solicit a response from the Wayside. b) If a valid ping response is received, it tries to connect to the wayside. c) When a valid connection is established it calls the MDS Data Upload function and Content Data Download function. d) If a valid ping response is not received, it retries at the next time interval. |

- FTP-MDS Data Upload

| ID | Fun- REQ-CNCS FTP-Update-01 |
| --- | --- |
| Function | Upload MDS Data files from the CNCS to the Wayside |
| Description | If a train is located in either Station or Yard, it may upload MDS data (error information) as soon as it connects to the Wayside Network. 1) If the next station code is “Union Station” or the GPS coordinate indicates “?? Yard”, then the Wayside timer interval is set to 5 seconds so that the train will connect to the Wayside Network ASAP and establish an FTP link. 2) It uploads any MDS data files which are saved on the local CNCS HDD to the Wayside MDS Server via FTP as soon as it has successfully established an FTP session and identified the host. |

- FTP-Content Data Download

| ID | Fun-REQ-CNCS FTP-Update-02 |
| --- | --- |
| Function | Download Content Data files from the Wayside to the CNCS |
| Description | If the Wayside Network connection is valid, carry out the functions listed below: 1) Turn the Wayside Timer off 2) Download various content elements from the wayside. 3) Store each content element on the CNCS HDD in the specified directory location once the download has been successfully completed. 4) If there is no content available or necessary to download from the Wayside, or if the Wayside Network connection is lost, then the FTP session will be terminated and the Wayside Timer will be set to the “On” state. |

- CNCS Content Update

| ID | Fun-REQ-CNCS ContentUpdate-Initial-01 |
| --- | --- |
| Function | Initializes the Content Update functional component |
| Description | Upon execution of Content update S/W component, it performs a number initialization processes. 1) Load default operating parameters 2) Opens a UDP port to the Main application 3) Loads set value of RS-485 4) Opens RS-485 port |

- Transfer PRIMARY Authority regarding RS-485 Communication from the PAC to the CNCS

| ID | Fun-REQ-CNCS ContentUpdate-RS485-01 |
| --- | --- |
| Function | Set / Define RS-485 data Transmit/Receive Mode |
| Description | In order to effective facilitate transferring large amounts of data as required by the Content Update function, it is necessary to temporarily pass RS-485 Car Line control from the Primary PAC to the CNCS. At the completion of the Content Update process control is returned to the PAC for normal operations. 1) Request primary authority transfer from PAC to CNCS a) When the Primary PAC requests a status update from the CNCS, the CNCS will respond with a request for authority transfer so that it can manage the RS-485 lines to perform the Content updates. b) Once the Master PAC has granted the CNCS master authority over the RS-485 Car Lines, then this function will call the “Content Update” function. 2) Received data packet passes checksum analysis 3) Upon return from the Data Update function, RS-485 control is passed back to the Primary PAC. |

- Content Update Call Command:

| ID | Fun-REQ-CNCS ContentUpdate-Update-01 |
| --- | --- |
| Function | Calls the “Content Update” function with the appropriate command parameters |
| Description | Calls the Content Update S/W with different parameters in order to select the appropriate update function based on the specific devices and type of content being updated. This function updates content for many of the CS sub-systems including the following: 1) Call the “CCP-Update” function if a new version of the train operating schedule has been downloaded to the “Update” directory of the CNCS HDD. 2) Call the “MP3-Update” function if new AMF files have been downloaded to the “Update” directory of the CNCS HDD. 3) Call the “LED-Update” new LED HEX Files have been downloaded to the “Update” directory of the CNCS HDD. 4) After all updates have been completed, return primary control of the RS-485 Car Line back over to the Primary PAC. 5) Close the RS-485 port. 6) Close the Content Update S/W and return to the Main Control S/W upon completion of the update process. |

- Update Train Schedule Database

| ID | Fun-REQ-CNCS ContentUpdate-Update-02 |
| --- | --- |
| Function | CCP-Update function |
| Description | This function Updates the Master Train Schedule components in the CCP as necessary. 1) Transfer ready command indicating update to the CCP is pending and will be performed by the CNCS in bulk data mode. 2) Confirm status for ready to update in each CCP. 3) Transmit updated Train Operating Schedule data. 4) Transmit updated GPS Trigger Point data. 5) Transmit updated Station Naming data. 6) Transmit updated Route Naming data. 7) Transmit updated Holiday Schedule data. 8) Validate the data transferred to the CCP after the Data packet transfer process has completed. 9) Return to the Content Update function regardless of the completion and validity status. |

- Update automated PA message file to SD memory in PAC via USB interface.

| ID | Fun-REQ-CNCS ContentUpdate-Update-03 |
| --- | --- |
| Function | MP3_Update function |
| Description | Update the AMF file library via USB communication to the PAC’s MP3 Player USB memory located in CRA. 1) Transfer ready command for USB update in PAC. 2) The PAC which receives the ready request command responds with confirmation of its ready status. 3) The CNCS transfers the AMF files from the CNCS HDD to the USB memory module. 4) Check the update status to ensure proper completion of the process. 5) Send an “Update Complete” command to the PAC. 6) The PAC disconnects the USB connection between the CNCS and the PAC’s USB memory. 7) This function then returns to the “Content Update” function . |

- Update LED signs with LED statement data using RS-485 communication.

| ID | Fun-REQ-CNCS ContentUpdate-Update-04 |
| --- | --- |
| Function | Updates LED Content Data using RS-485 |
| Description | Updates both the image based and text based LED content elements 1) LED Content update is carried out in order of FDI, SDI and PII. 2) Send a “Get Ready” command to each sign in the specified category, and record their “Ready” responses. 3) Transfer the new LED Content data to the LED signs by page(256 byte) 4) If communication errors have occurred, then repeat the process up to two times for each of the specific signs which indicated an erroneous transfer. 5) After all signs in the category have been updated successfully, then go on to the next category and repeat the process. 6) This function then returns to the “Content Update” function for completion of the LED display update process. |

FDI

- FDI Initialization Process

| ID | Fun-REQ-FDI-Initial-01 |
| --- | --- |
| Function | Initialize the FDI main board |
| Description | 1) Initialize the FDI operating S/W. 2) Set the FDI RS-485 DEVICE_ID after checking the Address Signal. 3) Open RS-485 communication port |

- Respond to FDI Status Request

| ID | Fun-REQ-FDI-Comm-01 |
| --- | --- |
| Function | Responds to requests for status data from the PAC indicating the operational status of the FDI. |
| Description | Responds to the Primary PAC’s request for status update via the Car Line RS-485 by sending the current status data of the device. The Master PAC then polls each Primary PAC and consolidates the status data in the CCP for review. |

- Display FDI Content

| ID | Fun-REQ-FDI-Display-01 |
| --- | --- |
| Function | Display the appropriate information on the FDI as indicated by the current train operating parameters. |
| Description | 1) Receives the Train Route code & related information from the PAC 2) Load LED Image Data relative to the station, Route Number, and Destination Content into internal memory 3) Display image data from internal memory |

- FDI Content Update Management

| ID | Fun-REQ-FDI-Update-01 |
| --- | --- |
| Function | Receives LED Image Data from CNCS and updates the content library stored in local FDI non-volatile memory. Does not delete previous content until FDI is completely updated. |
| Description | 1) Receive a command to update LED Image data from CNCS 2) Switch to content update mode 3) Receive LED Image data packets from CNCS via RS-485 4) Store received data in local FDI memory 5) Validate integrity of transferred content elements 6) Delete replaced and/or expired content elements 7) Return to normal operating mode |

SDI

- SDI Initialization Process.

| ID | Fun-REQ-SDI-Initial-01 |
| --- | --- |
| Function | Initialize the SDI main board |
| Description | 1) Initialize the SDI operating S/W. 2) Set SDI RS-485 DEVICE_ID after checking the Address Signal. 3) Open the RS-485 communication port |

- Respond to Request for SDI status data

| ID | Fun-REQ-SDI-Comm-01 |
| --- | --- |
| Function | Responds to requests for status data from the PAC indicating the operational status of the SDI. |
| Description | Responds to the Primary PAC’s request for status update via the Car Line RS-485 by sending the current status data of the device. The Master PAC then polls each Primary PAC and consolidates the status data in the CCP for review. |

- Display SDI Content

| ID | Fun-REQ-SDI-Display-01 |
| --- | --- |
| Function | Display the appropriate information on the SDI as indicated by the current train operating parameters. |
| Description | 1) Receives the Train Route code & related information from the PAC 2) Load LED Image Data relative to the station, Route Number and Destination Content into internal memory. 3) Display image data from internal memory |

- SDI Content Update Management

| ID | Fun-REQ-SDI-Update-01 |
| --- | --- |
| Function | Receives LED Image Data from CNCS and updates the content library stored in local SDI non-volatile memory. Does not delete previous content until SDI is completely updated. |
| Description | 1) Receive a command to update LED Image data from CNCS 2) Switch to content update mode 3) Receive LED Image data packets from CNCS via RS-485 4) Store received data in local SDI memory 5) Validate integrity of transferred content elements 6) Delete replaced and/or expired content elements 7) Return to normal operating mode |

PII

1) PII Board Initialization Process

| ID | Fun-REQ-PII-Initial-01 |
| --- | --- |
| Function | Initialize PII main board |
| Description | 1) Initialize the PII operating S/W. 2) Set PII RS-485 DEVICE_ID after checking the Address Signal. 3) Open RS-485 communication port |

2) Response to PII Status Request

| ID | Fun-REQ-PII-Comm-01 |
| --- | --- |
| Function | Responds to requests for status data from the PAC indicating the operational status of the PII. |
| Description | Responds to the Primary PAC’s request for status update via the Car Line RS-485 by sending the current status data of the device. The Master PAC then polls each Primary PAC and consolidates the status data in the CCP for review. |

3) Display AVL Station Arrival and Departure Messages

| ID | Fun-REQ-PII-Display-01 |
| --- | --- |
| Function | Display text font based AVL Station Arrival and Departure messages. |
| Description | This function provides for the ability to display the standard AVL arrival and departure messages, (BMP/HEX Based), which are designated in the master route schedule for the currently selected train number. 1) When a trigger point is discovered by function Fun-REQ-PII-Comm-01 it initiates a start command including parameters which specify the specific content element to be displayed, the method of display, and the duration. 2) Execute the content play command by loading the specified content data from the PII non-volatile memory into the display memory area. 3) Maintain the displayed image for the specified period of time, or until terminated by a subsequent play command or cancel command. 4) Exit the routine |

4) PII Content Update management

| ID | Fun-REQ-PII-Update-01 |
| --- | --- |
| Function | Receives LED Image Data from CNCS and updates the content library stored in local PII non-volatile memory. Does not delete previous content until PII is completely updated. |
| Description | 1) Receive a command to update LED Image data from CNCS 2) Switch to content update mode 3) Receive LED Image data packets from CNCS via RS-485 4) Store received data in local PII memory 5) Validate integrity of transferred content elements 6) Delete replaced and/or expired content elements 7) Return to normal operating mode |

LIC

- LIC Device Initialization Process

| ID | Fun-REQ-LIC MAIN-Initial-01 |
| --- | --- |
| Function | Initialize the LIC Board at Power-On |
| Description | 1) Initialize processor chip after power-on 2) Set LIC_ID after checking the LIC_ID signal. 3) Check LIC-LON S/W status. |

- Retrieves device fault data from the MDS via the LIC and transmits it to the CNCS.

| ID | Fun-REQ-LIC MAIN -Comm-01 |
| --- | --- |
| Function | Receives device fault data from the LIC and writes it to a file on the CNCS |
| Description | This function retrieves MDS Data from the MDS via the LonWorks interface and forwards the data stream to the CNCS-MDS S/W component via the RS-232 interface. The specific data retrieved will depend upon the location of the vehicle as determined by the CCP route processor. If however the vehicle is at a “YARD” then all of the available MDS data must be retrieved. The appropriate trigger data to accomplish this will be forwarded by the CCP to the CNCS main S/W via the RS-485 Car Line. The data will then be incorporated into operating parameters and sent to the CNCS-MDS Data Transfer S/W component via UDP. The CNCS-MDS S/W component will then issue an appropriate request for response to this LIC S/W component. 1) Receives a request for train fault data from CNCS 2) Requests either “Recent” or “Historical” fault data via LonWorks Communication provided by the LIC using the LonWorks S/W component 3) Receives fault data stream from the MDS and transmits it in real-time to the CNCNS using the RS-232 communications channel. |

- Receives CS fault data for each sub system from the PAC and transfers it via the LIC to the MDS.

| ID | Fun-REQ-LIC MAIN -Comm-02 |
| --- | --- |
| Function | The LIC Main Control S/W transfers device fault data received from the PAC via RS-485 to the LIC and then forwards it to the MDS. |
| Description | 1) The LIC Main Control S/W monitors the CS fault data from PAC. 2) MDS requests for the CS fault data to LIC Main Control S/W via the LonWorks interface. 3) LIC Main Control S/W transmits the current CS fault status to MDS device via LonWorks interface. |

- Send LIC Status Data to the PAC.

| ID | Fun-REQ-LIC MAIN -Comm-03 |
| --- | --- |
| Function | Transmit the LIC operational status data to the PAC |
| Description | 1) Receives a request for LIC status from the PAC. 2) Transmits the LIC’s current operational status to the PAC. |

- Send Time Information to the MDS

| ID | Fun-REQ-LIC MAIN -Comm-04 |
| --- | --- |
| Function | Transmit the time information to the MDS |
| Description | 1) LIC-MAIN monitors the current time information provided by CNCS MDS Data. 2) Receives a request for time information from the MDS. 3) Transmits the current time information to the MDS. |

- Send CS Software Versions to the MDS

| ID | Fun-REQ-LIC MAIN -Comm-05 |
| --- | --- |
| Function | Transmit the CS subcomponent S/W version information to the MDS |
| Description | 1) LIC-MAIN monitors the CS subcomponents SW version information provided by CNCS MDSData application. 2) Receives a request for SW versions from the MDS. 3) Transmits the SW versions to the MDS. |

- Device Initialization

| ID | Fun-REQ-LIC LON-Initiali-01 |
| --- | --- |
| Function | Initializing |
| Description | 1) Initialize processor chip upon power-on. 2) Send LIC Status to Lon Network. |

- Provide a Gateway between (Convert Data Format and Protocol) between the LonWorks Protocol and the LIC Main Control S/W for transmitting & receiving data.

| ID | Fun-REQ-LIC LON-Comm-01 |
| --- | --- |
| Function | This S/W Component (LIC LonWorks) functions in conjunction with the previously defined S/W Component (LIC LonworksMain Control) to provide for a Gateway/Bridge between the LonWorks data network and the CS device protocols. |
| Description | 1) Convert data from LonWorks Protocol and write to internal memory in a format that can be read via the LIC Main S/W and then forwarded to the CS Devices via either RS-232 or RS-485. 2) These two S/W components work bi-directionally |

Performance Requirements

- VTX

H/W Circuit of VTC should receive VTX_ID Address Signal and set its own ID at the same time of a power permission. It should not impact on RS-485 Interface when ID is not set.

Check if the video signal received from CNCS is existent and respond the status of H/W via PAC. The signal received by VTC in PAC-MA should be bypassed if video signals cannot be transmitted to VRX due to device failure or no video signal input to VTX.

- VRX

H/W Circuit of VRX should receive VRX_ID Address Signal and set its own ID at the same time of a power permission. It should not impact on RS-485 Interface when ID is not set.

Check if the video signal received from VTX or VRX is existent and respond the status of H/W via PAC. The signal received should be bypassed if video signals cannot be transmitted to VRX due to the device failure of VRX.

- FDI

FDI should receive FDI_ID Address Signal and set its own ID at the same time of a power permission. It should not impact on RS-485 Interface when ID is not set. It should response the status via PAC after ID setting. The image to be displayed on LED module should be stored in a specific image memory. It should be applicable after the update is completed when the image update via CNCS. The image of the former version should be normally displayed when the update is not completed.

- SDI

SDI should receive SDI_ID Address Signal and set its own ID at the same time of a power permission. It should not impact on RS-485 Interface when ID is not set. It should response the status via PAC after ID setting. The image to be displayed on LED module should be stored in a specific image memory. It should be applicable after the update is completed when the image update via CNCS. The image of the former version should be normally displayed when the update is not completed.

- PII

PII should receive PII_ID Address Signal and set its own ID at the same time of a power permission. It should not impact on RS-485 Interface when ID is not set. It should response the status via PAC after ID setting. The image to be displayed on LED module should be stored in a specific image memory. It should be applicable after the update is completed when the image update via CNCS. The image of the former version should be normally displayed when the update is not completed.

- PAC

PAC is the main device which manages the RS-485 communication within the Communication system. Every device which participates in the communication should not hold the RS-485 Enable on the communication line by malfunction. Train will be separated physically into 2 by the married car composition. PAC shows malfunction in the announcement, communication, and vehicle consist information collection, if the RS-485 which connects between the vehicle and another vehicle, and trainline RS-485 gets disconnected.

- DPO

DPO receives tone signal and delivers command with the tone signal. All DPO, DPH, VOIP, and PEI of vehicle which has been double-head connected should be connected on the signal line which delivers tone. The ID of the DPO should be distinguished with other DPOs in the same vehicle, and accurate operation is available when signal output address line has been connected to be distinguished.

- DPH

DPH receives tone signal and delivers command with the tone signal. All DPO, DPH, VOIP, and PEI of vehicle which has been double-head connected should be connected on the signal line which delivers tone. The ID of the DPH should be distinguished with other DPHs in the same vehicle, and accurate operation is available when signal output address line has been connected to be distinguished.

- PEI

PEI receives tone signal and delivers command with the tone signal. All DPO, DPH, VOIP, and PEI of vehicle which has been double-head connected should be connected on the signal line which delivers tone. The ID of the PEI should be distinguished with other PEIs in the same vehicle, and accurate operation is available when signal output address line has been connected to be distinguished. PEIs should be set not to occur howling as the PEI locates close to microphone and speaker.

Logical Database Requirements

None.

Design Constraints

General Design Constraints

- The amount of memory capacity used by the installed S/W shouldn’t be over 80% of the entire memory capacity.

- 32bit STR7 Chip is used for Microprocessor.

- The application S/W operated on CNCS is automatically updated by wireless network.

- Windows XP Embedded based Application S/W shall be automatically implemented when Operating System is booted.

- The Destination Sign System shall utilize Global Positioning System (GPS) technology to provide location information.

- Route display system should use GPS to provide location information.

- Communication System should be completely independent of communication systems of DEVER traffic system (bus/LRV/)

- When Communication System does not used, the sounds should be muted.

- PID should display 20 characters by 4 rows simultaneously, the height of character should be about 2 inches, and PID should display graphics and images.

- The updates of location database and system software should be carried out using RS-232 serial port and IBM compatible portable computer.

- The GPS obtained time SHALL be used for all system clocks on the train in UTC

Standard Compliance

Operational manual for end user shall follow the guide-line of IEEE STD 1063. - S/W Source code shall comply with industry standard procedures .

Software System Attributes

Reliability

- The sub S/W in the CNCS is monitored every 1second by Main control S/W. If any particular sub S/W does not respond for 5 second, it shall be re-operated after the relevant S/W is finished by Main control S/W.

- Each device which has built in MCU uses Watch Dog function in the Chip to recover it by resetting MCU when device operated abnormally.

- PAC reduces probability of malfunction by using dual channel to get over outside noise when receiving intercom control signal.

Availability

- PAC is the most important device for Communication System and it is dualized as CRA.  If Master PAC is not operating for 3 seconds, it should automatically operate slave PAC as Master

Security

- On-Board CS Software can only be accessed or updated via a) Password protected PTU, or b) secure automated Wayside updates controlled by DENVER RTD and incorporating version control techniques.

Maintainability

- For the effective maintenance of source code, internally defined S/W coding guide shall be followed and comment display rate by module shall be over 50%.

Portability

- Application S/W operated on CNCS is portable to Windows XP OS.