DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #7: HARDWARE MODIFICATION

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2016-0027 |
| REVISION: | 1.1 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 9/08/2016 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 6/29/2016 | Initial release | D. S. Jang |
| 1.1 | 9/08/2016 | Add the section 3.0 | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Audio tuning circuit description	4

4. Instructions	6

5. Testing	9

Introduction

This FMI provides instructions to install the Audio Filter board to CRA

Summary

This modification intends to implement the engineering change proposed as RNE1002- U518-11. The modification applies to CRA units installed on the whole fleet.

Audio tuning circuit description

The input-output impedance of the input audio signal and the noise signal changed from HI to LO. And the normal audio signal amplification and the noise signal is decreased -10dBm by configured B.P.B (Band-Pass Filter) circuit.

Figure 1 – AF Board block diagram

Figure 2 – AF Board schematics

| Description | First Improvement (End of June) | Last Improvement (End of August) | Remarks |
| --- | --- | --- | --- |
| A(R1) | 1K Ω 0 Ω | Same as First Improvement | Resistor |
| A(R2) | 1K Ω 0 Ω | Same as First Improvement | Resistor |
| B(R4) | - | 10 Ω 47K Ω | Resistor |
| B(C3) | - | 224P 102P | capacitor |
| C(R18) | - | 10 Ω 47K Ω | Resistor |
| D(R26) | - | NOP 8.2 Ω | Resistor |
| D(R27) | - | 82K Ω 0 Ω | Resistor |

- A : The audio input impedance change.

- When R1, R2 component changed (1kΩ ->0Ω), the line impedance changed from 1k Ω to 0Ω and remove the derelict noise signal on the transformer (TRF – T1 600:8).

- B : Noise remove.

- The noise signal is filtered by changing the input resistance (10Ω -> 47KΩ)   of R4 component due that the input audio signal includes the noise.

- C : The audio frequency change (B.P.B)

- The audio frequency from 4 KHz to 5 KHz is partially blocked by changing R18 component.

- D : Output impedance change.

- The impedance changed from HI to LO. The noise is blocked and the audio output signal is filtered by changing the R27 component.

Instructions

1) Place the COMCB to OFF position

2)Unscrew 2 mounting bolts on the SP panel

3) Remove the SP panel from CRA

4) Plug in the Audio Filter (AF) board to the slot where the SP panel was removed from

5) Fasten 2 mounting bolts on the AF front panel

6) Place the COMCB to ON position

Figure 3 – AF Board Location

Figure 4 – AF Board PCB

Figure 5 – CRA Location

Testing

1) Observe the LED indicators on the AF front panel

2) Verify that all LED indicators are lit

. Figure 6 – AF PCB LED.

3) Download the recorded audio files from CRA

4) Check the audio quality whether the recorded audio file can be recognized or not

(Recognized: Pass, Un-recognized: Failed)