DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #7: HARDWARE MODIFICATION

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2016-0027 |
| REVISION: | 1.1 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 9/08/2016 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 6/29/2016 | Initial release | D. S. Jang |
| 1.1 | 9/08/2016 | Section 3.0 추가. | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. 오디오 튜닝 회로 설명.	4

4. Instructions	6

5. Testing	9

Introduction

This FMI provides instructions to install the Audio Filter board to CRA

Summary

This modification intends to implement the engineering change proposed as RNE1002- U518-11. The modification applies to CRA units installed on the whole fleet.

오디오 튜닝 회로 설명.

입력되는 오디오 신호와 노이즈 신호의 입력-출력 임피던스를 HI -> LO 변경 하고, B.P.B(Band-Pass Filter) 회로 구성하여 출력되는 정상 오디오 신호 증폭 및 노이즈 신호를 -10dBm 감쇄 시킨다.

Figure 1 – AF Board 블록다이어그램

Figure 2 – AF Board schematics

| Description | First Improvement (End of June) | Last Improvement (End of August) | Remarks |
| --- | --- | --- | --- |
| A(R1) | 1K Ω 0 Ω | 1K Ω 0 Ω | Resistor |
| A(R2) | 1K Ω 0 Ω | 1K Ω 0 Ω | Resistor |
| B(R4) | - | 10 Ω 47K Ω | Resistor |
| B(C3) | - | 224P 102P | capacitor |
| C(R18) | - | 10 Ω 47K Ω | Resistor |
| D(R26) | - | NOP 8.2 Ω | Resistor |
| D(R27) | - | 82K Ω 0 Ω | Resistor |

- A : 오디오 입력 임피던스 변경.

- R1,R2 부품(1kΩ ->0Ω)을 변경하면, 라인 임피던스가 1k Ω ->0Ω 변경되고 인과 트랜스(TRF – T1 600:8)에 유기 되는 노이즈 신호를 1차로 제거한다.

- B : 노이즈 제거.

- 입력 되는 오디오 신호는 노이즈를 포함 하고 있어R4부품(10Ω -> 47KΩ)  입력 저항을 변경 하여 노이즈 신호를 필터한다.

- C : 오디오 주파수 변경.( B.P.B)

- R18 부품을 변경 하여 노이즈 주파수 4KHz~5KHz오디오 주파수를 일부 차단 한다.

- D :  출력 임피던스 변경.

- R27부픔을 변경을 통해, 임피던스(HI -> Lo) 변경하여 노이즈 차단 및 오디오 출력 신호를 필터.

Instructions

1) Place the COMCB to OFF position

2)Unscrew 2 mounting bolts on the SP panel

3) Remove the SP panel from CRA

4) Plug in the Audio Filter (AF) board to the slot where the SP panel was removed from

5) Fasten 2 mounting bolts on the AF front panel

6) Place the COMCB to ON position

Figure 3 – AF Board Location

Figure 4 – AF Board PCB

Figure 5 – CRA Location

Testing

1) Observe the LED indicators on the AF front panel

2) Verify that all LED indicators are lit

. Figure 6 – AF PCB LED.