MDS Fault Data Transmission System

- System requesting the fault status constantly

LIC requires fault data every 30 minutes

: Request MDS if there is any fault data every 30 minutes and MDS transmits the fault data to LIC.

The system of SEPTA is to request fault data once a day when train input after running. As the similar system of SEPTA, DENVER has changed the system to be requiring fault data every 30mins.

Part of the communication load can be occurred on Wimax network as the system of DENVER requires fault data every 30mins.

Advantages :

- The system has reliability in its function as it modifies and supplements the system verified by SEPTA.

Disadvantages :

- The fault data can be checked 30mins after fault occurrence.

(30 minutes means the time to request the fault data)

- The system load will be increased as the logic requiring faults every 30 minutes is added