Penta Information

1) Audio Streaming Transmission Method: RTP method

Woojin is fine with this method.

2) Audio Codec Method: G.711

 Woojin is fine with this method.

Woojin’s Questions

3) Interface Control Document: Calling each vehicle from the OCC, Operational procedure document. (Simple Flow Chart etc.)

Example)

- The distinguish method how to find out if it is the received data that I have to handle or is it other vehicle transmission data (Distinguish Method is very important as it is an UDP method.).

- Is there any response code required for the received data?

4) What method is used for the Text Message Transmission? (RTP method is for the audio transmission method. What do Penta use for the Text Message transmission?)

5) Interface Control Document: The text message transmission procedure for the OCC and each vehicle.

Example)

- The distinguish method how to find out if it is the received data that I have to handle or is it other vehicle transmission data

- Does it display as soon as the reception? Does it make any specific event display?

Avaya Information

6) The transmission method between PEI/DPO – OCC?

 Does Penta use RTP or SIP? (We used the SIP method for the previous test)

7) OCC – PEI/DPO Interface Control Document: Audio Communication Scenario

Example)

- The distinguish method how to find out each vehicle when calling the DPO/PEI from the OCC?

- The distinguish method how to find out the equipment which is calling during the communication between the OCC and DPO/ PEI?

- The distinguish method how to find out which PEI has called between the 2 PEIs in the vehicle?

8) Please define the priority order for each function.

Need to clarify the priority

- Which has the priority between Call from OCC to Onboard PEI and Call from OCC to Onboard Speakers (to All Passenger)?