Data Sheet for LIC

- TMS320VC33-150

  - Features   1페이지

High-Performance Floating-Point Digital Signal Processor (DSP):

− 13-ns Instruction Cycle Time

− 150 Million Floating-Point Operations

− 34K × 32-Bit (1.1-Mbit) On-Chip Words of Dual-Access Static Random-Access Memory (SRAM) Configured in 2 × 16K Plus 2 × 1K Blocks to Improve Internal Performance

− Very Low Power: < 200 mW @ 150 MFLOPS

32-Bit High-Performance CPU

  - Electrical characteristics   26페이지

- XC95288XLTQG144

2.1 Features   1페이지

• 6 ns pin-to-pin logic delays

• System frequency up to 208 MHz

• 288 macrocells with 6,400 usable gates

• Available in small footprint packages

- 144-pin TQFP (117 user I/O pins)

• Optimized for high-performance 3.3V systems

- Low power operation

- 5V tolerant I/O pins accept 5V, 3.3V, and 2.5V signals

- 3.3V or 2.5V output capability

- Advanced 0.35 micron feature size CMOS

FastFLASH™ technology

• Advanced system features

- In-system programmable

- Superior pin-locking and routability with FastCONNECT II™ switch matrix

- Extra wide 54-input Function Blocks

- Up to 90 product-terms per macrocell with individual product-term allocation

- Local clock inversion with three global and one product-term clocks

- Individual output enable per output pin with local inversion

- Input hysteresis on all user and boundary-scan pin inputs

- Bus-hold circuitry on all user pin inputs

- Full IEEE Standard 1149.1 boundary-scan (JTAG)

• Fast concurrent programming

• Slew rate control on individual outputs

• Enhanced data security features

• Excellent quality and reliability

- Endurance exceeding 10,000 program/erase cycles

- 20 year data retention

- ESD protection exceeding 2,000V

2.2 Electrical characteristics    3페이지

- FT5000

3.1 Features    1페이지

• 3.3V operation.

• Higher-performance Neuron® Core —internal system clock scales up to 80MHz.

• Substantial device price reduction.

• Serial memory interface for inexpensive external EEPROM and flash non-volatile memories.

• Supports up to 254 Network Variables (NVs) and 127 aliases.

• Low-cost surface mount FT-X3 Communications Transformer.

• User-programmable interrupts provide faster response time to external events.

• Includes hardware UART with 16-byte receive and transmit FIFOs.

• 7 mm x 7 mm 48-pin QFN package.

• Supports polarity-insensitive free topology star, daisy chain, bus, loop, or mixed topology wiring.

• Compliant with TP/FT-10 channels using FT 3120®/FT 3150® Smart Transceivers and FTT-10/FTT-10A/LPT-10/LPT-11 Transceivers.

• 12 I/O pins with 35 programmable standard I/O models.

• Supports up to 42KB of application code space.

• 64KB RAM (44KB user-accessible) and 16KB ROM on-chip memories.

• Unique 48-bit Neuron ID in every device for network installation and management.

• Very high common-mode noise immunity.

• -40°C to +85°C operating tempera-ture range.

  - Electrical characteristics   5페이지

- MAX3535E

  - Features    1페이지

♦2500VRMS RS-485 Bus Isolation Using On-Chip High-Voltage Capacitors

♦ 1000kbps Full-Duplex RS-485/RS-422 Communication

♦ +3V to +5.5V Power-Supply Voltage Range(MAX3535E)

♦ 1/8 Unit Receiver Load, Allowing 256 Devices on Bus

♦ ±15kV ESD Protection Using HBM

♦ Pin-Selectable Slew-Rate Limiting Controls EMI

♦ Hot-Swap-Protected Driver-Enable Input

♦ Undervoltage Lockout

♦ Isolation-Barrier Fault Detection

♦ Short-Circuit Protected

♦ Thermal Shutdown

♦ Open-Line and Shorted-Line Fail-Safe Receiver Inputs

  - Electrical characteristics  2페이지

- FT-X3    6페이지

FT-X3 is communications transformer that is designed to be used as pair with FT5000.

  - Pinout Diagram

  - Electrical connection schematic

- XR16L784    1페이지

XR16L784 is a quad Universal Asynchronous Receiver and Transmitter(UART).

  - Features

• 2.97V to 5.5V operation with 5V Tolerant Inputs

• 8-bit Intel or Motorola Data Bus Interface

• Single Open Drain Interrupt output for all 4 channels

• Global Interrupt Source Registers for all channels

• 5G (Fifth Generation) “Flat” Register Set

• Each UART is Independently Controlled with:

■ 16C550 Compatible Registers

■ 64-byte Transmit and Receive FIFOs

■ Transmit and Receive FIFO Level Counters

■ Programmable TX and RX FIFO Trigger Levels

■ Automatic RTS/CTS or DTR/DSR Flow Control

■ Selectable RTS Flow Control Hysteresis

■ Automatic Xon/Xoff Software Flow Control

■ Automatic RS485 Half-duplex Control Output with 16 Selectable Turn-around Delay

■ Infrared (IrDA 1.1) Data Encoder/Decoder

■ Programmable Data Rate with Prescaler

■ Up to 3.12 (16x) and 6.25 (8x) Mbps Data Rate

• A General Purpose 16-bit Timer/Counter

• Sleep Mode with Automatic Wake-up Indicator

• 64-pin LQFP Package (10x10x1.4 mm)

  - Electrical characteristics   41페이지