Communication System Control

Communication Control Panel

The Communication Control Panel (CCP) (Refer to Figure 1.55) is installed in each cab to provide the operator with an interface for control functionality and monitoring of the entire communications system within the train.  A built-in LCD touch screen allows the operator to view a summary of all communications system diagnostics, car location, schedule adherence, communications, passenger emergency activations, and other pertinent information.

Figure 1.55 Example of the CCP GUI Screen

Table 1.13 CCP GUI Controls

| Communication Control Panel–Controls |  |
| --- | --- |
| HOME | Navigates the CCP Graphic User Interface (GUI) to the home diagnostic screen. This screen provides the train operational status, such as train number, time, route (departure, actual and next station), device status, etc. |
| ROUTE | Navigates the CCP GUI to the train number entry screen. This screen allows the operator to enter the train number. |
| CREW | Navigates the CCP GUI to the crew ID entry screen. This screen allows the operator to enter CREW ID #. This function is not expected to be used in the Denver operation. |
| STOPS | Navigates the CCP GUI to the detailed route editing screen. This screen allows the operator to change the attributes for each station in the current route by the use of 4 buttons including: STOP, SKIP, EXPRESS, and LOCAL. |
| CONFIG | Navigates the CCP GUI to the system configuration sub menu screen. This screen allows the operator to configure attributes of the communications system, including speaker volume, LCD dimmer, and system reset. Many of these screens require a password for access. |
| CHECK | Navigates the CCP GUI to the detailed system status screen, which allows the operator to check the status of all RS-485 devices throughout the train. |
| PEI | Navigates the CCP GUI to the active PEI call screen, which allows the operator to see to where in the train an active PEI call was initiated from. Car # and PEI box ID are provided. |
| CM | The Custom Message (CM) screen allows the operator to send custom preset messages to the LED and LCD displays. |
| ? (HELP) | Navigates the CCP GUI to the sub-menu help screen. This sub-menu describes what each of the navigation buttons allows the operator to do. |
| Operational Info | Provides communication system device status and displays operating parameters information. |

Table 1.19 CCP GUI Controls (Cont.)

| Communication Control Panel–Controls |  |  |
| --- | --- | --- |
| Operational Info | Time display line. |
| Operational Info | Destination and train number display line. |  |
| Operational Info | Schedule adherence display line. |  |
| Operational Info | System alerts (PEI calls) and message display line. |  |

Passenger Emergency Intercom (PEI)

The Passenger Emergency Intercom (PEI) installed at each wheelchair location in each car interlocks with the PAC to allow passengers to communicate with the operator in the case of an emergency.  When a PEI is activated, the car number for the activated PEI is displayed on the CCP LCD screen.

Additionally, the automated massaging system is activated and generates an audio message indicating that, “A passenger needs assistance in Car # xxx“. (This announcement will be announced only in the cab area through the cab speaker for the driver)

The party-line principle will allow multiple passengers to communicate with the driver and/or crew members using separate PEI boxes throughout the train consist. PEI requests from multiple passengers will result in sequential automated emergency announcements, and will also add each subsequent person to the already existing Party Line session. In the event that an Automated Emergency Announcement is pre-empted by a higher priority manual announcement, the message will be restarted after the manual announcement has been completed.

3

2

1

6

5

4

Figure 1.56 Passenger Emergency Intercom (PEI)

Table 1.14 PEI Controls

| Passenger Emergency Intercom – Controls and Indicators |  |  |
| --- | --- | --- |
| 1 | Intercom speaker | This speaker is used for all passenger-to-operator emergency intercom communications. |
| 2 | Active LED Indicator | When the active indicator is red, the passenger is able to talk to the operator via the microphone. |
| 3 | POWER LED Indicator | When the power indicator is yellow, the PEI is receiving power. |
| 4 | OCC LED Indicator | When the OCC indicator is green, the passenger is able to talk to the OCC via the microphone. |
| 5 | Microphone | Internal microphone. |
| 6 | PEI call button | This call button initiates a passenger emergency call. This button will notify the operator that a passenger needs assistance. |

Operator Public Address (PA) Operation Box

The operator PA operation box (Refer to Figure 1.57) is installed in each cab and provides the operator with audio communication capability to multiple devices.  These devices include the other operator PA operation boxes, PEIs, the OCC, and the ability to deliver public announcements via either or both the internal and external speakers.  By de-pressing any one three speaker select buttons (INT, EXT and BOTH), the output audio of the cab handset or gooseneck microphones is routed to the specified speakers via the PA audio car wiring and the trainline.  All speaker select buttons are backlit during passive operation and are brightened when activated.  The brightness of the buttons is controlled via Pulse Width Modulation (PWM) circuitry, which manages all device lights in the cab with a single control.

The operator PA operation box also includes a speaker volume control knob, which provides a level range adjustment from 10% to 100% of the internal speaker output, a cancel button for deactivating PEI or intercom calls and an IC button for activating cab-to-cab intercom calls.

Figure 1.57 Operator PA Operation Box

Table 1.15 Operator PA Operation Box Controls

| Operator PA Operation Box – Controls |  |  |
| --- | --- | --- |
| 1 | Interior speaker PTT (INT) button | Manual PA announcements are routed only to the interior (passenger seating area) speakers. |
| 2 | Exterior speaker (EXT) PTT button | Manual PA announcements are routed only to the exterior speakers. |
| 3 | Both (INT/EXT) speaker PTT button | Manual PA announcements are routed to both the interior (passenger seating area) and exterior speakers. |
| 4 | Internal speaker | This speaker is used for all PEI and intercom communications. |
| 5 | OCC pushbutton | Used to voice communicate with the OCC (it is connected via WIMAX) by pushing the OCC pushbutton when the driver needed the communication with OCC. |
| 6 | Volume control knob | Used to adjust the volume of the integral speaker. |
| 7 | Cancel button | Cancels all communications on the IC and PEI trainlines. |
| 8 | PEI button | Answers PEI calls initiated by any PEI in the train. |
| 9 | IC button | Initiates and answers intercom communication calls to or from another cab. |
| 10 | Handset | Driver can make announcement by speaking through the handset |

Operator PA Handset

The operator PA handset is an auxiliary unit of PA operation box.  The operator PA handset is installed in the observer’s cab and is also interlocked with the PAC.  This helps to communicate with the driver and passenger through the PA operation box and via the PA through all interior and exterior speakers. It is an auxiliary unit of DPO and helps the driver can make announcement by speaking through the handset. Interior, exterior, and both announcement is available.

Figure 1.58 Operator PA Handset

Table 1.16 Operator PA Handset Controls

| Operator PA Handset–Controls |  |  |
| --- | --- | --- |
| 1 | Interior Speaker (INT button and handset PTT) | Manual PA announcements are routed only to the interior (passenger seating area) speakers. |
| 2 | Exterior Speaker (EXT button and handset PTT) | Manual PA announcements are routed only to the exterior speakers. |
| 3 | Both Speaker (button and handset PTT) | Manual PA announcements are routed to both the interior (passenger seating area) and exterior speakers. |
| 4 | Handset | Driver can make announcement by speaking through the handset |

Appendix 1. CCP_User_Interfaces_Rev.5

Appendix 2. 423-309_Rev.0_System Functional Description of Communications System(20120130)