DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #8: SOFTWARE UPDATE

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2016-0028 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/14/2016 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/14/2016 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Location of Equipment	4

3.2	Software Update	6

3.2.1	Updating CRA-MAIN	6

3.3	Test and Verification	6

3.4	Version Check	7

4. Tracking	7

Introduction

This FMI provides instructions to install the Communication System software rollback to the vehicles.

Summary

This FMI applies to following software configuration item:

- CRA-MAIN, version 1.0.04

Instructions

Location of Equipment

Following figures show the location of equipment to be updated.

Figure 1 – CNCS Board Location

Figure 2 – CRA Location

Software Update

Updating CRA-MAIN

- Plug in a USB flash drive and a wireless keyboard to the USB ports on the CNCS front panel.

- Terminate all running applications

- Copy the updated ‘Main.exe’ to CNCS directory: D:\Woojin\CNCS\

- Recycle COMCB to apply the update.

Test and Verification

After updating the CRA-Main software, follow the procedure below to verify the functionality:

- Key-on the cab

- Using the CCP touchscreen, enter route number 10 or 11

- Run the route simulation

- Verify the simulated announcements and displays function appropriately.

Version Check

Use the MDS Software Version menu to verify the rollback software versions.

Figure 3 – Version Check

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.