DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #11: RS485 BOARD REPLACEMENT

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2016-0031 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/27/2016 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/27/2016 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Unit Replacement	4

3.2	Test and Verification	6

4. Tracking	6

Introduction

This FMI provides instructions to replace the CRA-RS485 circuit boards on the fleet.

Summary

This field modification is a follow-up to the temporary relay replacement carried out on May 2016. To prevent the potential quality issues may have been produced due to the temporary on-PCB rework, Woojin plans to replace all RS485 circuit boards in complete PCB level.

The board replacement will be done by 10 PCB’s (=5 married pairs) at a time. The process will be repeated until the entire fleet is completed.

Instructions

Unit Replacement

1) Place the COMCB to OFF position

2) Unscrew 4 mounting bolts on the RS-485 front panel

3) Remove the RS-485 board from CRA

4) Plug in the New RS-485 board

5) Fasten 4 mounting bolts on the RS-485 front panel

6) Place the COMCB to ON position

Figure 1 – RS-485 Board Location

Figure 2 – CRA Location

Test and Verification

After updating the CRA-Main software, follow the procedure below to verify the functionality:

- Key-on the cab

- Using the CCP touchscreen, enter route number 10 or 11

- Run the route simulation

- Verify the simulated announcements and displays function appropriately.

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.