DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #11: RS485 BOARD REPLACEMENT

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2016-0031 |
| REVISION: | 1.1 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/27/2016 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/27/2016 | Initial release | D. S. Jang |
| 1.1 | 8/16/2016 | Revised according to the DTO Comments | D.S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Unit Replacement	4

3.2	Test and Verification	6

3.2.1	Pre-delivery Test at Woojin Facility	6

3.2.2	Post-modification Test at Denver CRMF	6

4. Tracking	6

Introduction

This FMI provides instructions to replace the CRA-RS485 circuit boards on the fleet.

Summary

This field modification is a follow-up to the temporary relay replacement carried out on May 2016. To prevent the potential quality issues may have been produced due to the temporary on-PCB rework, Woojin plans to replace all RS485 circuit boards in complete PCB level.

The board replacement will be done by 10 PCB’s (=5 married pairs) at a time. The process will be repeated until the entire fleet is completed.

Instructions

Unit Replacement

1) Place the COMCB to OFF position

2) Unscrew 4 mounting bolts on the RS-485 front panel

3) Remove the RS-485 board from CRA

4) Plug in the New RS-485 board

5) Fasten 4 mounting bolts on the RS-485 front panel

6) Place the COMCB to ON position

Figure 1 – RS-485 Board Location

Figure 2 – CRA Location

Test and Verification

Pre-delivery Test at Woojin Facility

To verify the RS-485 board functions, each individual RS485 PCB will undergo a carline and trainline tests at Woojin’s test lab before their delivery.

Summarized below shows the test items:

- Test for 24 hours

- There shall be no faulty data packets or waveform on the trainline communication channel for 24 hours

- There shall be no faulty data packets or waveform on the carline communication channgel for 24 hours

Post-modification Test at Denver CRMF

To verify the RS485 functions, following test shall be conducted:

- Make a 4 car train consist

- Key-on a cab

- Wait until the system inauguration completes

- Observe the CCP screen and verify all 4 cars are recognized

- Repeat test on the other end

Due to the complexity on the CRMF activities, Woojin foresees the fleet wide trainline test may be unrealistic.

If fleet wide trainline test cannot be conducted, the test maybe limited to a 2 cars test (single married pair) to verify the carline function only. The trainline test will be substituted by Woojin’s pre-delivery lab test.

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.