DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #11: RS485 BOARD REPLACEMENT

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2016-0031 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/27/2016 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/27/2016 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Unit Replacement	4

3.2	Test and Verification	6

4. Tracking	6

Introduction

This FMI provides instructions to replace the CRA-RS485 circuit boards on the fleet.

Summary

This field modification is a follow-up to the temporary relay replacement carried out on May 2016. To prevent the potential quality issues may have been produced due to the temporary on-PCB rework, Woojin plans to replace all RS485 circuit boards in complete PCB level.

The board replacement will be done by 10 PCB’s (=5 married pairs) at a time. The process will be repeated until the entire fleet is completed.

Instructions

Unit Replacement

1) Place the COMCB to OFF position

2) Unscrew 4 mounting bolts on the RS-485 front panel

3) Remove the RS-485 board from CRA

4) Plug in the New RS-485 board

5) Fasten 4 mounting bolts on the RS-485 front panel

6) Place the COMCB to ON position

Figure 1 – RS-485 Board Location

Figure 2 – CRA Location

Test and Verification

우진공장(중련 485 통신 에이징 TEST.)

신규 PCB의 정확한 중련485 통신확인을 위해, 각 PCB는 우진산전 공장에서 중련 TEST를 실시하며 다음 항목을 점검한다.

4량 중련 시험SET에서 24시간 통신 에이징 시험을 실시하면, 에이징 시험후 통신 데이타 및 통신 파형을 측정하여, 체크 시트로 관리 한다.

- 시리얼 번호.

- Car line 통신 데이타 확인 및 파형 측정.

- Train line 통신 데이타 확인 및 파형 측정.

덴버 CRMF

After updating the CRA-Main software, follow the procedure below to verify the functionality:

- Key-on the cab

- Using the CCP touchscreen, enter route number 10 or 11

- Run the route simulation

- Verify the simulated announcements and displays function appropriately.

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progess will be reported to DTP/H-Rotem in daily basis as the modification proceeds.