DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #11: HARDWARE MODIFICATION

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2016-xxxx |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/25/2016 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/25/2016 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	REPLACEMENT PARTS	4

3.2	Location of Equipment	5

4. Tracking	7

4.1	HARDWARE REVISON	7

Introduction

This FMI provides instructions to install the RS-485 board to CRA

Summary

This FMI 는 지난 5월에 현장에서 조치 완료된 RS-485 PCB에 대한 품질 확보를 위한 조치이다.

전편성에 대하여 10EA씩 순환 교체를 실시 할 것이다.

Instructions

1) Place the COMCB to OFF position

2) Unscrew 2 mounting bolts on the RS-485 panel

3) Remove the RS-485 panel from CRA

4) Plug in the New RS-485 board to the slot where the RS-485 panel was removed from

5) Fasten 2 mounting bolts on the RS-485 front panel

6) Place the COMCB to ON position

Figure 1 – RS-485 Board Location

Figure 2 – CRA Location

Test and Verification

After updating the CRA-Main software, follow the procedure below to verify the functionality:

- Key-on the cab

- Using the CCP touchscreen, enter route number 10 or 11

- Run the route simulation

- Verify the simulated announcements and displays function appropriately.

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.