DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

PTT BOX 
FUNCTIONAL DESCRIPTION

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2017-0015 |
| REVISION: | 1.0 |
| DESIGNED: | J. S. JEONG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 3/6/2017 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 3/6/2017 | Initial release | J. S. Jeong |

Table of Contents

1. Introduction	4

2. Abbreviations	4

3. General Description	5

4. Functional Description	5

5. IP Address	6

6. Technical Data	7

Introduction

This document provides technical information for the PTT box installed in the Operations Control Center (OCC) of RTD Commuter Rail.

Abbreviations

Table 1 – Abbreviations

| Abbreviation | Definition |
| --- | --- |
| CNCS | Content and Control Server |
| DPO | Driver PA Operation Box |
| DTMF | Dual Tone Multi Frequency |
| IP | Internet Protocol |
| OCC | Operations Control Center |
| PAC | Public Address Controller |
| PEI | Passenger Emergency Intercom |
| PTT | Push to Talk |
| RTP | Real-time Transport Protocol |
| SIP | Session Initiation Protocol |
| VoIP | Voice Over Internet Protocol |

General Description

The PTT box is designed to send the push-to-talk signal to the carborne communications system over the WiMax network. The carborne system utilizes this signal to toggle between ‘Talk’ and ‘Listen’ modes of the PEI.

Figure 1 shows the relation of Woojin PTT box and other system components.

Figure 1 – PTT Device Network Diagram

Functional Description

Below describes the overall OCC~PEI communications logic including the PTT control:

- PEI requests an emergency call

- No response from DPO for 13 seconds. This time is counted by PAC

- If PAC does not acknowledge DPO’s response by 13 seconds, PAC signals VoIP to call the specific phone number, i.e. #2000. PAC uses internal dry contact to signal VoIP

- OCC picks up the call

- Call session between OCC phone and on-board VoIP module is initiated

- CNCS and OCC PTT communicates in a different communication channel using custom protocol

- When OCC PTT is “active”:

  - PTT sends “PTT active” signal to CNCS over WiMax network

  - CNCS acknowledges “PTT active” and transfers the signal to RS485 serial link

  - PAC acknowledges “PTT active” via RS485. PAC converts this to analog DTMF signal and transfers to any activated PEI:

    - Activate PEI speaker

    - Deactivate PEI microphone

- When OCC PTT is “inactive”:

  - PTT sends “PTT inactive” signal to CNCS over WiMax network

  - CNCS acknowledges “PTT inactive” and transfers the signal to RS485 serial link

  - PAC acknowledges “PTT inactive” via RS485. PAC converts this to analog DTMF signal and transfers to any activated PEI:

    - Deactivate PEI speaker

    - Activate PEI microphone

- When the call session is terminated, VoIP module signals the PAC using dry contact

- PAC acknowledges the dry contact signal and clears the call session using DTMF signal (analog)

IP Address

Each PTT boxes are configured with unique IP addresses of:

10.151.1.200 – 10.151.1.203

A, B and C class addresses are pre-set and cannot be changed.

D class address is configured by use of hexadecimal switches installed on the device.

Table 2 shows the IP address configurations using hexadecimal switches.

Table 2 – IP Address Configuration using Hexadecimal Switch

| Device # | IP Address | Hex-switch ‘High’ | Hex-switch ‘Low’ |
| --- | --- | --- | --- |
| PTT Box #1 | 10.151.1.200 | C | 8 |
| PTT Box #2 | 10.151.1.201 | C | 9 |
| PTT Box #3 | 10.151.1.202 | C | A |
| PTT Box #4 | 10.151.1.203 | C | B |

Subnet mask and default gateway addresses are pre-set and cannot be changed. The addresses are:

Subnet Mask:	255.255.255.0

Gateway:	10.151.1.1

Technical Data

Table 3 – Technical Data

| Item | Specifications |
| --- | --- |
| Controller | STM ARM Cortex-M4 32bit |
| Connections | Ethernet and power connectors |
| Illuminated Pushbutton | EAO Series 56 – 56-3113.2389 |
| Input Voltage | 12 Vdc |