DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #14: ANSM Replacement

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2017-0017 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 3/16/2017 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 3/16/2017 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. ANSM repair process	4

4. Instructions	5

5. Testing	6

Introduction

This FMI provides instructions to replace ANSM.

Summary

ANSM failure was founded on the fleet in Denver. It is assumed that the failure occurred after the surge test at Philadelphia where the ANSM functionality was tested. The ANSM repair was proceeded after consultation with HR. The additional failure caused by the repair work was not occurred.

ANSM repair process

- Replace the failed ANS Microphone to normal product.

- The functional test was conducted by using TEST SET and other test equipment from WOOJIN.

Figure 1 – ANSM components

Figure 2 – ANSM pic.

| FIG & INDEX NUMBER |  | PART NUMBER | C/L | DESCRIPTION |  | UNITSPER ASSY | SUPPLIER PART NO. | SUPL CODE | USED ON CODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0- |  | ESP01390 |  | ANS MICROPHONE |  | REF | R1D1361-21499 | WJIS | SAB |
|  | -1 |  |  | . MICROPHONE |  | 1 | AOM-4544P-2 | PJU | SAB |
|  | -2 |  |  | . BRACKET |  | 1 | ,A366-91,t.5 | WJIS | SAB |
|  | -3 |  |  | . CONNECTOR |  | 1 | FRITS4106F14S-07SB1F6 | GLENA | SAB |
|  | -4 |  |  | . CABLE GRAND |  | 1 | PG7 | DONGA | SAB |
|  | -5 |  |  | . NAME LABEL |  | 1 | COTTON PAPER | WJIS | SAB |
|  | -6 |  |  | . MICROPHONE CAP |  | 1 | AL6063 | WJIS | SAB |
|  |  |  |  | CODE | USE ON |  |  |  |  |
|  |  |  |  | S A B | Single Car Married Pair A-Car Married Pair B-Car |  |  |  |  |

Instructions

1) Place the COMCB to OFF position

2) Fasten 2 mounting bolts on the ANSM-1.

3) Fasten 2 mounting bolts on the ANSM-2.

4) Place the COMCB to ON position

Figure 3 – ANSM Location

ANSM

(Ambient Noise Sensor Microphone)

Figure 4 – ANSM installation

Testing

1) Implement the simulation by CCP without ambient noise environment.

2) Check the automatic announcement volume via the saloon speaker.

3) Input the white noise to ANSM (over 3 seconds) using by cell phone at the moment of finishing the automatic announcement and staring the next announcement..

4) Check the output volume is higher while the next automatic announcement is played.