DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #15: SOFTWARE UPDATE

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2017-0053 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/10/2017 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/10/2017 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Location of Equipment	4

3.2	Software Update DESCRIPTION	6

3.2.1	Updating CRA-LICL	6

3.3	TEST AND VERIFICATION.	6

3.4	VERSION ChECK	7

4. Tracking	7

Introduction

This FMI provides instructions to install the Communication System update to the vehicles.

Summary

This FMI applies to following software configuration item:

- CRA-LICL SW change.

Instructions

Location of Equipment

Following figures show the location of equipment to be updated.

Figure 1 –CRA- LIC Board Location

Figure 2 – CRA Location

Software Update DESCRIPTION

Updating CRA-LICL

1) Time Stamp address change according to MDS LonWorks MA/MB car communication interface change.

2) Update LonWorks SW(LICL) of CRA-LIC PCB.

3) Complete QT confirmation according to SW change.

- Check the updated SW by LonScanner file QT which was changed on March 2017.

TEST AND VERIFICATION.

After updating the CRA-LICL software, follow the procedure below to verify the functionality:

1) MDS <-> CRA-LIC interface function check

- Information for Time, equipment status and version

VERSION ChECK

Use the MDS Software Version menu to verify the updated software versions.

- CRA-LICL : Version-1.1.03, Build Date – 2017-03-20.

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.