DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

# FIELD MODIFICATION INSTRUCTIONS 
FMI #15: SOFTWARE UPDATE

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2017-0053 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/10/2017 |

# Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/10/2017 | Initial release | D. S. Jang |

# Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Location of Equipment	4

3.2	Software Update DESCRIPTION	6

3.2.1	Updating CRA-LICL	6

3.3	TEST AND VERIFICATION.	6

3.4	VERSION ChECK	7

4. Tracking	7

# Introduction

This FMI provides instructions to install the Communication System update to the vehicles.

# Summary

This FMI applies to following software configuration item:

- CRA-LICL SW change.

# Instructions

## Location of Equipment

Following figures show the location of equipment to be updated.

Figure 1 –CRA- LIC Board Location

Figure 2 – CRA Location

## Software Update DESCRIPTION

### Updating CRA-LICL

1) Time Stamp address change according to MDS LonWorks MA/MB car communication interface change.

2) Update LonWorks SW(LICL) of CRA-LIC PCB.

3) Complete QT confirmation according to SW change.

- Check the updated SW by LonScanner file QT which was changed on March 2017.

4) The logic of Time Stamp

CRA must provide the Time Stamp to MDS and must receive the request of sending for the Time Stamp from MDS.

The below block diagram is described for the Time Stamp. (Blue Line is indicated to the Ethernet Network, Red Line is indicated to the LonWorks.)

CRA must provide the Time Stamp to MDS and must receive the request of sending for the Time Stamp from MDS.

## TEST AND VERIFICATION.

After updating the CRA-LICL software, follow the procedure below to verify the functionality:

1) MDS <-> CRA-LIC interface function check

- Information for Time, equipment status and version

## VERSION ChECK

Use the MDS Software Version menu to verify the updated software versions.

- CRA-LICL : Version-1.1.03, Build Date – 2017-03-20.

# Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.