DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #16: PA announcement

Hardware, software updates.

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2017-0068 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 7/18/2017 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 7/18/2017 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Location of Equipment	4

3.2	HARDWARE UPDATE DESCRIPTION	7

3.2.1	FAN Installation Method	7

3.2.2	FAN Operation Method	8

3.2.3	FAN Operation Results	9

3.3	Software Update DESCRIPTION	10

3.3.1	Algorithm Block Diagram	10

3.3.2	현재 문제점.	11

3.4	VERSION ChECK	12

4. Tracking	12

Introduction

This FMI provides instructions to install the Communication System update to the vehicles.

Summary

This FMI applies to following software & Hardware configuration item:

- CRA FAN Installation

- CCP SW change.

Instructions

Location of Equipment

Following figures show the location of equipment to be updated.

Figure 1 –CRA-FAN Install.

Figure 1 – CRA Location

Figure 2 – CCP Location

HARDWARE UPDATE DESCRIPTION

: CPU high temperature occurs due to overload in GPS data process of CRA-CNCS. Therefore, forced cooling method using FAN was performed for the automatic broadcasting abnormal operation.

FAN Installation Method

1) Remove CRA from the car.

Figure 4 – CRA.

2) Use the CRA side bolt to fix the FAN

Figure 5 –  FAN Install.

3) FAN installation in CAR

<Before FAN installation>

<After FAN installation>

FAN Operation Method

1) Use train power as operating power

2) The operating temperature can be set by the temperature sensor.

3) The FAN operates when the CRA ambient temperature is higher than the set temperature.

<FAN >

<temperature sensor>

Figure 6 – FAN components.

FAN Operation Results

: Test result of FAN installation

-> CPU internal temperature 95 degree  62 degree.

-> CNCS heating plate 75 degree  43 degree.

# Before

4066 CRA-CNCS Heating plate temperature change.

<CRA-CNCS Heating plate >

maximum 75℃ / Average 70℃

<CRA CNCS ambient temperature >

maximum 40℃ / Average 30℃

# 4066 CRA-CNCS heat plate temperature is 70℃ on average.

# After

4066 CRA-CNCS Heating plate temperature change.

<CRA-CNCS Heating plate >

maximum 43℃ / Average 40℃ 이상

<CRA CNCS ambient temperature >

maximum 35℃/ Average 30℃

# 4066 CRA-CNCS heat plate temperature is 40℃ on average.

Software Update DESCRIPTION

Algorithm Block Diagram

- Receives GPS data from PTC via Ethernet communication.

- The CRA identifies the GPS data corresponding to the car among the Ethernet data and converts it into 485 communication.

- The CCP retrieves the schedule data received by 485 communication GPS data. According to the search result, PA information is transmitted to the CRA.

- The CRA plays the PA and play information is shared with the PAC-MCU. The PA is output to the speaker through the AMP.

Current Issue

1) Before Change.

: Currently, the CCP does not check the GPS data information provided by the CRA.

2) After Change.

: The GPS data received from the CRA is confirmed by the filter algorithm and if it is determined that the data is abnormal, it communicates with the rear CRA to process the information

VERSION ChECK

Use the MDS Software Version menu to verify the updated software versions.

- CCP : 1.0.06

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.