DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #17: CRA FAN installation

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42A-2018-0001 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 12/20/2017 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 01/04/2018 | Initial release | D. S. Jang |
| 1.1 | 01/10/2018 | Section 5.5,5.6,5.7 | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Location of Equipment	4

3.2	HARDWARE UPDATE DESCRIPTION	6

3.2.1	FAN Installation Method	6

3.2.2	FAN Operation Method	7

3.2.3	FAN Operation Results	8

4. Tracking	9

Introduction

This FMI provides instructions to install the CRA FAN installation to the vehicles.

Summary

This FMI applies to following item:

- CRA FAN Installation

Instructions

Location of Equipment

Following figures show the location of equipment to be installed.

Figure 1 –CRA-FAN Installation

Figure 2 – CRA Location

HARDWARE UPDATE DESCRIPTION

: CPU high temperature occurs due to overload in GPS data process of CRA-CNCS. Therefore, forced cooling method using FAN was performed for the automatic PA abnormal operation.

FAN Installation Method

1) Remove CRA from the car.

Figure 3 – CRA

2) Use the CRA side bolt to fix the FAN

Figure 4 – FAN Installation

3) FAN installation in CRA

<Before FAN installation>

<After FAN installation>

FAN Operation Method

1) Use train power as operating power

2) The operating temperature can be set by the temperature sensor

3) The FAN operates when the CRA ambient temperature is higher than the set temperature

<FAN >

<temperature sensor>

Figure 5 – FAN components.

FAN Operation Results

: Test result of FAN installation

-> CPU internal temperature 95 degree  62 degree

-> CNCS heating plate 75 degree  43 degree

# Before

4066 CRA-CNCS Heating plate temperature change.

<CRA-CNCS Heating plate >

maximum 75℃ / Average 70℃

<CRA CNCS ambient temperature >

maximum 40℃ / Average 30℃

# 4066 CRA-CNCS heat plate temperature is 70℃ on average.

# After

4066 CRA-CNCS Heating plate temperature change.

<CRA-CNCS Heating plate >

maximum 43℃ / Average 40℃ 이상

<CRA CNCS ambient temperature >

maximum 35℃/ Average 30℃

# 4066 CRA-CNCS heat plate temperature is 40℃ on average.

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.

DTP Q&A (2017.7 DTP Presentation).

Fans are blowing air into CRA or sucking air from CRA? What will be final configuration?

- Fans are blowing air from CRA.

- The test sample will also change direction.(blowing air from CRA)

What circuit breaker will fans be connected to?  What is the updated load on that circuit breaker?

FAN

- COMCB circuit breaker

->  FAN 1EA -0.3A

Where is fresh air coming from in the electrical locker?

- Fresh air comes from ceiling

Is the thermostat necessary?  What is the thermostat setting?  Will fans be on all the time?

- The function of the temperature sensor is to stop the fan operation until CRA fully warm up at a cold start in winter.

- Thermostat will be set at 20 degree C.

the monitoring result after FAN installation (Monitoring period from july to august)

spare part supply for FAN

- WOOJIN will supply the spare part 10% of the total quantity.

65SET (Total quantity) + 7SET (Spare part) = 72SET

FAN air flow

- The air from the FAN will flow upward.

- The test sample installed on the car and the remained production unit will be installed on same direction.