DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #19: HARDWARE MODIFICATION

DPO / PEI

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42B-2018-0019 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 2/08/2018 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 2/08/2018 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Circuit changes	4

3.1	DPO / PEI changes	4

4. Instructions	8

4.1	DPO Instructions	8

4.2	PEI Instructions	10

5. Testing	11

Introduction

This FMI provides instructions to install the DPO and PEI.

Summary

This modification intends to implement the engineering change proposed as FMI #19. The modification applies to DPO and PEI units installed on the train.

Circuit changes

DPO / PEI changes

As a result of the DPO / PEI NCR cause analysis, most errors are caused by a malfunction of the AMP IC MP7720 at the speaker output terminals.

In the initial AMP IC design phase, designed DPO/PEI using a 10kΩ resistor (using AMP IC reference circuit) to stabilize the audio output level of the speaker output terminal.

However, the 10kΩ resistor cannot be protected from overload to the AMP IC and caused the operation to be disabled.

Therefore, the board changes the 10kΩ resistor to a 332pF capacitor and switches a circuit that bypasses external noise.

Other previous projects in 2017 have been verified by applying the change circuit. (Domestic Project - Seoul Metro Line No. 2)

Figure 1 – DPO AMP IC Location.

Figure 2 – PEI AMP IC Location.

- Before Change

- Proceeded the DPO and PEI design according to AMP IC MP7720 reference data sheet

- Request the cause analysis to the IC manufacturer due that the NCR was consistently occurred to same IC

- Received the scheme change for the Capacitor from IC manufacturer

Figure 3 –AMP IC(MP7720) Reference circuit

Figure 4 – DPO Board schematics

Figure 5 – PEI Board schematics

- After Change.

- Performed the simulation for the changes of Resistor to Capacitor

- Confirmed the same audio output

- Confirmed the improvement of the output waveform (Shown as the mark in  Figure 7 and 8)

Figure 7 – 10kΩ Resistor simulation.

Figure 8 – 332pF capacitor simulation.

| Description | Before Improvement | After Improvement | Remarks |
| --- | --- | --- | --- |
| DPO | R32 10kΩ | 332pF | capacitor |
| PEI | R17 10kΩ | 332pF | capacitor |

Instructions

DPO Instructions

1) Place the COMCB and PICB to OFF position

2) Unscrew 4 mounting bolts on the DPO and PEI

3) Remove the DPO / PEI

4) Conduct PCB operations at designated locations

5) Installation the DPO / PEI

6) Place the COMCB and PICB to ON position

Figure 9 – DPO Location

Figure 10 – DPO Board PCB

PEI Instructions

Figure 11 – PEI Location

Figure 12 – PEI Board PCB

Testing

1) Key-on the cab

2) Test PEI-DPO, PEI-OCC and DPO-OCC communication

※ OCC simulator will perform as PBX server and OCC dispatcher.

3) Will verify to make sure that there is no issue or problem during these communication test.