DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #20: HARDWARE MODIFICATION

CCP

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42B-2018-0020 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 2/08/2018 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 2/08/2018 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Circuit changes	4

3.1	CCP CHANGES	4

4. Instructions	7

4.1	CCP Instructions	7

5. Testing	9

Introduction

This FMI provides instructions to install the CCP.

Summary

This modification intends to implement the engineering change proposed as FMI #20. The modification applies to CCP units installed on the train.

Circuit changes

CCP CHANGES

As a result of CCP communication failure cause analysis, RTS failure which controls RS-485 communication was detected.

CCP Board controls RS-485 RTS using 74LS123 IC and RTS control time is controlled by capacitor and resistor.

Figure 1 – CCP Borad RTS Control Circuit

Figure 1 – CCP Board RTS control mechanism.

1) Before Change

Figure 2 – Before RTS Control Circuit (R296 1.2kΩ)

CCP Board RTS control is designed to have a delay time of 50ns after transmitting the data. The length of protocol was longer than before since the functionalities were added. In case that the capacitor characteristics are temporary changed by the external factors, the RTS control time will be changed shown as figure 1.

If the RTS time is changed, the last Byte loss of the transferred data will be occurred and recognized as the communication fault

Since the delay time was short, the RTS control time due to external factors (noise and power supply) varied and RS-485 communication failure occurred.

2) After Change

Figure 3 – After RTS Control Circuit (R296 10KΩ)

The resistance value was changed from 1.2kΩ to 10kΩ and the delay time increased from 50ns to about 1.7ms by about 30,000 times (1,700,000 ns / 50 ns = 34000)

Even if the RTS is changed due to external factors, there will be no issue with RS-485 communication because the sufficient RST control time was secured.

Figure 4 – CCP RS-485 Communication TEST(R296 10KΩ)

| Description | Before Improvement | After Improvement | Remarks |
| --- | --- | --- | --- |
| CCP | R296 1.2kΩ(50ns) | R296 10kΩ(1.7ms) | Resistor |

Instructions

CCP Instructions

1) Place the COMCB and PICB to OFF position

2) Unscrew 4 mounting bolts on the CCP

3) Remove the CCP

4) Conduct PCB operations at designated locations

5) Installation the CCP

6) Place the COMCB and PICB to ON position

Figure 4 – CCP Location

Figure 5 – CCP Board PCB

Testing

1) Key-on the cab

2) Using the CCP touchscreen, enter route number 10 or 11

3) Run the route simulation

4) Verify the simulated announcements and displays function appropriately.