DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #21: HARDWARE MODIFICATION

PID

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42B-2018-0021 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 15/NOV/2018 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 15/NOV/2018 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Changes	4

3.1	PID CHANGE	4

4. Instructions	5

4.1	PID Instructions	5

5. Testing	6

Introduction

This FMI provides instructions to install the PID power board.

Summary

This modification intends to implement the engineering change proposed as FMI #21. The modification applies to PID units installed on the train.

Changes

PID CHANGE

As a result of PID failure cause analysis, the spacer material between heatsink and baseboard of PID power will change to polyacetal(DURACON).

Figure 1 – PID POWER BOARD CIRCUIT

Instructions

PID Instructions

1) Place circuit breaker of PID power to OFF position

2) Unscrew 2 mounting bolts on PID.

3) Take off PID power board from PID unit.

4) Unscrew 2 mounting bolts backward of PID power board

5) Unscrew 6 mounting bolts between heatsink and DC/DC converter

6) Change spacer material to polyacetal

Figure 2 – PID unit

Figure 3 – PID Power Board

Testing

1) Turn on PID power circuit breaker

2) Verify the proper operation of PID units.