DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #18-1: LIC

software updates.

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42B-2018-0036 |
| REVISION: | 1.1 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 4/06/2018 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 2/08/2018 | Initial release | D. S. Jang |
| 1.1 | 4/05/2018 | - CAB Change Algorithm - Firmware boot time | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Instructions	4

3.1	Location of Equipment	4

3.2	Software Update DESCRIPTION	6

3.2.1	LIC Algorithm Block Diagram	6

3.3	TEST AND VERIFICATION	8

3.4	VERSION ChECK	9

4. Tracking	9

Introduction

This FMI provides instructions to install the Communication System update to the vehicles.

Summary

This FMI applies to following software configuration item:

- LIC SW change.

Instructions

Location of Equipment

Following figures show the location of equipment to be updated.

LIC Board

Figure 1 – LIC Board Location

Figure 2 – CRA Location

Software Update DESCRIPTION

LIC Algorithm Block Diagram

- #CAB Change Algorithm

Figure 3 – CAB Change Algorithm

- Before Change

- CAB change is occurred at the last station during the train service.

- The CRA-PAMCU is operated "Inauguration" to check the multi-coupled train information of the vehicle.

- Pause the communication of sub device during "Inauguration".

- MDS fault is occurred due to the interruption of the sub-device communication.

- When complete 'Inauguration', the MDS fault is cleared.

- After Change.

- CAB change is occurred at the last station during the train service.

- CRA-PAMCU is operated "Inauguration" to check the multi-coupled train information of the vehicle.

- Pause the communication of sub device during "Inauguration".

- When the LIC Board detects "Inauguration" operation, it suspends the communication error counter until "Inauguration" is completed.

- When the "Inauguration" is completed and  after 1 cycle of the sub-system communication check,

- the communication error check will be started.

- # Firmware boot time

- Before Change

- When the CRA CB is ON, LIC PCB starts the communication with MDS.

- VOIP, CCP and CNCS take longer booting time than LIC PCB and the MDS failure is occurred until the normal operation of each devices.

- After Change.

- When the CRA CB is ON, LIC PCB starts the communication with MDS.

- LIC PCB starts the MDS communication after completing CNCS booting which takes the longest booting time.

TEST AND VERIFICATION

After updating the LIC and VOIP software, follow the procedure below to verify the functionality:

1) Woojin engineer will compare the difference of the LIC software change to identify the COMM. fault was decreased after testing on the two pair of trains.

2) Woojin engineer will also report the result of the test to HR and DTO.

VERSION ChECK

Use the MDS Software Version menu to verify the updated software versions.

- CRA-LICM : 1.1.05

Tracking

Status of the field modification activities will be tracked and managed for car number and items completed. The progress will be reported to DTP/H-Rotem in daily basis as the modification proceeds.