DENVER RTD EAGLE P3 
COMMUNICATION SYSTEM

FIELD MODIFICATION INSTRUCTIONS 
FMI #19: HARDWARE MODIFICATION

DPO / PEI

| CONTRACT NO. | P110701149 |
| --- | --- |
| CDRL NO. | N/A |
| WIS DOC. NO. | 42B-2018-0019 |
| REVISION: | 1.0 |
| DESIGNED: | D. S. JANG |
| REVIEWED: | J. Y. LEE |
| APPROVED: | B. J. JONE |
| DATE: | 2/08/2018 |

Record of Revisions

| Rev. | Date | Description | Entered By |
| --- | --- | --- | --- |
| 1.0 | 2/08/2018 | Initial release | D. S. Jang |

Table of Contents

1. Introduction	4

2. Summary	4

3. Circuit changes	4

3.1	DPO / PEI changes	4

4. Instructions	8

4.1	DPO Instructions	8

4.2	PEI Instructions	10

5. Testing	11

Introduction

This FMI provides instructions to install the DPO,PEI and CCP.

Summary

This modification intends to implement the engineering change proposed as FMI #19. The modification applies to DPO,PEI and CCP units installed on the train.

Circuit changes

DPO / PEI changes

As a result of the DPO / PEI NCR cause analysis, most errors are caused by a malfunction of the AMP IC MP7720 at the speaker output terminals.

In the initial AMP IC design phase, designed DOP/PEI using a 10kΩ resistor (using AMP IC reference circuit) to stabilize the audio output level of the speaker output terminal.

However, the 10kΩ resistor generated a current load in the AMP IC and caused the operation to be disabled.

Therefore, the board changes the 10kΩ resistor to a 332pF capacitor and switches a circuit that bypasses external noise.

Other previous projects in 2017 have been verified by applying the change circuit.(Seoul Metrol)

Figure 1 – DPO AMP IC Location.

Figure 2 – PEI AMP IC Location.

- Before Change

- AMP IC MP7720 reference data sheet 에 따라, DPO 및 PEI 설계를 진행.

- 지속적으로 동일 IC에 대하여, NCR이 발생하여 IC 업체에 검토 요청.

- Capacitor 변경에 대한 IC 업체 회신 수신.

Figure 3 –AMP IC(MP7720) Reference circuit

Figure 4 – DPO Board schematics

Figure 5 – PEI Board schematics

Figure 6 – Capacitor technology review e-mail.

- After Change.

- Resistor   Capacitor 변경에 따른, 출력 파형 시뮬레이션을 통한 기능 검증.

- 출력 음원에 대한, 동일한 특성 확인.

- Resistor  Capacitor 변경시 출력 파형 향상.(그림7,8에  원으로 표시)

Figure 7 – 10kΩ Resistor simulation.

Figure 8 – 332p capacitor simulation.

| Description | Before Improvement | After Improvement | Remarks |
| --- | --- | --- | --- |
| DPO | R32 10kΩ | 332pF | capacitor |
| PEI | R17 10kΩ | 332pF | capacitor |

Instructions

DPO Instructions

1) Place the COMCB and PICB to OFF position

2) Unscrew 4 mounting bolts on the DPO and PEI and CCP

3) Remove the DPO / PEI / CCP

4) Conduct PCB operations at designated locations

5) Installation the DPO / PEI /CCP

6) Place the COMCB and PICB to ON position

Figure 9 – DPO Location

Figure 10 – DPO Board PCB

PEI Instructions

Figure 11 – PEI Location

Figure 12 – PEI Board PCB

Testing

1) Key-on the cab

2) Using the CCP touchscreen, enter route number 10 or 11

3) Run the route simulation

4) Verify the simulated announcements and displays function appropriately.

| NUM | HW |  |  | SW |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | DPO | PDI | CCP | CRA-LICM | CRA-VOIP |  |
| 4001-2 ( - - ) |  |  |  |  |  |  |
| 4003-4 ( - - ) |  |  |  |  |  |  |
| 4005-6 ( - - ) |  |  |  |  |  |  |
| 4007-8 ( - - ) |  |  |  |  |  |  |
| 4009-10 ( - - ) |  |  |  |  |  |  |
| 4011-12 ( - - ) |  |  |  |  |  |  |
| 4013-14 ( - - ) |  |  |  |  |  |  |
| 4015-16 ( - - ) |  |  |  |  |  |  |
| 4017-18 ( - - ) |  |  |  |  |  |  |
| 4019-20 ( - - ) |  |  |  |  |  |  |
| 4021-22 ( - - ) |  |  |  |  |  |  |
| 4023-24 ( - - ) |  |  |  |  |  |  |
| 4025-26 ( - - ) |  |  |  |  |  |  |
| 4027-28 ( - - ) |  |  |  |  |  |  |
| 4029-30 ( - - ) |  |  |  |  |  |  |
| 4031-32 ( - - ) |  |  |  |  |  |  |
| 4033-34 ( - - ) |  |  |  |  |  |  |
| 4035-36 ( - - ) |  |  |  |  |  |  |
| 4037-38 ( - - ) |  |  |  |  |  |  |
| 4039-40 ( - - ) |  |  |  |  |  |  |
| 4041-42 ( - - ) |  |  |  |  |  |  |
| 4043-44 ( - - ) |  |  |  |  |  |  |
| 4045-46 ( - - ) |  |  |  |  |  |  |
| 4047-48 ( - - ) |  |  |  |  |  |  |
| 4049-50 ( - - ) |  |  |  |  |  |  |
| 4051-52 ( - - ) |  |  |  |  |  |  |
| 4053-54 ( - - ) |  |  |  |  |  |  |
| 4055-56 ( - - ) |  |  |  |  |  |  |
| 4057-58 ( - - ) |  |  |  |  |  |  |
| 4059-60 ( - - ) |  |  |  |  |  |  |
| 4061-62 ( - - ) |  |  |  |  |  |  |
| 4063-64 ( - - ) |  |  |  |  |  |  |
| 4065-66 ( - - ) |  |  |  |  |  |  |